MicroMind / NanoCorteX

Software Requirements Specification

SRS-MicroMind-v1.2.1  |  Post SB-4 Approved Corrections  |  31 March 2026

Supersedes SRS-MicroMind-v1.1. Integrates approved corrections R-01..R-06, P-01..P-04, E-01..E-04, V-01..V-04, O-01, S-01 from the v1.1 Review. All corrections integrated directly into body text.

| Field | Value |
| --- | --- |
| Document ID | SRS-MicroMind-v1.2.1 |
| Version | v1.3 — April 2026 — Navigation architecture amendments: NAV-02 rewritten for orthophoto image matching replacing RADALT-NCC (AD-01). AVP-01 removed from §1.4 AVP table (deferred). AVP scope notes added to §1.1 and NAV unit tests. §1.3 taxonomy extended with AVP fallback event types. §10.2 Mission Envelope Schema extended with AVP fields. §10.16 logging schema extended with AVP and OM fallback events. §14 traceability updated: NAV-02 status revised, FR-107 and FR-110a gap rows added. §15 gaps updated: GAP-10 OM SIL gap, GAP-11 AVP-04 envelope gap, GAP-12 BIM adaptive spoof, AMB-06 SAT-01 gap added. §17 EC-13 added: orthophoto matching SIL gate. |
| Supersedes | SRS-MicroMind-v1.1 (31 March 2026) |
| Status | BASELINED — SB-5 Execution Baseline |
| Classification | Programme Confidential |
| Corrections Applied | R-01..R-06 (Retask), P-01..P-04 (PX4), E-01..E-04 (Endurance), V-01..V-04 (Demo), O-01 (Ownership), S-01 (SB-5 Entry) Added User Requirements in Section 1.4 and 1.5 that must be modelled by the Test Planned to execute test cases in this document. Profile driven validation approach, refence UAV profiles, Operational Scenarios, Terrain dependent sensor guidance, environmental expansion, and profile based testing / boundary condition structure. |
| Architecture Baseline | Pre-HIL Software Completion Plan v1.2 + Post SB-4 |

# 1. Introduction and Scope

## 1.1 Purpose and Scope

This Software Requirements Specification (SRS) v1.3 is the authoritative requirements document for the MicroMind/NanoCorteX autonomy stack. It supersedes v1.1 and integrates all approved corrections identified in the SRS v1.1 Review. No new requirement families have been introduced. No sections have been rewritten unless a correction required it. The document is the SB-5 execution baseline.

Corrections integrated in this version:

- R-01..R-06: Dynamic retasking — terrain ordering, EW map staleness, rollback completeness, waypoint upload sequencing, INS-only retask constraint, timeout normalisation

- P-01..P-04: PX4 state persistence — SHM in Checkpoint, Mission Manager gate before autonomous resume, stale setpoint discard on recovery, ETA in Checkpoint

- E-01..E-04: Long-endurance — checkpoint accumulation/purge, route fragment cleanup, process restartability classification, log rolling policy

- V-01..V-04: Mission Demonstration Tool — authoritative mission clock, log-driven event overlays, pause/resume sync rules, reproducibility requirement

- O-01: Recovery Ownership Matrix added as §16

- S-01: SB-5 Entry Criteria added as §17

MicroMind validation is profile-driven rather than platform-specific. The system shall be tested across multiple reference UAV classes. The active profiles for validation in Phase 1 are AVP-02 (Reusable Loitering Munitions), AVP-03 (Hybrid Platform), and AVP-04 (Deep Strike Expendable). AVP-01 (FPV/Quadrotor) is deferred — the low-altitude regime (10–150 m AGL) requires separate sensor assumptions and validation campaigns not in scope for Phase 1. Test cases shall be constructed using representative combinations of speed, altitude, terrain, GNSS availability, RF availability, and sensor degradation for AVP-02/03/04 profiles rather than a single nominal flight profile.

## 1.2 Terminology Standardisation (Normative)

NORMATIVE: The following canonical terms are used throughout this document. Any deviation in code, logs, or tests constitutes a documentation defect.

| Canonical Term | Deprecated Terms | Definition |
| --- | --- | --- |
| Mission Demonstration Tool | Replay, Visualization, Animated Replay | The full output layer: static dashboard + animated export + live demo environment |
| Live Mission Demo | Live demo, animated replay, synchronized replay | Timeline-driven moving-marker output (post-SB-5, dependency-gated) |
| Static Mission Report | Report, HTML report, dashboard export | Current SB-4 static HTML/PNG output |
| Mission Manager | MissionMgr, FSM, Mission Controller | Module: state machine + envelope enforcement + SHM logic |
| Navigation Manager | NavMgr, NanoCorteX Navigation, Nav Stack | Module: ESKF + mode arbitration + TRN/VIO integration |
| Safe Hold Mode (SHM) | Emergency Hold, Safe Mode, Failsafe Hold | The single recovery state: loiter at current position, await clearance |
| OFFBOARD Loss | OFFBOARD exit, OFFBOARD continuity failure, PX4 drop | PX4 transition out of OFFBOARD mode for any reason |
| BIM Trust Score | trust score, trust_state score | Float 0.0–1.0 from bim.py |
| BIM Trust State | trust state, RED state, spoof reject state | Enum: GREEN / AMBER / RED |
| Vehicle A | Baseline vehicle, unprotected vehicle | Dead-reckoning-only track in BCMP-2 |
| Vehicle B | MicroMind vehicle, protected vehicle | Full MicroMind stack track in BCMP-2 |
| Retask Command | Dynamic retask, mid-mission update, operator command | Signed operator instruction to change target/route mid-mission |
| Checkpoint | Nav checkpoint, state snapshot, restore point | Persisted navigation and mission state enabling process restart recovery |
| EW Exclusion Zone | EW avoidance zone, threat avoidance area | Geometric exclusion zone derived from EW cost map threat entries |
| No-Fly Zone (NFZ) | Exclusion zone, terrain exclusion, restricted area | Pre-planned geographic exclusion loaded at mission start |

## 1.3 Recovery Behaviour Taxonomy (Normative)

All recovery behaviours in this document are drawn exclusively from the following typed list.

| Recovery Class | Code | Definition | Escalates To |
| --- | --- | --- | --- |
| Continue Nominally | CONT_NOM | No action. System continues at full capability. | — |
| Continue Degraded | CONT_DEG | Continue mission with reduced capability. Log degradation event. | RETRY or SHM after timeout |
| Retry | RETRY_N | Retry operation up to N times at defined interval. On Nth failure: escalate. | Defined per requirement |
| Retry Alternate Source | RETRY_ALT | Retry using backup sensor or nav source. | CONT_DEG or SHM |
| Hold Current Path | HOLD_PATH | Maintain current waypoint and heading. Do not advance. Await clearance. | SHM after 10 s |
| Replan Route | REPLAN | Invoke Hybrid A* with current constraints. On failure: ROUTE_DEADEND. | SHM if ROUTE_DEADEND |
| Enter SHM | SHM | Command loiter at current position. Log SHM_ACTIVATED with reason. | ESCALATE_OP after 120 s |
| Abort Terminal Only | ABORT_TERM | Cancel terminal approach only. Vehicle continues in cruise. | DMRL re-acquire or SHM |
| Abort Mission | ABORT_MISS | Full mission abort. Return to safe waypoint if defined, else SHM. | — |
| Escalate to Operator | ESCALATE_OP | Log OPERATOR_ACTION_REQUIRED. Await retask command up to 120 s. | ABORT_MISS on timeout |
| Restart Process | PROC_RESTART | Watchdog kills and restarts failed process. SHM during restart. | ABORT_MISS if not restored in 30 s |
| Restore from Checkpoint | CHECKPOINT_RESTORE | Load last valid Checkpoint. Merge with current vehicle state. | PROC_RESTART if corrupt |
| AVP Fallback — Altitude Reduction | AVP_ALT_REDUCED | Operational altitude reduced below AVP profile floor due to navigation or sensor constraint. Log AVP_FALLBACK_ALTITUDE_REDUCTION with {avp_profile_id, planned_alt_m_agl, actual_alt_m_agl, reason, mission_km}. Continue mission at reduced altitude. | CONT_DEG after timeout |
| AVP Fallback — Speed Reduction | AVP_SPD_REDUCED | Operational speed reduced below AVP profile cruise speed due to navigation, EW, or route constraint. Log AVP_FALLBACK_SPEED_REDUCTION with {avp_profile_id, planned_speed_kmh, actual_speed_kmh, reason, mission_km}. Continue mission at reduced speed. | CONT_DEG |

## 1.4 User Input Parameters that need to be modelled during Test Runs

- Mission Envelope includes Platform Specific Speed, Altitude and Survivability Constraints.

- The following air vehicle profiles must be modelled in all test scenarios

| Profile ID | Platform Class | Cruise Speed | Top Speed | Altitude (AGL) | Range | GNSS-Denied Segment |
| --- | --- | --- | --- | --- | --- | --- |
| AVP-02 | Re-usable Loitering Munitions (ALS-25 or equivalent) | 100 km/h | 140 km/h | 100–800 m AGL | 100–250 km | 50–100 km |
| AVP-03 | Hybrid Platform | 100 km/h | 200 km/h | 100–1500 m AGL | 100–250 km | 100 km |
| AVP-04 | Deep Strike - Expendable | 150 km/h | 200 km/h | 500–2000 m AGL | 1000+ km | 100+ km |

AVP-01 Deferred: AVP-01 (FPV/Quadrotor, 10–150 m AGL, 5–20 km range) is not in scope for Phase 1 validation. The low-altitude regime requires separate sensor assumptions (orthophoto matching performance at < 150 m AGL, VIO feature density at low altitude) that have not been validated. AVP-01 will be addressed in a separate validation campaign after AVP-02/03/04 Phase 1 closure.

- Validation & Testing

All micromind modules will go through a tiered Validation & testing procedure. The test cases shall be written / used that effectively test the program for its nominal, stress and boundary conditions.

The test cases be designed to effectively use the Air Vehicle Profile for assessing the usability of MicroMind solution to various products.

- No Test Scenario shall start with an AVP other than the one specified above. Where fallback behavior dictates reduction in altitude, and speed, the same shall be logged and analyzed post test. For e.g., if terrain texture is insufficient for orthophoto matching over an extended featureless segment, and the vehicle accumulates drift beyond the CONT_DEG threshold, the log file should note these circumstances.

5. AVP fallback events (AVP_FALLBACK_ALTITUDE_REDUCTION, AVP_FALLBACK_SPEED_REDUCTION) shall be logged per §1.3 whenever operational parameters fall below the specified AVP profile. Post-test analysis shall include a count of fallback events per AVP profile and a root-cause classification.

| Tier | Purpose |
| --- | --- |
| Tier 1 | Unit tests for IMU, DEM, VIO, TRN, BIM, SDR, RADALT |
| Tier 2 | Subsystem integration tests |
| Tier 3 | Profile-based mission tests |
| Tier 4 | Full mission stress tests |
| Tier 5 | Failure injection and boundary-condition tests |

Vehicle Boundary Conditions to be used for defining Test Scenarios

| Parameter | Nominal | Stress | Boundary |
| --- | --- | --- | --- |
| Speed | 100 km/h | 150 km/h | 200 km/h |
| Altitude | 300–800 m AGL | 800–1000 m AGL | 1500 m AGL |
| GNSS-denied duration | 20 km | 50 km | 100 km |
| RF-severed duration | 10 min | 30 min | 60 min |
| Bank angle | 15° | 30° | 45° |
| Heading change between legs | 20° | 45° | 60° |

## 1.5 Environmental Test Conditions

The missions shall be subjected to the following environmental conditions. The scenarios, wherever possible, shall be created within these environmental conditions. If needed, a certain environmental degradation factor can be built into the sensor pipeline to assess the robustness of our implementation.

Operating Temperature: -20oC to +55oC

Storage Temperature: -30oC to +70oC

Relative Humidity: 95% RH at 25oC

Operations in Rain: Yes

EMI/EMC:  As per 461E/F

Reference testing environments shall include plains, desert, mountains, snow-covered terrain, and mixed-terrain operational corridors. Environmental testing shall consider reduced observability, haze, snow cover, repetitive desert textures, thermal loading, and low-feature terrain

FPV-class UAVs, reusable loitering munitions, deep-strike expendable systems, and hybrid GNSS-denied strike platforms.

# 2. Navigation Requirements

## 2.1 NAV-01 — INS Drift Limit

| Req ID | NAV-01 |
| --- | --- |
| Title | INS-Only Navigation Drift Limit |
| Priority | Critical |
| Description | When operating in GNSS-denied mode with no TRN or VIO corrections active, lateral drift from the planned route shall not exceed 100 m per 5 km segment (2% of distance). Drift is measured as perpendicular cross-track error from the planned corridor centreline. |
| Inputs | IMU accel+gyro at 200 Hz (STIM300). Initial GNSS fix before denial. Planned route waypoints. |
| Outputs | cross_track_error_m, drift_rate_m_per_km, corridor_breach_flag (bool), drift_at_km60_m, drift_at_km100_m, drift_at_km120_m |
| Preconditions | Valid GNSS fix acquired and ESKF converged before GNSS denial. IMU bias calibrated. Frozen constants: _ACC_BIAS_RW=9.81e-7, _GYRO_BIAS_RW=4.04e-8. |
| Success Threshold | cross_track_error_m within C-2 envelopes: km60=[5,80]m, km100=[12,350]m, km120=[15,650]m for 3 canonical seeds (42,101,303). |
| Failure Threshold | cross_track_error_m > 500 m (corridor half-width) before km 30 of INS-only phase. OR drift > 1000 m at any point. OR any NaN in position vector. |
| Recovery Behaviour | drift > 80% of corridor limit: CONT_DEG — increase TRN polling 2 km → 1 km. drift > corridor: log CORRIDOR_BREACH, set corridor_breach_flag=True. SHM if breach persists > 15 s with no TRN correction available. |
| Logging (mandatory) | cross_track_error_m at 10 Hz. CORRIDOR_BREACH: {mission_km, lateral_error_m, threshold_m, seed}. TRN_POLL_INCREASED on 80% trigger. |
| Dependencies | error_state_ekf.py, baseline_nav_sim.py (BCMP-2), bcmp2_drift_envelopes.py |
| Linked Tests | UT-NAV-01-A, UT-NAV-01-B, IT-NAV-01, ST-END-01 |
| Calibration Caveat | Calibration Caveat (mandatory in all external reports): The C-2 drift envelopes (km60=[5,80]m, km100=[12,350]m, km120=[15,650]m) were calibrated using Monte Carlo N=300 on the BASELINE IMU noise model (ARW 0.05 degrees/sqrt-hr). The STIM300 tactical IMU candidate has ARW 0.15 degrees/sqrt-hr. Re-calibration with STIM300 noise profile is pending (OI-03). Any result citing these envelopes must note: C-2 envelopes calibrated on BASELINE IMU; STIM300 re-calibration pending. These envelopes apply to AVP-02/03 corridor geometry. AVP-04 GNSS-denied segments exceeding km 120 are not characterised — this is a known gap (see §15 GAP-11). |

## 2.2 NAV-02 — TRN Correction Accuracy

| Req ID | NAV-02 |
| --- | --- |
| Title | L2 Absolute Reset — Orthophoto Matching Correction Accuracy |
| Priority | Critical |
| Description | The L2 Absolute Reset layer shall reduce accumulated INS drift to within 50 m CEP-95 at each correction point when orthophoto match confidence >= match_threshold. Corrections are applied every 2–5 km over textured terrain (sigma_terrain >= 30 m). Match suppression is logged and counted toward the consecutive-suppression limit. This requirement replaces RADALT-NCC TRN as specified in SRS v1.2.1. The correction mechanism is 2D patch NCC against preloaded satellite imagery tiles using the nadir EO/LWIR camera. See Part Two V7.2 Section 1.7.2. |
| Inputs | INS position estimate (north_m, east_m), nadir camera frame (EO or LWIR), preloaded satellite tile set for current mission area, terrain_texture_score (sigma_terrain computed from tile). |
| Outputs | om_correction_north_m, om_correction_east_m, match_confidence (0.0–1.0), correction_applied (bool), consecutive_suppressed_count, om_last_fix_km_ago. |
| Preconditions | Satellite tile set loaded for mission area. Camera delivering valid frames >= 5 FPS. ESKF running. Terrain texture score computed. |
| Success Threshold | Correction CEP-95 < 50 m. match_confidence >= match_threshold. At least 1 successful correction per 5 km over textured terrain (sigma_terrain >= 30 m). consecutive_suppressed_count resets to 0 on each successful correction. |
| Failure Threshold | match_confidence < match_threshold for >= 3 consecutive windows. Correction CEP > 100 m. No successful correction for > 10 km continuous over terrain with sigma_terrain >= 30 m. |
| Recovery Behaviour | match_confidence < match_threshold: log OM_SUPPRESSED, CONT_DEG. After 3 consecutive suppressions: RETRY_ALT — increase VIO fusion weight, log OM_EXTENDED_GAP. After 10 km no correction over textured terrain: NAV_DEGRADED_INS_ONLY, notify route planner to seek textured terrain. |
| Logging (mandatory) | match_confidence, correction_applied, correction_magnitude_m, om_last_fix_km_ago, mission_km at each OM attempt. OM_SUPPRESSED: {match_confidence, mission_km, consecutive_count, sigma_terrain}. OM_EXTENDED_GAP: {gap_km, sigma_terrain_min}. OM_RESUMED after recovery. |
| Frozen Constants | match_threshold (TBD — SIL Sprint S3); correction_interval_min_km=2.0; correction_interval_max_km=5.0; max_consecutive_suppressed=3; featureless_threshold_sigma=10 m; preferred_terrain_sigma=30 m |
| Linked Tests | UT-NAV-02-A (rewrite required), UT-NAV-02-B (rewrite required), IT-NAV-02, IT-NAV-03, ST-MULTI-01 |
| Linked Tests | UT-NAV-02-A (rewrite required), UT-NAV-02-B (rewrite required), IT-NAV-02, IT-NAV-03, ST-MULTI-01 |

| SIL Status Note | Zero SIL tests currently exist for orthophoto matching. UT-NAV-02-A and UT-NAV-02-B were written for the superseded RADALT-NCC mechanism and must be rewritten before NAV-02 can be validated. The existing test IDs are retained to preserve traceability; their stimulus and assertions will change entirely. |
| --- | --- |

## 2.3 NAV-03 — VIO Drift Rate

| Req ID | NAV-03 |
| --- | --- |
| Title | Visual-Inertial Odometry Drift Rate Limit |
| Priority | High |
| Description | OpenVINS shall maintain position drift ≤ 1.0 m/km under nominal illumination with ≥ 50 tracked feature points. VIOMode state transitions are defined in Interface §10.3. |
| Inputs | Camera frames at ≥ 25 FPS, IMU at 200 Hz, VIONavigationMode current state. |
| Outputs | vio_position_delta_m (north, east), vio_covariance_3x3, vio_mode (ACTIVE/DEGRADED/OUTAGE/INITIALISING), accepted (bool), innov_mag (m). |
| Preconditions | VIONavigationMode initialised. Camera delivering valid frames. Feature count ≥ 50. |
| Success Threshold | Stage-2 drift proxy ≤ 1.0 m/km. accepted=True ≥ 95% of updates under nominal. Cross-sequence variance ≤ 5%. OpenVINS Stage-2 proxies 0.94 and 0.97 m/km. |
| Failure Threshold | Drift proxy > 2.0 m/km for > 30 consecutive s. vio_mode=OUTAGE for > 3 s with no recovery path. |
| Recovery Behaviour | Drift > 1.5 m/km: CONT_DEG — flag VIO_DEGRADED, halve VIO fusion weight, increase TRN weight. OUTAGE > 3 s: RETRY_ALT — demote VIO, rely on TRN+INS, log VIO_OUTAGE_EXTENDED. OUTAGE > 30 s with no TRN: SHM if cross_track_error_m > 400 m. |
| Logging (mandatory) | vio_mode at 10 Hz. drift_proxy_m_per_km, accepted, innov_mag at each update. VIO_DEGRADED: {drift_proxy, mission_km}. VIO_OUTAGE_EXTENDED: {duration_ms}. VIO_RESUMED after recovery. |
| Linked Tests | UT-NAV-03, IT-NAV-03, ST-MULTI-01 |

## 2.4 NAV-04 — Navigation Mode Transition Latency

| Req ID | NAV-04 |
| --- | --- |
| Title | Navigation Mode Transition Latency and Continuity |
| Priority | High |
| Description | Transitions between navigation modes (GNSS→TRN, TRN→VIO, VIO→INS_ONLY, and reverse) shall complete within 500 ms. Position continuity shall be maintained: no jump > 10 m nominal, no jump > 50 m under any condition. |
| Inputs | BIM Trust State transition event, GNSS availability flag, vio_mode, TRN correction availability, Navigation Manager arbitration logic. |
| Outputs | active_nav_mode (GNSS/TRN_PRIMARY/VIO_PRIMARY/INS_ONLY), transition_latency_ms, position_discontinuity_m, NAV_MODE_TRANSITION event. |
| Success Threshold | Transition latency ≤ 500 ms. position_discontinuity_m < 10 m nominal. Never > 50 m. |
| Failure Threshold | Transition latency > 2 s. position_discontinuity_m > 50 m. |
| Recovery Behaviour | Transition failure (latency > 2 s): RETRY (1× at 500 ms). After 1 retry failure: SHM. Discontinuity > 50 m: log POSITION_JUMP, freeze ESKF for 500 ms, reinitialise from best available source, log NAV_REINIT. |
| Logging (mandatory) | NAV_MODE_TRANSITION: {from_mode, to_mode, trigger_event, latency_ms, position_discontinuity_m} at every transition. |

## 2.5 NAV-05 — BIM Trust Score Update and Spoof Detection

| Req ID | NAV-05 |
| --- | --- |
| Title | BIM Trust Score Update Rate and GNSS Spoof Detection |
| Priority | Critical |
| Description | The Navigation Manager's BIM component shall update the BIM Trust Score ≥ 10 Hz. A GNSS spoofing event shall be detected and BIM Trust State transitioned to RED within 250 ms. GNSS shall be gated out of ESKF immediately on RED. |
| Inputs | GNSS measurement stream (position, velocity, hdop, fix_type). BIM internal Bayesian state. |
| Outputs | bim_trust_score (float 0.0–1.0), bim_trust_state (GREEN/AMBER/RED), spoof_detected (bool), gnss_gated (bool), detection_latency_ms. |
| Preconditions | GNSS stream active. BIM initialised. bim_trust_state = GREEN. |
| Success Threshold | Update rate ≥ 10 Hz. Spoof detection latency ≤ 250 ms. bim_trust_state = RED when bim_trust_score < 0.10. GNSS gated within 50 ms of RED. |
| Failure Threshold | Update rate < 5 Hz for > 500 ms. Spoof undetected within 1 s. GNSS incorporated into ESKF when bim_trust_state = RED. |
| Recovery Behaviour | BIM update failure: CONT_DEG — hold last valid score, log BIM_STALE. Spoof detected: RETRY_ALT — gate GNSS, transition to TRN-primary, log SPOOF_DETECTED. bim_trust_state stays RED until 10 consecutive clean GNSS readings with score > 0.70. |
| Logging (mandatory) | bim_trust_score, bim_trust_state at 10 Hz. SPOOF_DETECTED: {injection_offset_m, detection_latency_ms, score_at_detection}. GNSS_GATED: {reason, bim_trust_state}. BIM_STALE: {age_ms}. BIM_RESTORED after recovery. |
| Frozen Constants | trust→RED threshold < 0.10, detection_latency ≤ 250 ms, update_rate ≥ 10 Hz (FR-101) |

## 2.6 NAV-06 — Two-Theatre Navigation Architecture

| Req ID | NAV-06 |
| --- | --- |
| Title | Eastern vs Western Corridor Navigation Mode Selection |
| Priority | High |
| Description | The Navigation Manager shall select the primary navigation architecture based on corridor designation before GNSS denial. Eastern Himalayan corridor (σ_terrain ≥ 10 m): TRN-primary, VIO-secondary. Western plains corridor (σ_terrain < 10 m, high EW density): VIO-primary, TRN suppressed unless σ_terrain rises above threshold. σ_terrain is computed as the standard deviation of RADALT readings over 1 km in the PRE_GNSS_DENIAL phase. Architecture is fixed at mission start; mid-mission switching requires an explicit Retask Command. |
| Inputs | terrain_roughness_sigma_m (computed from RADALT readings over 1 km), corridor_designation (EASTERN/WESTERN), phase_params. |
| Outputs | nav_architecture (TRN_PRIMARY/VIO_PRIMARY), otm_enabled (bool), architecture_logged (bool). |
| Success Threshold | Correct architecture selected and logged before first GNSS denial. σ_terrain threshold = 10 m. Architecture entry in mission log before km 1. |
| Failure Threshold | TRN-primary selected when σ_terrain < 10 m in western corridor. Architecture not logged. |
| Recovery Behaviour | Incorrect architecture detected post-denial: CONT_DEG — log ARCHITECTURE_MISMATCH, continue with selected architecture, flag for post-mission review. No mid-mission switch without Retask Command. |
| Logging (mandatory) | NAV_ARCHITECTURE_SELECTED: {architecture, corridor, sigma_terrain_m, mission_km=0}. |

# 3. Electronic Warfare Requirements

## 3.1 EW-01 — EW Cost Map Update Rate

| Req ID | EW-01 |
| --- | --- |
| Title | EW Threat Cost Map Update Latency |
| Priority | High |
| Description | The EW Manager shall update the EW cost map within 500 ms of receiving a new EW observation. The cost map shall cover ≥ 10 km radius at ≤ 100 m grid resolution. Stale maps (age > 2 s) shall be flagged and the Route Planner informed. |
| Inputs | EW observation event: {frequency_mhz, bearing_deg, rssi_dbm, type (JAMMER/SPOOFER/UNKNOWN)}, vehicle position from Navigation Manager. |
| Outputs | ew_cost_map (defined in §10.4), map_stale_flag (bool), map_age_ms, threat_count. |
| Success Threshold | Update latency ≤ 500 ms from event receipt. Coverage ≥ 10 km radius. Grid resolution ≤ 100 m. map_stale_flag=True when map_age_ms > 2000. |
| Failure Threshold | Update latency > 1 s. Coverage < 5 km. map_stale_flag not set when map_age_ms > 2000. |
| Recovery Behaviour | Update failure: CONT_DEG — retain previous map, set map_stale_flag=True, log EW_MAP_STALE. Route Planner applies 20% additional threat weight margin to stale maps. If map age > 10 s: HOLD_PATH until updated or timeout 30 s then REPLAN with last known map. |
| Logging (mandatory) | ew_cost_map_update_latency_ms, threat_count, map_coverage_km at each update. EW_MAP_STALE: {age_ms, threat_count}. |

## 3.2 EW-02 — Route Replanning Under EW Threat

| Req ID | EW-02 |
| --- | --- |
| Title | EW-Triggered Route Replanning |
| Priority | High |
| Description | When a new EW threat intersects the planned route within 5 km lookahead, the Mission Manager shall trigger route replanning within 1 s. The new route shall maintain ≥ 500 m clearance from the threat envelope boundary and must not enter any No-Fly Zone. |
| Inputs | Updated ew_cost_map, current vehicle position, current waypoint, destination waypoint, no_fly_zones[], terrain_exclusion_zones[]. |
| Outputs | new_route (WaypointList), replan_latency_ms, threat_clearance_m, replan_trigger_reason, route_validated (bool). |
| Success Threshold | replan_latency_ms ≤ 1000. threat_clearance_m ≥ 500. No NFZ or terrain exclusion zone entered. route_validated=True. |
| Failure Threshold | replan_latency_ms > 2000. threat_clearance_m < 200. New route enters NFZ. |
| Recovery Behaviour | Timeout (> 2 s): CONT_DEG — continue on current route with threat_active_flag=True, reattempt replan every 5 s up to 3 attempts. After 3 failures: REPLAN with relaxed constraint level 1. No valid route after level 3: ROUTE_DEADEND → SHM. |
| Logging (mandatory) | ROUTE_REPLAN: {trigger_reason, replan_latency_ms, threat_clearance_m, waypoint_count, constraint_level}. ROUTE_DEADEND: {constraint_level_reached, attempt_count, vehicle_position}. |

## 3.3 EW-03 — GNSS Spoof Rejection via BIM

| Req ID | EW-03 |
| --- | --- |
| Title | GNSS Spoof Rejection via BIM Trust Arbitration |
| Priority | Critical |
| Description | A GNSS spoofing event shall be detected by the Navigation Manager's BIM component within 250 ms. The spoofed position shall not be incorporated into the ESKF. The system shall transition to TRN-primary navigation immediately on detection. |
| Inputs | Spoofed GNSS measurement (offset from true position). BIM state machine. |
| Outputs | spoof_detected=True, bim_trust_state=RED, gnss_gated=True, nav_mode=TRN_PRIMARY, detection_latency_ms. |
| Success Threshold | detection_latency_ms ≤ 250. gnss_gated=True within 50 ms of detection. INS drift after rejection < 50 m in 60 s with TRN active. |
| Failure Threshold | Spoofed position in ESKF state. detection_latency_ms > 500. gnss_gated=False while bim_trust_state=RED. |
| Recovery Behaviour | Spoof detected: RETRY_ALT — gate GNSS immediately, transition to TRN-primary, log SPOOF_DETECTED. bim_trust_state remains RED until 10 consecutive clean readings. If TRN unavailable: CONT_DEG on VIO+INS, log SPOOF_TRN_FALLBACK. |
| Logging (mandatory) | SPOOF_DETECTED: {injection_offset_m, detection_latency_ms, bim_trust_score_at_detection, nav_mode_before, nav_mode_after}. GNSS_GATED: {reason=SPOOF, timestamp_ms}. |

# 4. Route Planning Requirements

## 4.1 PLN-01 — Route Planner Response Time

| Req ID | PLN-01 |
| --- | --- |
| Title | Hybrid A* Route Planner Response Time |
| Priority | High |
| Description | The Hybrid A* planner shall return a valid route for a 10 km planning horizon on a 100 m grid within 2 s nominal, 5 s maximum. Route length shall not exceed 120% of straight-line distance. Route shall not intersect any EW Exclusion Zone or No-Fly Zone. |
| Inputs | start position (north_m, east_m), destination waypoint, ew_cost_map, no_fly_zones[], terrain_exclusion_zones[], constraint_relaxation_level (0–3, default 0). |
| Outputs | waypoint_list (WaypointList), route_length_km, route_ew_cost (0.0–1.0), plan_latency_ms, route_validated (bool), constraint_level_used. |
| Preconditions | EW cost map loaded (age ≤ 10 s). No-Fly Zones loaded. DEM tile loaded. Start and destination within DEM coverage. |
| Success Threshold | plan_latency_ms ≤ 2000 nominal, ≤ 5000 maximum. route_length_km ≤ 1.2 × straight_line_km. route_validated=True. constraint_level_used=0 preferred. |
| Failure Threshold | plan_latency_ms > 5000. No valid route at constraint level 0–3. route_length_km > 2.0 × straight_line. |
| Recovery Behaviour | Timeout at 5 s: return best partial route found with partial_route_flag=True. No valid route at level 0: REPLAN at level 1. No valid route at level 3: ROUTE_DEADEND → SHM if airborne. Constraint relaxation: level 1 = relax NFZ buffer 50%; level 2 = relax EW clearance to 200 m; level 3 = relax terrain constraint. |
| Logging (mandatory) | ROUTE_PLANNED: {plan_latency_ms, waypoint_count, route_length_km, route_ew_cost, constraint_level_used}. PLAN_TIMEOUT. ROUTE_DEADEND: {constraint_level, attempt_count}. |

## 4.2 PLN-02 — Dynamic Retasking

| Req ID | PLN-02 |
| --- | --- |
| Title | Dynamic Retask — Mid-Mission Target and Route Update |
| Priority | Medium |
| Description | On receipt of a valid signed Retask Command, the system shall execute the following ordered sequence: (1) regenerate terrain corridor from the new target position and updated mission envelope before any route computation begins — terrain generation uses the C-3 committed DEM parameterisation for the target mission phase; (2) check EW map staleness before invoking the Route Planner — if map_stale_flag=True, wait up to 2 s for a fresh update, then proceed with stale map at elevated threat weight if no update arrives; (3) compute new route using Hybrid A* against updated terrain and EW map; (4) validate new route and new envelope — envelope validation must complete before any PX4 waypoint upload; (5) only on successful validation, upload new waypoint queue to PX4 and resume mission. The entire sequence shall complete within 15 s. Retasking is permitted in mission phases CRUISE and GNSS_DENIED. Retasking is prohibited in TERMINAL_APPROACH and COMMITTED. |
| Inputs | RetaskCommand (see §10.12): new_target_position, new_mission_envelope (signed), new_no_fly_zones[] (optional), new_ew_exclusion_zones[] (optional). |
| Outputs | retask_accepted (bool), retask_latency_ms, retask_rejection_reason (if rejected), new_route (WaypointList), new_envelope_valid (bool), retask_ew_stale (bool). |
| Preconditions | Retask Command received with valid signature (SHA-256 hash verified). Vehicle in CRUISE or GNSS_DENIED phase. Navigation Manager has a position estimate. Terrain corridor for new target phase must be generatable from C-3 DEM parameterisation. |
| Success Threshold | new_envelope_valid=True. retask_latency_ms ≤ 15000. Terrain corridor regenerated before route computation. EW map staleness checked before route computation. Envelope validated before PX4 waypoint upload. New route avoids all NFZs and EW Exclusion Zones. |
| Failure Threshold | retask_latency_ms > 15000. Envelope validation bypassed. PX4 waypoints uploaded before new_envelope_valid=True. Retask accepted in TERMINAL_APPROACH or COMMITTED phase. |
| Recovery Behaviour | Invalid signature: CONT_NOM — reject within 1 s, log RETASK_REJECTED_INVALID_ENVELOPE. Invalid phase: CONT_NOM — reject, log RETASK_REJECTED_INVALID_PHASE. Terrain generation failure: CONT_NOM — reject retask, log RETASK_TERRAIN_GEN_FAILED, previous terrain and route remain active. EW map stale at retask time: wait 2 s for fresh update; if none, proceed with stale map, set retask_ew_stale=True, log RETASK_EW_MAP_STALE, Route Planner applies 20% additional threat weight margin (EW-01 rule). Route computation failure at level 0: RETRY (1×), then REPLAN at constraint level 1. No valid route after level 1: log RETASK_ROUTE_FAILED, enter ROLLBACK — restore previous route, restore previous mission envelope, restore previous EW map from Checkpoint ew_map_snapshot_path, restore previous terrain corridor, restore previous ETA; log RETASK_ROLLBACK: {previous_target, restored_ew_map_age_ms, restored_terrain_phase, reason}. Rollback shall complete within 2000 ms. Retask timeout > 15 s: ABORT_MISS, log RETASK_TIMEOUT. |
| Logging (mandatory) | RETASK_RECEIVED: {retask_id, target_position, phase_at_receipt, timestamp_ms}. RETASK_ACCEPTED: {retask_latency_ms}. RETASK_REJECTED: {reason}. RETASK_EW_MAP_STALE: {map_age_ms}. RETASK_TERRAIN_GEN_FAILED: {phase, reason}. RETASK_ROUTE_FAILED: {constraint_level}. RETASK_ROLLBACK: {previous_target, restored_ew_map_age_ms, restored_terrain_phase, reason}. RETASK_COMPLETE: {new_target, new_route_length_km}. |
| Retask Phase Matrix | PRE_GNSS_DENIAL: PERMITTED — GNSS-aided, full accuracy. CRUISE: PERMITTED — GNSS-aided, full accuracy. GNSS_DENIED with VIO/TRN active: PERMITTED — route uses nav uncertainty margin. GNSS_DENIED with INS_ONLY (both VIO and TRN unavailable): PERMITTED WITH CONSTRAINT — retask accepted only if cross_track_error_m ≤ (route_corridor_half_width_m − 100); if this condition is not met, reject retask, enter HOLD_PATH, log RETASK_NAV_CONFIDENCE_TOO_LOW: {cross_track_error_m, corridor_half_width_m, deficit_m}, await nav confidence improvement or operator SHM clearance. TERMINAL_APPROACH: PROHIBITED — phase lock active. COMMITTED: PROHIBITED — point of no return. SHM (trigger=ROUTE_DEADEND): PERMITTED — retask is primary SHM exit path. ABORT: PROHIBITED. |

## 4.3 PLN-03 — Route Dead-End Recovery

| Req ID | PLN-03 |
| --- | --- |
| Title | Route Dead-End Detection and Recovery |
| Priority | High |
| Description | When no valid route exists after attempting all constraint relaxation levels (0→3), the Mission Manager shall declare ROUTE_DEADEND, enter Safe Hold Mode, and escalate to operator. The constraint relaxation trace shall be logged in full. |
| Inputs | Hybrid A* plan results at constraint levels 0, 1, 2, 3. |
| Outputs | route_deadend_declared (bool), constraint_trace (list of {level, reason, attempt_latency_ms}), shm_activated (bool). |
| Success Threshold | ROUTE_DEADEND declared after exhausting all 4 constraint levels. SHM entered within 2 s of declaration. Full constraint trace logged. |
| Failure Threshold | System attempts to fly without valid route. SHM not entered within 5 s. |
| Recovery Behaviour | ROUTE_DEADEND: SHM. ESCALATE_OP — await Retask Command for 120 s. On valid Retask: exit SHM, replan. On timeout: ABORT_MISS. |
| Logging (mandatory) | ROUTE_DEADEND: {vehicle_position, constraint_trace[], attempt_count, mission_km}. SHM_ACTIVATED: {reason=ROUTE_DEADEND}. OPERATOR_ACTION_REQUIRED. |

# 5. Mission Management Requirements

## 5.1 MM-01 — Mission Phase State Machine Transitions

| Req ID | MM-01 |
| --- | --- |
| Title | Mission Phase FSM Transition Latency |
| Priority | Critical |
| Description | The Mission Manager FSM (7-state NanoCorteX machine) shall complete all state transitions within 2 s of the triggering event. Only defined edges shall be traversed. The full FSM is defined in §10.4. Transitions to SHM from any state shall complete within 2 s. |
| Inputs | Trigger event (GNSS_LOST, TERMINAL_APPROACH_INITIATED, FAULT_DETECTED, OPERATOR_ABORT, L10S_ABORT, OFFBOARD_LOSS, ROUTE_DEADEND). |
| Outputs | new_fsm_state, transition_latency_ms, transition_reason, FSM_TRANSITION event. |
| Success Threshold | Transition latency ≤ 2 s. Only defined edges traversed. No undefined state entered. No circular transition. |
| Failure Threshold | Transition latency > 5 s. Undefined state entered. More than 1 circular transition in 60 s. |
| Recovery Behaviour | Transition timeout (> 5 s): PROC_RESTART — force FSM to SHM state, log FSM_TRANSITION_TIMEOUT, restart Mission Manager process. Undefined state: PROC_RESTART — halt FSM, log FSM_UNDEFINED_STATE, emergency SHM command to PX4 direct. |
| Logging (mandatory) | FSM_TRANSITION: {from_state, to_state, trigger_event, latency_ms} at every transition. FSM_TRANSITION_TIMEOUT: {from_state, trigger_event, latency_at_timeout_ms}. FSM_UNDEFINED_STATE: {unexpected_state_value}. |
| Frozen Constants | Transition latency ≤ 2 s (NFR-002) |

## 5.2 MM-02 — Mission Envelope Validation

| Req ID | MM-02 |
| --- | --- |
| Title | Signed Mission Envelope Enforcement |
| Priority | Critical |
| Description | The Mission Manager shall validate the signed mission envelope (SHA-256 hash) before authorising any terminal action. Geographic boundary check, target class check, and time window check must all pass. Invalid envelope = immediate rejection. The envelope schema is defined in §10.2. |
| Inputs | Mission envelope (MissionEnvelope schema), proposed terminal action type, current vehicle position, current UTC time. |
| Outputs | envelope_valid (bool), action_authorised (bool), violation_type (GEOGRAPHIC/TARGET_CLASS/TIME_WINDOW/SIGNATURE/NONE). |
| Success Threshold | 100% of terminal actions validated before execution. Invalid signature rejected within 200 ms. No out-of-boundary, wrong-class, or out-of-window actions authorised. |
| Failure Threshold | Any terminal action executed without valid envelope. Any violation not caught. |
| Recovery Behaviour | Violation detected: ABORT_TERM — halt terminal sequence, log ENVELOPE_VIOLATION, enter SHM. |
| Logging (mandatory) | ENVELOPE_VALIDATED: {action_type, violation_type=NONE, timestamp_ms}. ENVELOPE_VIOLATION: {violation_type, proposed_action, boundary_or_class_or_window_exceeded}. |
| AVP Constraint Enforcement | The Mission Envelope shall carry AVP-specific speed, altitude, and survivability constraints via the schema fields defined in Section 10.2. The Mission Manager shall verify that vehicle operational parameters remain within the AVP profile envelope throughout the mission. Breach of AVP constraints shall be logged as AVP_FALLBACK events per Section 1.3 — they do not constitute an ENVELOPE_VIOLATION unless they also breach the geographic boundary, target class, or time window constraints. |

## 5.3 MM-03 — Safe Hold Mode Activation

| Req ID | MM-03 |
| --- | --- |
| Title | Safe Hold Mode Entry Triggers and Behaviour |
| Priority | Critical |
| Description | The Mission Manager shall activate Safe Hold Mode (SHM) within 2 s of any of the following triggers: (a) bim_trust_score < 0.05 for > 5 s, (b) ENVELOPE_VIOLATION, (c) OPERATOR_ABORT command, (d) OFFBOARD Loss > 5 s, (e) L10s-SE ABORT decision, (f) ROUTE_DEADEND, (g) FSM_UNDEFINED_STATE, (h) corridor_breach_flag=True for > 15 s. In SHM: command PX4 to loiter at current position. Do not advance. Do not execute terminal approach. |
| Inputs | SHM trigger event (one of triggers a–h above), current vehicle position, PX4 OFFBOARD status. |
| Outputs | shm_active (bool), shm_trigger_reason, shm_entry_latency_ms, hold_position (north_m, east_m, alt_m). |
| Success Threshold | SHM entry latency ≤ 2 s from any trigger. PX4 commanded to loiter within 2 s. SHM_ACTIVATED logged with trigger_reason. |
| Failure Threshold | SHM entry > 5 s from trigger. PX4 not in loiter after 5 s of SHM. |
| Recovery Behaviour | SHM is the recovery state. Exit requires explicit Retask Command (if trigger=ROUTE_DEADEND or bim_trust_score recovery), or trigger clearance, or OPERATOR_ABORT clearance. If SHM active > 120 s: ESCALATE_OP. If SHM active > 240 s: ABORT_MISS. |
| Logging (mandatory) | SHM_ACTIVATED: {trigger_reason, vehicle_position, mission_km, timestamp_ms, fsm_state_at_entry}. SHM_DEACTIVATED: {recovery_reason, shm_duration_ms}. |

## 5.4 MM-04 — Internal Message Queue Latency

| Req ID | MM-04 |
| --- | --- |
| Title | Critical Event Message Queue Latency |
| Priority | High |
| Description | The internal event bus shall deliver critical events (SHM trigger, OFFBOARD Loss, ENVELOPE_VIOLATION, SPOOF_DETECTED, CORRIDOR_BREACH) to the Mission Manager within 100 ms of generation. Queue utilisation shall not exceed 80% of capacity under nominal load. |
| Inputs | Critical events from Navigation Manager, EW Manager, L10s-SE, BIM, PX4 Bridge. |
| Outputs | event_delivery_latency_ms, queue_utilization_percent, queue_overflow_count. |
| Success Threshold | Critical event latency ≤ 100 ms. queue_utilization_percent ≤ 80% nominal. queue_overflow_count = 0 for critical events. |
| Failure Threshold | Critical event latency > 500 ms. queue_overflow_count > 0 for critical events. |
| Recovery Behaviour | Queue utilisation > 80%: CONT_DEG — drop INFO-level events only, log QUEUE_HIGH. Queue overflow dropping critical event: PROC_RESTART — log QUEUE_CRITICAL_OVERFLOW, restart event bus process. |
| Logging (mandatory) | Queue metrics (utilization, depth) at 1 Hz. QUEUE_HIGH: {utilization_percent}. QUEUE_CRITICAL_OVERFLOW: {dropped_event_type, timestamp_ms}. |

# 6. PX4 Integration Requirements

## 6.1 PX4-01 — OFFBOARD Setpoint Rate

| Req ID | PX4-01 |
| --- | --- |
| Title | OFFBOARD Setpoint Command Rate and Continuity |
| Priority | Critical |
| Description | The PX4 Bridge shall publish SET_POSITION_TARGET_LOCAL_NED setpoints at ≥ 20 Hz continuously. OFFBOARD continuity shall be ≥ 99.5% of mission duration. Setpoints shall be buffered locally for up to 200 ms to absorb transient MAVLink gaps without rate drop below 20 Hz. On MAVLink link recovery after any gap, the PX4 Bridge shall NOT replay buffered stale setpoints — the buffer shall be discarded and a fresh setpoint issued from the current Navigation Manager state. Stale setpoints are maintained only to prevent PX4 from exiting OFFBOARD during a transient gap and are not a navigation control input. |
| Inputs | NanoCorteX setpoint command (position_target or velocity_target). PX4 OFFBOARD mode acknowledgment. |
| Outputs | MAVLink SET_POSITION_TARGET_LOCAL_NED at ≥ 20 Hz. setpoint_rate_hz (measured). offboard_active (bool). stale_setpoint_age_ms. stale_setpoints_discarded_on_recovery (bool, always True). |
| Success Threshold | setpoint_rate_hz ≥ 20 continuously. OFFBOARD continuity ≥ 99.5% over 30 min. stale_setpoint_age_ms ≤ 200. ≤ 1 unintended OFFBOARD Loss per 30-minute mission. stale_setpoints_discarded_on_recovery=True on every link recovery. |
| Failure Threshold | setpoint_rate_hz < 10 for > 200 ms. OFFBOARD continuity < 99.0%. stale_setpoint_age_ms > 400. |
| Recovery Behaviour | Rate drop below 20 Hz: CONT_DEG — increase scheduler priority, log SETPOINT_RATE_LOW. OFFBOARD Loss: RETRY (attempt re-entry every 1 s for up to 5 s). After 5 s no re-entry: SHM. After re-entry: discard buffer, issue fresh setpoint from current nav state, log OFFBOARD_RESTORED. |
| Logging (mandatory) | setpoint_rate_hz, offboard_active at 1 Hz. SETPOINT_RATE_LOW: {measured_rate_hz}. OFFBOARD_LOSS: {gap_ms, recovery_attempt_count}. OFFBOARD_RESTORED: {gap_duration_ms, stale_setpoints_discarded=True}. |
| Note | NVIDIA RTX 5060 Ti: __EGL_VENDOR_LIBRARY_FILENAMES=10_nvidia.json required in px4-rc.gzsim. Mesa gallium crashes on this GPU. |

## 6.2 PX4-02 — MAVLink Heartbeat and Reconnection

| Req ID | PX4-02 |
| --- | --- |
| Title | MAVLink Heartbeat Monitoring and Reconnection |
| Priority | High |
| Description | The PX4 Bridge shall monitor the MAVLink HEARTBEAT from PX4 (expected at 1 Hz). Heartbeat loss > 2 s = STALE. Loss > 10 s = LOST. On LOST: activate SHM and attempt reconnection. Reconnect sequence: close socket, wait 500 ms, reopen on same port, retry 1 Hz for up to 30 s. |
| Inputs | MAVLink HEARTBEAT message at 1 Hz from PX4 (system ID filtered per SIA v1.0). |
| Outputs | heartbeat_status (ALIVE/STALE/LOST), last_heartbeat_ms, reconnect_attempts, reconnect_latency_ms. |
| Success Threshold | STALE declared at 2 s gap. LOST declared at 10 s gap. Reconnect within 5 s nominal, 30 s maximum. SHM entered on LOST and deactivated on reconnect. |
| Failure Threshold | STALE not declared at 2 s. Reconnect > 30 s. SHM not entered on LOST. |
| Recovery Behaviour | STALE (2–10 s): RETRY — reconnect at 1 Hz. LOST (> 10 s): SHM + continue RETRY at 1 Hz for 30 s. Reconnect success: log HEARTBEAT_RESTORED, discard stale setpoint buffer, issue fresh setpoint, restore setpoint stream. No reconnect at 30 s: ABORT_MISS. |
| Logging (mandatory) | HEARTBEAT_STALE: {gap_ms}. HEARTBEAT_LOST: {gap_ms}. HEARTBEAT_RESTORED: {gap_ms, reconnect_attempts}. RECONNECT_FAILED: {total_gap_ms}. |

## 6.3 PX4-03 — Sensor Driver MAVLink Interface

| Req ID | PX4-03 |
| --- | --- |
| Title | Sensor Driver Interface via MAVLink Subscriber |
| Priority | High |
| Description | IMU, GNSS, and RADALT sensor data shall be received exclusively via MAVLink subscriber interfaces. MicroMind is a listener on the sensor bus only and does not own or supply sensors. The EO camera driver uses MIPI/USB3 direct (not MAVLink). All sensor drivers are stubs implemented as MAVLink subscribers per SIA v1.0. |
| Inputs | MAVLink HIL_SENSOR (IMU: accel_xyz, gyro_xyz) at 200 Hz. MAVLink GPS_RAW_INT (GNSS) at 1 Hz. MAVLink DISTANCE_SENSOR (RADALT) at 10 Hz. |
| Outputs | imu_measurement (IMUSample), gnss_measurement (GNSSSample), radalt_m (float). |
| Success Threshold | IMU 200 Hz ±5%. GNSS 1 Hz ±10%. RADALT 10 Hz ±10%. Dropped messages < 1% over any 30-minute window. |
| Failure Threshold | IMU dropout > 100 ms. GNSS dropout > 5 s. RADALT dropout > 10 s. |
| Recovery Behaviour | IMU dropout > 100 ms: CONT_DEG — freeze ESKF propagation, log IMU_DROPOUT. GNSS dropout > 5 s: RETRY_ALT — begin GNSS-denied nav sequence. RADALT dropout > 10 s: CONT_DEG — suspend TRN, log TRN_RADALT_SUSPENDED. |
| Logging (mandatory) | message_rate_hz per sensor at 1 Hz. IMU_DROPOUT, GNSS_DROPOUT, RADALT_DROPOUT: {duration_ms, last_valid_ms}. |

## 6.4 PX4-04 — PX4 Reboot Recovery

| Req ID | PX4-04 |
| --- | --- |
| Title | PX4 Process Reboot Recovery and State Restore |
| Priority | High |
| Description | If PX4 reboots during mission, the PX4 Bridge shall detect the reboot within 3 s (via HEARTBEAT sequence number reset or SYS_STATUS reset), re-establish MAVLink, and restore OFFBOARD mode. After Checkpoint restore, the Mission Manager shall evaluate the restored Checkpoint state before autonomous operation resumes: if pending_operator_clearance_required=True, the Mission Manager shall remain in SHM and command PX4 to loiter, awaiting explicit operator clearance before proceeding; if pending_operator_clearance_required=False and no abort or active SHM condition exists in the Checkpoint, the Mission Manager shall validate the restored state and autonomous operation may resume automatically. Mission shall not advance until this evaluation is complete. If PX4 returns in HOLD instead of OFFBOARD, the bridge shall command OFFBOARD mode re-entry. |
| Inputs | MAVLink HEARTBEAT (seq number), SYS_STATUS.onboard_control_sensors_health, Checkpoint (full schema per §10.15 including shm_active, shm_trigger_reason, pending_operator_clearance_required, mission_abort_flag). |
| Outputs | reboot_detected (bool), offboard_restored (bool), waypoint_restored (bool), autonomous_resume_approved (bool), reboot_recovery_latency_ms. |
| Success Threshold | Reboot detected ≤ 3 s. OFFBOARD restored ≤ 15 s. Checkpoint loaded and Mission Manager evaluation complete before first autonomous setpoint. Mission resumes with no position jump > 20 m if pending_operator_clearance_required=False. |
| Failure Threshold | Reboot undetected > 10 s. OFFBOARD not restored in 30 s. Autonomous operation resumes before Mission Manager evaluation. Position jump > 50 m on resume. |
| Recovery Behaviour | Reboot detected: SHM immediately, begin reconnect (PX4-02 RETRY). OFFBOARD restored: CHECKPOINT_RESTORE — load Checkpoint, execute Mission Manager evaluation gate (D8a). If pending_operator_clearance_required=False and no abort/SHM in Checkpoint: autonomous_resume_approved=True, command waypoint queue, log MISSION_RESUMED. If pending_operator_clearance_required=True: remain in SHM, log AWAITING_OPERATOR_CLEARANCE. PX4 in HOLD after reconnect: command SET_MODE OFFBOARD, retry 3× at 2 s. After 3 failures: ABORT_MISS. |
| Logging (mandatory) | PX4_REBOOT_DETECTED: {seq_reset_value, recovery_start_ms}. OFFBOARD_RESTORED: {recovery_latency_ms}. CHECKPOINT_RESTORED: {checkpoint_age_ms, waypoint_index}. AUTONOMOUS_RESUME_APPROVED: {pending_operator_clearance_required=False}. AWAITING_OPERATOR_CLEARANCE: {shm_trigger_reason}. MISSION_RESUMED: {position_discrepancy_m}. |

## 6.5 PX4-05 — Checkpoint Save and Restore

| Req ID | PX4-05 |
| --- | --- |
| Title | Navigation Checkpoint Save and Restore |
| Priority | High |
| Description | The system shall save a Checkpoint every 5 km of mission progress and on every mission phase transition. Checkpoints shall be written to persistent storage within 200 ms. The Checkpoint schema is defined in §10.15 and includes the complete Mission Manager safety state: shm_active, shm_trigger_reason, pending_operator_clearance_required, and mission_abort_flag. It also includes eta_to_destination_ms and mission_start_utc_ms to enable ETA recomputation after restore. On process restart (RS-03) or PX4 reboot (PX4-04), the most recent valid Checkpoint shall be loaded and the navigation state restored. Checkpoint retention follows a rolling window policy: a maximum of 5 valid Checkpoints are retained; on each new write, the oldest is deleted. If disk free < 500 MB, retention is reduced to 2. If disk free < 100 MB, writes are suspended and the last valid Checkpoint is held in RAM only. |
| Inputs | Current USV, active_nav_mode, bim_trust_state, current_waypoint_index, ew_cost_map snapshot, retask_state, shm_active, shm_trigger_reason, pending_operator_clearance_required, mission_abort_flag, eta_to_destination_ms, mission_start_utc_ms. |
| Outputs | checkpoint_written (bool), checkpoint_path (str), checkpoint_age_ms, checkpoint_valid (bool), checkpoints_retained_count. |
| Success Threshold | Checkpoint written ≤ 200 ms after trigger. Every Checkpoint valid (passes integrity check). Restore latency ≤ 2 s from checkpoint load to ESKF running. ≤ 5 checkpoints on disk at any time. After restore: ETA recomputed and envelope time window check re-evaluated. |
| Failure Threshold | Checkpoint write fails 3 consecutive times. Checkpoint restore fails integrity check. Restore latency > 10 s. More than 5 checkpoints accumulate without purge. |
| Recovery Behaviour | Write failure: RETRY (3× at 200 ms). After 3 failures: CONT_DEG — log CHECKPOINT_WRITE_FAILED, continue without checkpoint. Corrupt checkpoint: CHECKPOINT_RESTORE from previous checkpoint. No valid checkpoint: PROC_RESTART with clean nav initialisation, log NO_VALID_CHECKPOINT. After restore, Mission Manager recomputes ETA and logs CHECKPOINT_ETA_RECOMPUTED: {original_eta_ms, restored_eta_ms, envelope_remaining_ms}. If restored ETA exceeds envelope time_window_end_utc: log ENVELOPE_TIME_WINDOW_EXPIRED, enter SHM. |
| Logging (mandatory) | CHECKPOINT_SAVED: {mission_km, timestamp_ms, trigger_reason, checkpoints_retained_count}. CHECKPOINT_PURGED: {purged_checkpoint_id, purge_reason=ROLLING_WINDOW}. CHECKPOINT_RETENTION_REDUCED: {reason, max_retained}. CHECKPOINT_RESTORED: {checkpoint_age_ms, trigger_reason}. CHECKPOINT_ETA_RECOMPUTED: {original_eta_ms, restored_eta_ms, envelope_remaining_ms}. CHECKPOINT_WRITE_FAILED. CHECKPOINT_CORRUPT. NO_VALID_CHECKPOINT. |

# 7. Terminal Guidance Requirements

## 7.1 TERM-01 — DMRL Target Lock Confidence

| Req ID | TERM-01 |
| --- | --- |
| Title | DMRL Target Lock Confidence Threshold and Dwell |
| Priority | Critical |
| Description | The DMRL engine shall achieve lock_confidence ≥ 0.85 before authorising terminal approach. Lock shall be maintained for ≥ 5 consecutive frames at 25 FPS (200 ms minimum dwell). Aimpoint correction shall not exceed ±15°. Reacquisition timeout is 1.5 s from lock loss. |
| Inputs | EO/IR frame at 25 FPS, thermal target signature, background temperature model, current navigation state from USV. |
| Outputs | lock_confidence (0.0–1.0), locked_target_position_m, dwell_frame_count (int), approach_authorised (bool), aimpoint_correction_deg (float ±15). |
| Success Threshold | lock_confidence ≥ 0.85 for ≥ 5 consecutive frames. approach_authorised=True only after dwell ≥ 5 frames. aimpoint_correction_deg ≤ ±15. |
| Failure Threshold | lock_confidence < 0.70 at terminal commit. dwell_frame_count < 5 at commit. aimpoint_correction_deg > ±15. |
| Recovery Behaviour | Insufficient confidence (0.70–0.85): CONT_DEG — hold approach, extend dwell up to 30 frames. Confidence collapse (< 0.70): ABORT_TERM — log DMRL_LOCK_FAILED, attempt re-acquire. Reacquisition timeout 1.5 s: ABORT_MISS. aimpoint > ±15°: ABORT_TERM, log AIMPOINT_EXCEEDED. |
| Logging (mandatory) | DMRL_UPDATE: {lock_confidence, dwell_frame_count, aimpoint_correction_deg} at 25 Hz. DMRL_LOCK_ACQUIRED: {confidence, mission_km}. DMRL_LOCK_FAILED: {reason, dwell_count, mission_km}. DMRL_ABORT: {reason, mission_km}. |
| Frozen Constants | Lock ≥ 0.85, dwell ≥ 5 frames @ 25 FPS, aimpoint ≤ ±15°, reacquisition_timeout = 1.5 s (FR-103) |

## 7.2 TERM-02 — Thermal Decoy Rejection

| Req ID | TERM-02 |
| --- | --- |
| Title | Thermal Decoy Detection and Rejection |
| Priority | Critical |
| Description | When a thermal decoy is present, DMRL shall detect and reject it without accepting it as the primary target. Decoy rejection is triggered when decoy_confidence ≥ 0.80 for ≥ 3 consecutive frames. Primary lock shall be maintained throughout. |
| Inputs | EO/IR frame at 25 FPS, thermal signatures of primary target and decoy, spatial separation. |
| Outputs | decoy_detected (bool), decoy_confidence (0.0–1.0), primary_lock_maintained (bool), decoy_rejection_frame_count (int), abort_triggered (bool). |
| Success Threshold | decoy_detected=True when decoy_confidence ≥ 0.80 for frame 1. Rejection triggered at frame 3. primary_lock_maintained=True throughout. approach_authorised remains True after rejection. |
| Failure Threshold | Decoy accepted as primary target. Primary lock lost during decoy event. |
| Recovery Behaviour | Decoy rejection confirmed: ABORT_TERM — log DECOY_REJECTED, re-acquire primary. Primary lock lost during decoy: ABORT_TERM — attempt re-acquire within 1.5 s. Re-acquire timeout: ABORT_MISS. |
| Logging (mandatory) | DECOY_DETECTED: {decoy_confidence, primary_confidence, frame_count, spatial_separation_m}. DECOY_REJECTED: {final_decoy_confidence, primary_lock_maintained}. PRIMARY_LOCK_LOST_DURING_DECOY. |
| Frozen Constants | Decoy abort threshold ≥ 0.80 for ≥ 3 frames (FR-103) |

## 7.3 TERM-03 — L10s-SE Compliance Gate

| Req ID | TERM-03 |
| --- | --- |
| Title | Last-10-Seconds Safety Envelope Compliance |
| Priority | Critical |
| Description | 100% of terminal approach commitments shall pass through the L10s-SE gate. The gate shall evaluate all L10sInputs and return a decision within 2 s. Civilian detection ≥ 0.70 shall trigger immediate ABORT_TERM. A failed gate shall never be bypassed. |
| Inputs | L10sInputs: {dmrl_output, civilian_detection_confidence, nav_state, mission_envelope, timestamp_ms}. |
| Outputs | L10sOutput: {decision (PROCEED/ABORT), abort_reason, decision_latency_ms, civilian_confidence, nav_confidence}. |
| Success Threshold | 100% gate coverage. decision_latency_ms ≤ 2000. civilian_detection_confidence ≥ 0.70 always produces decision=ABORT. |
| Failure Threshold | Any approach without L10s gate invocation. civilian_confidence ≥ 0.70 with decision=PROCEED. decision_latency_ms > 2000. |
| Recovery Behaviour | ABORT decision: ABORT_TERM — halt terminal sequence, log L10S_ABORT, re-evaluate after 5 s hold. Gate timeout (> 2 s): ABORT_TERM. Gate failure (crash): ABORT_MISS — log L10S_COMPONENT_FAILURE. |
| Logging (mandatory) | L10S_INVOKED: {mission_km, civilian_confidence}. L10S_DECISION: {decision, abort_reason, decision_latency_ms, civilian_confidence, nav_confidence}. L10S_ABORT: {abort_reason, mission_km}. L10S_TIMEOUT. L10S_COMPONENT_FAILURE. |
| Frozen Constants | Civilian threshold = 0.70 (FR-105), decision_latency ≤ 2 s (FR-105) |

# 8. Runtime Stability Requirements

## 8.1 RS-01 — CPU Utilisation Ceiling

| Req ID | RS-01 |
| --- | --- |
| Title | CPU Utilisation Under Mission Load |
| Priority | High |
| Description | The complete MicroMind stack shall operate within 90% CPU utilisation (mean over any 60 s window) on the target compute platform (Jetson Orin NX 8GB). Peak utilisation during replanning or terminal phase shall not exceed 95% for any window > 10 s. This requirement cannot be validated in SIL; it requires Jetson Orin NX hardware (see Gap GAP-06). |
| Inputs | System CPU metrics per core, current mission phase, thermal_state (NORMAL/THROTTLING). |
| Outputs | cpu_utilization_percent (mean, peak), thermal_state, throttle_active (bool). |
| Success Threshold | Mean CPU ≤ 70% during CRUISE. Peak ≤ 90% nominal. Peak ≤ 95% during REPLAN or TERMINAL. No thermal throttling during navigation-critical phase. |
| Failure Threshold | Mean CPU > 90% for > 60 s. Peak > 95% for > 10 s. Thermal throttle during GNSS_DENIED navigation phase. |
| Recovery Behaviour | CPU > 90%: CONT_DEG — reduce EW map update rate 2 Hz → 0.5 Hz, reduce VIO update fusion weight by 50%, log CPU_HIGH. Thermal throttle in navigation-critical phase: CONT_DEG — suspend non-critical telemetry logging, log THERMAL_THROTTLE_NAV_CRITICAL. |
| Logging (mandatory) | cpu_utilization_percent, thermal_state, throttle_active at 1 Hz. CPU_HIGH: {cpu_percent, phase}. THERMAL_THROTTLE_NAV_CRITICAL: {cpu_percent, thermal_temp_c}. |

## 8.2 RS-02 — Mission Log Completeness

| Req ID | RS-02 |
| --- | --- |
| Title | Mission Log Completeness, Retention, and Rolling Policy |
| Priority | High |
| Description | Mission telemetry log completeness shall be ≥ 99% of expected records over the full mission. Critical events (SHM_ACTIVATED, SPOOF_DETECTED, DMRL_ABORT, ENVELOPE_VIOLATION, OFFBOARD_LOSS, CORRIDOR_BREACH) shall never be dropped under any condition, including disk pressure. Log Rolling Policy: log files shall be rolled every 30 minutes or 200 MB, whichever comes first. Completed log files shall be gzip-compressed after rolling. Total log storage target is ≤ 2 GB for a 3-hour mission. CRITICAL events (P0) are duplicated to a separate non-rolling critical_events.log that is never rotated. |
| Inputs | All log records from all modules. Log buffer status. Disk free space. |
| Outputs | log_completeness_percent, dropped_records_total, dropped_critical_events_count, disk_free_mb, current_log_file_mb. |
| Success Threshold | log_completeness_percent ≥ 99%. dropped_critical_events_count = 0 always. Log buffer ≤ 80% full under nominal load. Log rolled every 30 min or 200 MB. Total log storage ≤ 2 GB for 3-hour mission. |
| Failure Threshold | log_completeness_percent < 95%. dropped_critical_events_count > 0 at any point. Total log storage > 3 GB. |
| Recovery Behaviour | Buffer > 80%: CONT_DEG — drop INFO-level events only, log LOG_BUFFER_HIGH. Disk < 200 MB free: CONT_DEG — suspend non-critical telemetry, retain critical events in 1000-event RAM ring buffer, log LOG_DISK_WARNING. Disk exhausted: CONT_DEG — critical events to RAM buffer only, log LOG_DISK_FULL. RAM buffer flush on disk recovery. Roll failure: RETRY (3×), then log LOG_ROLL_FAILED. |
| Logging (mandatory) | dropped_record_count at 1 Hz. LOG_ROLLED: {new_file_path, previous_file_size_mb}. LOG_BUFFER_HIGH: {buffer_percent}. LOG_DISK_WARNING: {disk_free_mb}. LOG_DISK_FULL. LOG_OVERFLOW: {dropped_count, dropped_level}. LOG_ROLL_FAILED. |
| Frozen Constants | Log completeness ≥ 99% (NFR-013) |

## 8.3 RS-03 — Process Restart Recovery

| Req ID | RS-03 |
| --- | --- |
| Title | Process Watchdog and Restart Recovery |
| Priority | Medium |
| Description | The process watchdog shall detect failure of any MicroMind process within 500 ms and initiate recovery. Navigation capability shall be restored from Checkpoint within 10 s of restart. Process Restartability Classification governs the watchdog behaviour on failure: RESTARTABLE WITHOUT SHM — EW Manager (holds no flight-critical state), Log Bus (restores from checkpoint); RESTARTABLE WITH SHM — Navigation Manager (restore from Checkpoint, nav paused during restart), Mission Manager (restore from Checkpoint, FSM state preserved), PX4 Bridge (OFFBOARD reacquired per PX4-04); NOT RESTARTABLE — ESKF core computation loop (if this crashes: ABORT_MISS immediately, no restart attempt). The watchdog shall use this classification to determine SHM-or-abort on each process failure. |
| Inputs | Process heartbeat from each module at 2 Hz. Process exit codes. Restartability classification per process. PX4 OFFBOARD status during restart. |
| Outputs | process_failure_detected (bool), restart_latency_ms, nav_restored_latency_ms, checkpoint_loaded (bool), restartability_class (str). |
| Success Threshold | Failure detected ≤ 500 ms. RESTARTABLE processes restarted ≤ 2 s. Nav capability from Checkpoint ≤ 10 s. Position error after restore < 20 m. NOT RESTARTABLE failure triggers ABORT_MISS within 2 s. |
| Failure Threshold | Failure undetected > 2 s. Nav not restored ≤ 30 s. NOT RESTARTABLE process failure does not trigger ABORT_MISS. |
| Recovery Behaviour | RESTARTABLE process failure: SHM (if RESTARTABLE WITH SHM), PROC_RESTART, CHECKPOINT_RESTORE. On nav restore: log MISSION_RESUMED. Restart failure (3× retries): ABORT_MISS. NOT RESTARTABLE failure: ABORT_MISS immediately, log ESKF_CORE_FAILURE. |
| Logging (mandatory) | PROCESS_FAILURE: {process_name, exit_code, restartability_class, restart_attempt, timestamp_ms}. PROCESS_RESTARTED: {restart_latency_ms}. NAV_RESTORED: {nav_restored_latency_ms, position_error_m}. MISSION_RESUMED: {checkpoint_age_ms}. ESKF_CORE_FAILURE: {process_name, exit_code}. |

## 8.4 RS-04 — Memory Consumption Bound

| Req ID | RS-04 |
| --- | --- |
| Title | Memory Consumption Bound During Endurance |
| Priority | Medium |
| Description | No module shall exhibit unbounded memory growth. Total stack RSS shall not increase by more than 50 MB over any 2-hour continuous run. route_planner_heap_mb is monitored separately as a retask-specific memory vector: on RETASK_ROLLBACK or RETASK_COMPLETE, the Route Planner shall release all partial route candidates from constraint levels 1–3 within 500 ms. If route_planner_heap_mb does not decrease within 1 s of retask completion, log ROUTE_PLANNER_MEMORY_HIGH. Across 5 retask attempts in 30 minutes, route_planner_heap_mb shall not accumulate beyond 20 MB. |
| Inputs | RSS per process sampled at 60 s intervals. OOM kernel events. route_planner_heap_mb sampled at 10 s intervals. |
| Outputs | rss_growth_mb_per_hour, oom_events_count, rss_total_mb, route_planner_heap_mb. |
| Success Threshold | rss_growth_mb_per_hour ≤ 25. oom_events_count = 0. Linear regression over 2-hour window shows bounded growth. route_planner_heap_mb decreases within 1 s of retask completion. route_planner_heap_mb ≤ 20 MB across 5 retask attempts. |
| Failure Threshold | rss_growth > 100 MB in 2 hours. Any OOM kill. route_planner_heap_mb grows unboundedly across retasks. |
| Recovery Behaviour | RSS growth > 50 MB at 1 hour: CONT_DEG — log MEMORY_WARNING, identify top consumer, trigger restart of that module if non-critical. OOM imminent (< 100 MB free): PROC_RESTART for largest non-critical process. route_planner_heap_mb > 20 MB: log ROUTE_PLANNER_MEMORY_HIGH, force release of all partial route candidates. |
| Logging (mandatory) | rss_total_mb, rss_per_process, route_planner_heap_mb at 60 s intervals. MEMORY_WARNING: {rss_growth_mb, top_consumer}. OOM_KILL: {process_name}. ROUTE_PLANNER_MEMORY_HIGH: {heap_mb, retask_count}. |

# 9. Mission Demonstration Tool

IMPLEMENTATION SEQUENCING NOTE: The Mission Demonstration Tool is not an immediate implementation priority. Demonstration capability is dependent on the successful completion of PX4 OFFBOARD continuity (EC-01), Checkpoint restore (EC-02), PX4 reboot recovery (EC-03), dynamic retasking (EC-04), 2-hour endurance stability (EC-05), queue behaviour (EC-08), and three-seed repeatability (EC-11). The visualisation and demonstration layer is a later-layer presentation system built on stable, proven mission behaviour. Development of the Live Mission Demo and tactical demonstration environment shall not begin until all prerequisite entry criteria in §17 are met.

Amit: This Section will be updated once we reach this Milestone (SB-5) after completing Tests in Section 1 thru 8.

Section 9 should be revised to reflect that the Mission Demonstration Tool is not a short replay clip or post-processed video artifact.

The intended end-state is a live operator-driven demonstration environment that proves MicroMind can command a PX4-based vehicle stack dynamically during mission execution.

The demonstration architecture should be defined as a two-layer system:

1. Tactical Live Demonstration Layer

2. Long-Range Mission Demonstration Layer

Tactical Live Demonstration Layer

Purpose:

- Demonstrate real-time command authority over PX4

- Demonstrate live GNSS-denied navigation continuity

- Demonstrate dynamic mission replanning

- Demonstrate operator-in-the-loop mission updates

- Demonstrate the difference between standard PX4 autonomy and PX4 with MicroMind autonomy layer

This mode should use a Gazebo-class simulation environment or equivalent live SITL/HITL environment.

The live display should show:

- Vehicle A = standard PX4 behaviour

- Vehicle B = PX4 + MicroMind autonomy layer

- Both vehicles moving simultaneously in real time

- Operator commands injected during flight

- GNSS denial event during motion

- Visible divergence after GNSS denial

- Vehicle B maintaining route continuity

- Vehicle A drifting, failing corridor, or requiring manual intervention

- Dynamic retask event during flight

- Updated corridor, target, or exclusion zone

- Live status panels for:

- navigation source

- trust state

- GNSS state

- VIO/TRN state

- EW state

- PX4 mode

- Mission Manager state

- SHM state

The live demonstration should allow the operator to change:

- Target coordinate

- No-fly zone

- EW exclusion zone

- Route corridor width

- Mission timing constraints

Vehicle B should respond live and visibly.

Long-Range Mission Demonstration Layer

Purpose:

- Demonstrate endurance, terrain reasoning, EW-aware routing, corridor management, and comparative drift behaviour over 100–150 km missions

- Provide a TASL / DISC-facing business demonstration that can be understood within 2–3 minutes

This mode should use:

- DEM-style terrain underlay

- Srinagar Valley-inspired terrain profile

- Plains, ridge crossings, mountain ingress, industrial terminal area

- Live mission timeline synchronized across all panels

- Side-by-side Vehicle A and Vehicle B comparison

- Log-driven event overlays

- Corridor breach markers

- GNSS denial markers

- EW reroute markers

- TRN correction markers

- Retask markers

- SHM markers

The long-range demonstration must not behave like a static HTML report or pre-rendered video.

It should behave like a live mission playback environment where the operator can:

- Pause

- Resume

- Scrub timeline

- Change speed

- Switch camera view

- Toggle overlays

- Inspect events

- Re-render the mission from logs without rerunning the simulation

Mandatory Design Principle

The purpose of the demonstration environment is not only to prove technical correctness.

It must make the operational difference between Vehicle A and Vehicle B visually obvious within the first 30–60 seconds.

The viewer should not need a technical explanation to understand:

- when GNSS failed

- when Vehicle A began to drift

- why Vehicle B maintained the mission

- when retasking occurred

- how MicroMind changed the outcome

Gazebo-class live demonstration capability should therefore not be treated as optional or post-TASL scope. It is part of the intended MicroMind product narrative and should remain in the long-term roadmap even if implementation is phased after regression hardening.

## 9.1 Purpose and Architecture

The Mission Demonstration Tool is the external-facing presentation layer of the MicroMind programme. Its purpose is to make the operational difference between Vehicle A (standard PX4, no MicroMind) and Vehicle B (PX4 + MicroMind autonomy layer) visually obvious to a non-technical audience within 30–60 seconds, without requiring verbal explanation.

The intended end-state is not a short replay clip or a pre-rendered video. It is a live operator-driven demonstration environment that proves MicroMind can command a PX4-based vehicle stack dynamically during mission execution. This is a two-layer architecture:

| Layer | Name | Purpose | Readiness Dependency |
| --- | --- | --- | --- |
| Layer 1 | Static Mission Report | Post-run HTML/PNG output. Current SB-4 delivery. Provides technical evidence of comparative outcome. | Already delivered (VIZ-01). No further prerequisites. |
| Layer 2 | Long-Range Mission Demonstration | Interactive log-driven mission playback environment over 100–150 km mission. DEM-style terrain underlay. Log-driven event overlays. Operator can pause, scrub, inspect. Designed for TASL/DISC leadership engagement. | Requires EC-01 through EC-11 (§17) before scope entry. |
| Layer 3 | Tactical Live Demonstration | Real-time Gazebo-class SITL/HITL environment. Operator injects commands during flight. Both vehicles move simultaneously in real time. Live status panels for all subsystem states. | Long-term roadmap. Requires stable HIL integration as prerequisite. |

The operational difference that the demonstration must communicate visibly:

- When GNSS failed — the exact moment of denial, visible as a transition event, not a post-hoc annotation

- When Vehicle A began to drift — cross-track separation growing over time, not a chart label

- Why Vehicle B maintained the mission — TRN/VIO correction markers, corridor adherence visible

- When retasking occurred — visible route change on Vehicle B, Vehicle A unable to respond

- How MicroMind changed the outcome — decisive separation at the terminal phase

## 9.2 VIZ-01 — Static Mission Report (SB-4, Delivered)

| Req ID | VIZ-01 |
| --- | --- |
| Title | Static BCMP-2 Comparative Mission Report |
| Priority | High — SB-4 Complete |
| Description | The Mission Demonstration Tool shall produce a self-contained HTML mission report containing: (1) business verdict block as first visible content, (2) Vehicle A vs Vehicle B drift comparison chart, (3) C-2 gate summary table, (4) causal chain narrative distinguishing Vehicle A failure from Vehicle B success, (5) fault injection event log. The report shall be air-gap safe (no CDN dependencies). All charts shall be base64 embedded. |
| Inputs | BCMP-2 run output JSON: {disturbance_schedule, vehicle_a states, vehicle_b states, comparison block} |
| Outputs | self_contained_html_file (< 5 MB), json_report_file |
| Success Threshold | HTML generated within 30 s. Business verdict block is first non-header element. All charts base64 embedded. Renders offline. File size < 5 MB. |
| Failure Threshold | HTML not generated within 60 s. Business block missing. Any external resource reference. |
| Recovery Behaviour | Chart generation failure: CONT_DEG — replace chart with plain text data table, log VIZ_CHART_FAILED: {chart_id, error}. HTML write failure: RETRY (3×), then log VIZ_HTML_WRITE_FAILED. |
| Logging (mandatory) | VIZ_REPORT_GENERATED: {generation_latency_ms, file_size_bytes, chart_count, html_path}. |

## 9.3 VIZ-02 — Long-Range Mission Demonstration (Post-Prerequisite)

SCOPE GATE: VIZ-02 shall not be implemented until all SB-5 entry criteria EC-01 through EC-11 (§17) are confirmed PASS. This requirement defines the target capability. Implementation timing is governed entirely by §17.

| Req ID | VIZ-02 |
| --- | --- |
| Title | Long-Range Mission Demonstration — Log-Driven Interactive Playback |
| Priority | High — Post-Prerequisite (§17 entry criteria must be met first) |
| Description | The Long-Range Mission Demonstration shall be an interactive log-driven playback environment for 100–150 km comparative missions. It shall use a DEM-style terrain underlay appropriate to the mission corridor (Srinagar Valley-inspired profile: plains, ridge crossings, mountain ingress, industrial terminal area). Both Vehicle A and Vehicle B shall move simultaneously, driven by logged simulation data. All event overlays shall be driven exclusively from the logged run data (BCMP-2 KPI JSON disturbance_schedule and fault_events[]). No annotation shall be hardcoded. The operator shall be able to pause, resume, scrub the timeline, change playback speed, toggle overlays, and inspect event logs. Given the same KPI JSON and render parameters, the tool shall produce byte-identical output, enabling re-rendering without rerunning simulation. |
| Mission Clock Rule | sim_timestamp_ms is the single authoritative clock, owned by bcmp2_runner.py. All per-state records (Vehicle A at 40 Hz, Vehicle B at 10 Hz) carry this timestamp. Vehicle B position is linearly interpolated between 10 Hz samples at a display cadence of 100 ms for animation purposes only — interpolated values are never logged as mission data. Both tracks shall align on phase boundaries: a phase boundary event in Vehicle B at a given mission_km forces a corresponding record in Vehicle A at the same mission_km, regardless of its 40 Hz schedule. |
| Event Overlay Rule | All overlay markers shall be derived exclusively from logged run data: GNSS denial marker = first gnss_available=False entry in the replay record; TRN correction markers = each trn_correction_applied=True entry; EW replan marker = each replan_event=True entry; retask marker = each retask_event=True entry; SHM marker = SHM_ACTIVATED events from mission log. No event position shall deviate from the logged event by more than one animation frame (100 ms). |
| Sync Rule | The mission timeline bar is the master cursor. All panels (route map, drift chart, event log sidebar) render the state at the cursor position. Panel update shall complete within 50 ms of cursor position change. Pause/resume: cursor freezes, all panels hold that state, no panel advances independently. |
| Reproducibility | Given identical KPI JSON and render parameters: byte-identical output. No randomness, no wall-clock timestamps, no environment-dependent rendering. |
| Required Visual Elements | Moving vehicle markers (Vehicle A and Vehicle B simultaneously), planned route centreline, corridor boundary (±500 m), GNSS denial event marker (explicit visual transition, not just a dot), TRN correction markers at each Vehicle B correction point, EW threat overlay, EW replan event (visible route change), DEM terrain underlay, event timeline bar (bottom of display), phase label (P1–P5), SHM markers, retask markers. |
| Terrain Underlay | DEM-style terrain appropriate to mission corridor. Srinagar Valley-inspired profile preferred for TASL-facing demonstrations. 12 contour levels, alpha ≤ 0.20. Best-effort: failure shall never block playback. All terrain calls wrapped in try/except. |
| Export Formats | MP4: ≥ 720p, ≥ 10 FPS, fixed-speed playback (pause frames excluded), generated from replay record not from screen capture, one frame per 100 ms of mission time at 10× speed. GIF: fallback if ffmpeg unavailable. PNG frame: divergence highlight (maximum Vehicle A/B separation). |
| Demo Modes | Executive: 2–3 min, starts at P2 GNSS denial (P1 skipped), key moments labelled, auto-play. Engineering: full timeline with pause/resume/scrub, all overlays, event log sidebar. Side-by-side: Vehicle A left, Vehicle B right, time-locked cursor shared between both windows. |
| Gazebo / HIL Compatibility | The interface shall not preclude future Gazebo-linked live telemetry overlay. No architectural decision in VIZ-02 shall prevent Layer 3 (Tactical Live Demonstration) being added without rework of VIZ-02's data pipeline. |
| Logging (mandatory) | VIZ_DEMO_RENDERED: {format, duration_s, file_size_mb, render_latency_ms, seed, json_hash}. VIZ_MP4_FAILED. VIZ_GIF_FALLBACK. |

## 9.4 VIZ-03 — Tactical Live Demonstration (Long-Term Roadmap)

ROADMAP ITEM: The Tactical Live Demonstration is part of the intended MicroMind product narrative and long-term roadmap. It is not scoped for SB-5. It requires stable HIL integration as a prerequisite. It is defined here to ensure that VIZ-02 architectural decisions do not foreclose this capability.

| Req ID | VIZ-03 |
| --- | --- |
| Title | Tactical Live Demonstration — Real-Time Gazebo-Class Environment |
| Priority | Roadmap — Not SB-5 scope |
| Description | The Tactical Live Demonstration shall provide a real-time operator-driven environment using a Gazebo-class simulation (SITL or HITL). Both Vehicle A (standard PX4 behaviour) and Vehicle B (PX4 + MicroMind autonomy layer) shall move simultaneously in real time. The operator shall be able to inject commands during flight to demonstrate dynamic mission authority. |
| Intended Demonstration Scope | GNSS denial during motion — visible divergence after denial. Vehicle B maintains route continuity. Vehicle A drifts, fails corridor, or requires manual intervention. Dynamic retask during flight — updated corridor, target, or exclusion zone. Vehicle B responds live and visibly. Live status panels: navigation source, BIM Trust State, GNSS state, VIO/TRN mode, EW state, PX4 mode, Mission Manager FSM state, SHM state. |
| Operator Command Injection | Target coordinate change, No-Fly Zone insertion, EW Exclusion Zone insertion, route corridor width adjustment, mission timing constraints. |
| Prerequisite | Stable SITL on micromind-node01 with Gazebo rendering operational (OI-20 resolved). Both Vehicle A and Vehicle B autonomy stacks running simultaneously in Gazebo SITL. VIZ-02 (Layer 2) fully implemented. run_demo.sh verified (OI-30). HIL on Jetson Orin NX is a parallel workstream and is not a gate for VIZ-03. See AD-22 (06 April 2026) in MICROMIND_DECISIONS.md. |
| Note | A long-range DEM-underlay comparative mission environment using Srinagar Valley-inspired terrain remains part of the long-term roadmap for TASL and DISC-facing demonstration. This is distinct from the C-3 synthetic DEM used in BCMP-2 and does not require modification of existing BCMP-2 infrastructure. |

## 9.5 Business Communication Objective (Normative)

NORMATIVE: The Mission Demonstration Tool serves a business purpose distinct from technical validation. The primary audience is TASL leadership, DISC evaluators, OEM reviewers, and investors. The demonstration must make the operational difference between Vehicle A and Vehicle B visually obvious within 30–60 seconds without verbal explanation. The viewer must be able to identify: when GNSS failed, when Vehicle A began to drift, why Vehicle B maintained the mission, when retasking occurred, and how MicroMind changed the outcome. This objective governs all VIZ requirements and cannot be satisfied by a static HTML report or a pre-rendered video. The demonstration architecture is built on a stable, proven, repeatable autonomy stack — not the other way around.

# 10. Interface Definitions

## 10.1 Unified State Vector (USV)

| Field | Type | Units | Range | Rate Hz | Mandatory | Source | Consumer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| timestamp_ns | int64 | ns | 0–2^63 | 200 | Yes | Navigation Manager | All |
| north_m | float64 | m | ±1e6 | 200 | Yes | Navigation Manager (ESKF) | Mission Manager, PX4 Bridge |
| east_m | float64 | m | ±1e6 | 200 | Yes | Navigation Manager (ESKF) | Mission Manager, PX4 Bridge |
| alt_m_asl | float64 | m | 0–8000 | 200 | Yes | Navigation Manager (ESKF) | Mission Manager |
| vn_ms | float64 | m/s | ±200 | 200 | Yes | Navigation Manager (ESKF) | PX4 Bridge |
| ve_ms | float64 | m/s | ±200 | 200 | Yes | Navigation Manager (ESKF) | PX4 Bridge |
| vd_ms | float64 | m/s | ±50 | 200 | Yes | Navigation Manager (ESKF) | PX4 Bridge |
| quat_wxyz | float64[4] | — | unit norm | 200 | Yes | Navigation Manager (ESKF) | Mission Manager |
| active_nav_mode | str | — | GNSS/TRN_PRIMARY/VIO_PRIMARY/INS_ONLY | 10 | Yes | Navigation Manager | Mission Manager, Demo Tool |
| nav_confidence | float32 | — | 0.0–1.0 | 10 | Yes | Navigation Manager (BIM) | Mission Manager |
| cross_track_error_m | float32 | m | 0–5000 | 10 | Yes | Navigation Manager | Mission Manager, Demo Tool |
| bim_trust_score | float32 | — | 0.0–1.0 | 10 | Yes | Navigation Manager (BIM) | Mission Manager, Demo Tool |
| bim_trust_state | str | — | GREEN/AMBER/RED | 10 | Yes | Navigation Manager (BIM) | Mission Manager |
| mission_km | float32 | km | 0–200 | 10 | Yes | Mission Manager | All |
| mission_phase | int8 | — | 0–4 | 10 | Yes | Mission Manager | All |
| fsm_state | str | — | See §10.4 | 10 | Yes | Mission Manager | All |

## 10.2 Mission Envelope Schema

| Field | Type | Units | Mandatory | Failure if Missing |
| --- | --- | --- | --- | --- |
| envelope_id | str (UUID) | — | Yes | Reject envelope, log ENVELOPE_INVALID |
| signature_hash | str (SHA-256) | — | Yes | Reject envelope immediately |
| target_class | str[] | — | Yes | Reject envelope |
| geographic_boundary | float64[N][2] | lat/lon deg | Yes | Reject envelope |
| time_window_start_utc | int64 | Unix ms | Yes | Reject envelope |
| time_window_end_utc | int64 | Unix ms | Yes | Reject envelope |
| max_retask_count | int8 | — | Yes (default 0) | Reject envelope |
| l10s_civilian_threshold | float32 | 0.0–1.0 | Yes | Use default 0.70 |
| abort_on_envelope_violation | bool | — | Yes | Default True |
| operator_id | str | — | Yes | Log warning, continue |
| avp_profile_id | str | — | Yes | Reject envelope — AVP profile required for test and operational compliance |
| max_cruise_speed_kmh | float32 | km/h | Yes | Use AVP profile default; log warning |
| max_top_speed_kmh | float32 | km/h | Yes | Use AVP profile default; log warning |
| alt_floor_m_agl | float32 | m AGL | Yes | Use AVP profile default; log warning |
| alt_ceiling_m_agl | float32 | m AGL | Yes | Use AVP profile default; log warning |
| gnss_denied_segment_km | float32 | km | Yes | Use mission default; log warning |

Valid values for avp_profile_id: "AVP-02", "AVP-03", "AVP-04". "AVP-01" is not a valid value in Phase 1 — any envelope specifying AVP-01 shall be rejected with ENVELOPE_INVALID and logged.

## 10.3 BIM Trust State Model

| State | bim_trust_score | Enter Condition | Exit Condition | Navigation Effect | Recovery Class |
| --- | --- | --- | --- | --- | --- |
| GREEN | 0.70–1.00 | score > 0.70 for ≥ 5 consecutive 100ms ticks | score drops below 0.70 | GNSS at full fusion weight | CONT_NOM |
| AMBER | 0.10–0.69 | score ≤ 0.70 | score > 0.70 × 5 ticks or score < 0.10 | GNSS at 0.5× fusion weight | CONT_DEG |
| RED | 0.00–0.09 | score < 0.10 | 10 consecutive clean readings score > 0.70 | GNSS gated out of ESKF entirely | RETRY_ALT → TRN_PRIMARY |

## 10.4 Mission Phase State Machine

| State | Code | Entry Condition | Exit Condition | Retask Permitted | SHM Permitted |
| --- | --- | --- | --- | --- | --- |
| INIT | 0 | System start | GNSS fix acquired + ESKF converged | No | No |
| PRE_GNSS_DENIAL | 1 | INIT complete, GNSS GREEN | Mission km ≥ GNSS_DENIAL_KM or GNSS_LOST | Yes | Yes |
| GNSS_DENIED | 2 | GNSS_LOST event or km ≥ threshold | EW_TERMINAL_ZONE_ENTERED or mission abort | Yes (conditions per PLN-02) | Yes |
| TERMINAL_APPROACH | 3 | EW terminal zone entered, DMRL initialised | DMRL lock + L10s PROCEED, or abort | No | Yes |
| COMMITTED | 4 | L10s PROCEED + approach_authorised=True | Mission complete or ABORT | No | No |
| SHM | 5 | Any SHM trigger (MM-03) | Trigger cleared + operator clearance | Yes (if trigger=ROUTE_DEADEND) | N/A |
| ABORT | 6 | ABORT_MISS or unrecoverable terminal failure | Terminal state | No | No |

## 10.5 Recovery Mode State Machine

| Recovery Class | Trigger Condition | Duration / Retries | Escalation | Exit Condition |
| --- | --- | --- | --- | --- |
| CONT_NOM | No fault | Indefinite | None | Fault occurrence |
| CONT_DEG | Minor fault | Indefinite until clears | RETRY if threshold exceeded | Fault clears or escalation |
| RETRY | Operation failure | N retries at interval (per req) | Next recovery class | Success within budget |
| RETRY_ALT | Primary source failed | N retries of backup | CONT_DEG or SHM | Backup provides valid data |
| HOLD_PATH | Route uncertain / INS_ONLY retask rejected | 10 s max | SHM after 10 s | Threat resolved or operator retask |
| REPLAN | Route invalidated | 1 attempt per level (0→3) | ROUTE_DEADEND → SHM | Valid route found |
| SHM | Critical fault or route deadend | Until cleared. ESCALATE_OP at 120 s. ABORT_MISS at 240 s | ESCALATE_OP | Trigger cleared + clearance |
| ABORT_TERM | Terminal fault only | Immediate | SHM if re-acquire fails | DMRL re-acquire or SHM |
| ABORT_MISS | Unrecoverable fault | Immediate, terminal | None | Mission ends |
| ESCALATE_OP | SHM active > 120 s | 120 s await retask | ABORT_MISS | Retask received |
| PROC_RESTART | Watchdog trip (RESTARTABLE processes only) | Restart ≤ 2 s + restore ≤ 10 s | ABORT_MISS after 30 s | Process online + nav restored |
| CHECKPOINT_RESTORE | Process restart or PX4 reboot | Load + restore ≤ 10 s | PROC_RESTART if corrupt | Nav running from checkpoint |

## 10.6 PX4 Interface Contract

| Contract Item | Specification | Failure Behaviour |
| --- | --- | --- |
| Setpoint message | SET_POSITION_TARGET_LOCAL_NED, coordinate_frame=MAV_FRAME_LOCAL_NED | OFFBOARD Loss if not received within 500 ms |
| Setpoint rate | ≥ 20 Hz continuous | < 10 Hz for 200 ms: log SETPOINT_RATE_LOW; < 10 Hz for 500 ms: OFFBOARD Loss recovery |
| Stale setpoint | stale_setpoint_age_ms ≤ 200 ms. Stale setpoints discarded on link recovery — fresh setpoint issued from current nav state | > 400 ms: PX4 may exit OFFBOARD |
| Heartbeat | HEARTBEAT expected at 1 Hz from PX4 | Gap > 2 s: STALE; > 10 s: LOST → SHM |
| OFFBOARD re-entry | SET_MODE MAVLink command | 3 retries at 2 s intervals; then ABORT_MISS |
| Waypoint queue | MAVLink MISSION_ITEM_INT, queue ≤ 100 waypoints. Old waypoints remain active until new_envelope_valid=True and route_validated=True. | Overflow: log WAYPOINT_QUEUE_FULL, truncate to 50 |
| Max waypoint queue | 100 waypoints | Excess waypoints logged and discarded |
| PX4 reboot detection | HEARTBEAT sequence reset OR SYS_STATUS reset | Detected ≤ 3 s: initiate PX4-04 recovery sequence |
| Post-reboot state | PX4 may return in HOLD. Mission Manager evaluation gate (D8a) required before autonomous resume. | D8a: if pending_operator_clearance_required=True: stay SHM. If False and no abort/SHM in checkpoint: autonomous resume. |
| stale_setpoints_discarded_on_recovery | Always True — stale setpoints never replayed on link recovery | N/A |

## 10.7–10.10 EW Heatmap, Route Planner, DMRL, Fault Injection Schemas

Schemas §10.7, §10.8, §10.9, and §10.10 are unchanged from v1.1. Refer to §10.7 (EW Heatmap), §10.8 (Route Planner I/O), §10.9 (DMRL Output), §10.10 (Fault Injection) in the SRS-MicroMind-v1.1 baseline for the full field definitions. These schemas require no modification in v1.2.

10.7 EW Heatmap Schema

| Field | Type | Units | Range | Rate Hz | Mandatory | Source | Consumer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| grid_north_origin_m | float64 | m | ±1e6 | On update | Yes | EW Manager | Route Planner |
| grid_east_origin_m | float64 | m | ±1e6 | On update | Yes | EW Manager | Route Planner |
| resolution_m | float32 | m | 50–500 | On update | Yes | EW Manager | Route Planner |
| grid_rows | int32 | — | 1–500 | On update | Yes | EW Manager | Route Planner |
| grid_cols | int32 | — | 1–500 | On update | Yes | EW Manager | Route Planner |
| cost_matrix | float32[rows][cols] | 0.0–1.0 | 0.0–1.0 | 2 | Yes | EW Manager | Route Planner, Demo Tool |
| map_stale_flag | bool | — | — | 2 | Yes | EW Manager | Route Planner |
| map_age_ms | int64 | ms | 0–∞ | 2 | Yes | EW Manager | Route Planner |
| last_update_ms | int64 | ms | — | 2 | Yes | EW Manager | Demo Tool |
| threat_list[].id | str | — | — | 2 | Yes | EW Manager | Demo Tool |
| threat_list[].north_m | float64 | m | ±1e6 | 2 | Yes | EW Manager | Route Planner, Demo Tool |
| threat_list[].east_m | float64 | m | ±1e6 | 2 | Yes | EW Manager | Route Planner, Demo Tool |
| threat_list[].radius_m | float32 | m | 100–5000 | 2 | Yes | EW Manager | Route Planner, Demo Tool |
| threat_list[].intensity | float32 | 0.0–1.0 | 0.0–1.0 | 2 | Yes | EW Manager | Route Planner |
| threat_list[].type | str | — | JAMMER/SPOOFER/UNKNOWN | 2 | Yes | EW Manager | Demo Tool |

10.8 Route Planner I/O Schema

| Field | Direction | Type | Units | Mandatory | Failure if Missing |
| --- | --- | --- | --- | --- | --- |
| start_north_m | Input | float64 | m | Yes | Plan rejected |
| start_east_m | Input | float64 | m | Yes | Plan rejected |
| destination_north_m | Input | float64 | m | Yes | Plan rejected |
| destination_east_m | Input | float64 | m | Yes | Plan rejected |
| ew_cost_map | Input | EWHeatmap | — | Yes | Use empty map, log PLAN_NO_EW_MAP |
| no_fly_zones[] | Input | Polygon[]{north_m,east_m}[] | m | No (default empty) | No NFZ avoidance |
| terrain_exclusion_zones[] | Input | Polygon[] | m | No (default empty) | No terrain exclusion |
| constraint_relaxation_level | Input | int8 (0–3) | — | No (default 0) | Use 0 |
| waypoint_list | Output | WP[]{north_m,east_m,alt_m} | m | Yes | ROUTE_DEADEND if empty |
| route_length_km | Output | float32 | km | Yes | — |
| route_ew_cost | Output | float32 | 0.0–1.0 | Yes | — |
| plan_latency_ms | Output | int32 | ms | Yes | — |
| route_validated | Output | bool | — | Yes | — |
| constraint_level_used | Output | int8 (0–3) | — | Yes | — |

## 10.9 DMRL Output Schema

| Field | Type | Units | Range | Rate Hz | Mandatory | Failure if Missing |
| --- | --- | --- | --- | --- | --- | --- |
| lock_confidence | float32 | — | 0.0–1.0 | 25 | Yes | ABORT_TERM |
| locked_target_north_m | float64 | m | ±1e6 | 25 | Yes (if lock_confidence > 0) | — |
| locked_target_east_m | float64 | m | ±1e6 | 25 | Yes (if lock_confidence > 0) | — |
| dwell_frame_count | int32 | frames | 0–1000 | 25 | Yes | ABORT_TERM if ≥ 5 not reached |
| approach_authorised | bool | — | — | 25 | Yes | ABORT_TERM |
| aimpoint_correction_deg | float32 | ° | ±15 | 25 | Yes | ABORT_TERM if > ±15 |
| decoy_detected | bool | — | — | 25 | Yes | — |
| decoy_confidence | float32 | — | 0.0–1.0 | 25 | Yes (if decoy_detected) | — |
| abort_reason | str | — | — | 25 | No (on abort only) | — |
| reacquisition_active | bool | — | — | 25 | Yes | — |

## 10.10 Fault Injection Schema

| Field | Type | Range | Mandatory | Description |
| --- | --- | --- | --- | --- |
| fault_id | str | FI-01..FI-13 | Yes | Fault identifier (fault_manager.py constants) |
| fault_type | str | GNSS_LOSS/VIO_LOSS/RADALT_LOSS/EO_FREEZE/IMU_JITTER/TERRAIN_CONF_DROP/... | Yes | Fault class enum |
| severity | float32 | 0.0–1.0 | Yes | Fault severity multiplier |
| duration_s | float32 | 0.0–∞ | Yes | Duration (0 = indefinite until cleared) |
| auto_expire | bool | — | Yes | True: clears after duration_s |
| source_module | str | — | Yes | Module injecting the fault |
| active | bool | — | Yes (read-only) | Current active state |
| injection_timestamp_ms | int64 | — | Yes (set on inject) | Wall time of injection |
| expiry_timestamp_ms | int64 | — | Yes (if auto_expire) | injection_timestamp_ms + duration_s×1000 |

## 10.11 Demo Tool Telemetry Schema (per-state replay record)

MISSION CLOCK RULE (V-01): sim_timestamp_ms is the single authoritative clock, owned by bcmp2_runner.py. All per-state records carry this timestamp. Vehicle B is logged at 10 Hz; for display purposes only, Vehicle B position is linearly interpolated between 10 Hz samples at a fixed cadence of 100 ms. Interpolated values are never logged as mission data. Both tracks align on phase boundaries: a phase boundary in Vehicle B at mission_km=X forces a corresponding Vehicle A record at the same mission_km=X regardless of its 40 Hz schedule.

| Field | Type | Rate Hz | Source | Notes |
| --- | --- | --- | --- | --- |
| sim_timestamp_ms | int64 | 40 | bcmp2_runner.py (authoritative clock owner) | Simulation time, not wall time. Shared by both tracks. |
| mission_km | float32 | 40 | bcmp2_runner.py | Along-track distance. Phase boundaries force record in both tracks at same mission_km. |
| true_north_m | float64 | 40 | Scenario | Ground truth |
| true_east_m | float64 | 40 | Scenario | Ground truth |
| va_north_m | float64 | 40 | baseline_nav_sim | Vehicle A estimated position |
| va_east_m | float64 | 40 | baseline_nav_sim | Vehicle A estimated position |
| va_cross_track_m | float32 | 40 | baseline_nav_sim | Vehicle A lateral error |
| gnss_available | bool | 40 | DisturbanceSchedule | From C-4 schedule. Used as authoritative source for GNSS denial event overlay position. |
| mission_phase | int8 | 40 | DisturbanceSchedule | Phase 0–4 |
| vb_north_m | float64 | 10 | BCMP-1 Runner | Vehicle B position. Interpolated at 100 ms cadence for display only. |
| vb_east_m | float64 | 10 | BCMP-1 Runner | Vehicle B position. Interpolated at 100 ms cadence for display only. |
| vb_nav_mode | str | 10 | BCMP-1 Runner | Active nav mode |
| vb_cross_track_m | float32 | 10 | BCMP-1 Runner | Vehicle B lateral error |
| trn_correction_applied | bool | 2 | BCMP-1 Runner | Source of TRN correction marker overlay. Must not be hardcoded. |
| ew_threats[] | EWThreat[] | events | EW Manager | Source of EW threat overlay. Must not be hardcoded. |
| fault_events[] | FaultEvent[] | events | Fault Manager | Fault injection events |
| replan_event | bool | events | Route Planner | Source of EW replan overlay marker. Must not be hardcoded. |
| retask_event | bool | events | Mission Manager | Source of retask overlay marker. Must not be hardcoded. |
| shm_event | bool | events | Mission Manager | Source of SHM overlay marker. Must not be hardcoded. |

## 10.12 Retask Command Schema

| Field | Type | Units | Mandatory | Failure if Missing or Invalid |
| --- | --- | --- | --- | --- |
| retask_id | str (UUID) | — | Yes | Reject immediately |
| signature_hash | str (SHA-256) | — | Yes | Reject: RETASK_REJECTED_INVALID_ENVELOPE |
| new_target_north_m | float64 | m | Yes | Reject: missing target |
| new_target_east_m | float64 | m | Yes | Reject: missing target |
| new_mission_envelope | MissionEnvelope | — | Yes | Reject: no envelope |
| new_no_fly_zones[] | Polygon[] | m | No | No NFZ update |
| new_ew_exclusion_zones[] | EWExclusionZone[] | m | No | No EW zone update |
| permitted_phases[] | int8[] | — | No (default [1,2]) | Use default CRUISE+GNSS_DENIED |
| retask_timestamp_utc | int64 | Unix ms | Yes | Reject: stale if > 60 s old |

## 10.13 No-Fly Zone and EW Exclusion Zone Schemas

| Schema | Field | Type | Units | Mandatory | Description |
| --- | --- | --- | --- | --- | --- |
| NoFlyZone | zone_id | str | — | Yes | Unique identifier |
| NoFlyZone | boundary | float64[N][2] | north_m, east_m | Yes | Polygon vertices in local NED |
| NoFlyZone | alt_min_m | float32 | m ASL | No | Minimum altitude (0 if unbounded) |
| NoFlyZone | alt_max_m | float32 | m ASL | No | Maximum altitude (∞ if unbounded) |
| NoFlyZone | active | bool | — | Yes | False = soft exclusion (logged, not enforced) |
| EWExclusionZone | zone_id | str | — | Yes | Unique identifier |
| EWExclusionZone | centre_north_m | float64 | m | Yes | Threat centre |
| EWExclusionZone | centre_east_m | float64 | m | Yes | Threat centre |
| EWExclusionZone | radius_m | float32 | m | Yes | Exclusion radius |
| EWExclusionZone | threat_type | str | — | Yes | JAMMER/SPOOFER/UNKNOWN |
| EWExclusionZone | buffer_m | float32 | m | No (default 500) | Clearance added to radius |

## 10.14 Process Watchdog Interface

| Field | Direction | Type | Rate / Trigger | Description |
| --- | --- | --- | --- | --- |
| process_id | Input | str | On register | Module name |
| heartbeat_interval_ms | Input | int32 | On register | Expected heartbeat period |
| process_heartbeat | Input | bool (True=alive) | Per interval | Watchdog reset signal from monitored process |
| missed_heartbeat_count | Output (internal) | int32 | Per interval | Incremented on miss, reset on receipt |
| failure_threshold | Input | int8 | On register | Consecutive misses before failure (default 3) |
| restartability_class | Input | str | On register | RESTARTABLE_NO_SHM / RESTARTABLE_WITH_SHM / NOT_RESTARTABLE |
| process_failed | Output | event | On threshold | Triggers PROC_RESTART or ABORT_MISS based on restartability_class |
| restart_command | Output | shell command | On failure (RESTARTABLE only) | Command to restart process |
| watchdog_log_event | Output | event | On fail/restart | PROCESS_FAILURE or PROCESS_RESTARTED or ESKF_CORE_FAILURE |

## 10.15 Checkpoint / Restore Interface

| Field | Type | Units | Mandatory | Description |
| --- | --- | --- | --- | --- |
| checkpoint_id | str (UUID) | — | Yes | Unique per checkpoint |
| timestamp_ms | int64 | ms | Yes | Wall time of save |
| mission_km | float32 | km | Yes | Along-track at save |
| mission_phase | int8 | — | Yes | Phase at save |
| fsm_state | str | — | Yes | Mission Manager FSM state |
| nav_north_m | float64 | m | Yes | ESKF position at save |
| nav_east_m | float64 | m | Yes | ESKF position at save |
| nav_alt_m | float64 | m | Yes | ESKF altitude at save |
| active_nav_mode | str | — | Yes | Navigation mode at save |
| bim_trust_state | str | — | Yes | BIM Trust State at save |
| bim_trust_score | float32 | — | Yes | BIM Trust Score at save |
| current_waypoint_index | int32 | — | Yes | PX4 waypoint index at save |
| waypoint_list_snapshot | WP[] | m | Yes | Current route waypoints |
| ew_map_snapshot_path | str | — | Yes (mandatory in v1.2) | Path to serialised EW cost map. Required for correct rollback restore. |
| terrain_corridor_phase | int8 | — | Yes (new in v1.2) | Mission phase of current terrain corridor. Required for retask rollback terrain restore. |
| route_corridor_half_width_m | float32 | m | Yes (new in v1.2) | Current route corridor half-width. Required for INS_ONLY retask constraint check. |
| retask_state | str | — | Yes | NONE/PENDING/ACTIVE — current retask state |
| shm_active | bool | — | Yes (new in v1.2) | True if Safe Hold Mode is active at save time. Restored on CHECKPOINT_RESTORE to re-assert SHM. |
| shm_trigger_reason | str | — | Yes (new in v1.2) | SHM trigger reason at save. 'NONE' if shm_active=False. |
| pending_operator_clearance_required | bool | — | Yes (new in v1.2) | True if operator clearance was required before autonomous resume at save time. Governs post-reboot autonomous resume gate (PX4-04 D8a). |
| mission_abort_flag | bool | — | Yes (new in v1.2) | True if ABORT_MISS was in progress at save time. Triggers ABORT_MISS on restore if True. |
| eta_to_destination_ms | int64 | ms | Yes (new in v1.2) | ETA at save time. Recomputed after restore and checked against envelope time_window_end_utc. |
| mission_start_utc_ms | int64 | Unix ms | Yes (new in v1.2) | Original mission start time. Used for ETA recomputation after restore. |
| integrity_hash | str (SHA-256) | — | Yes | Hash of all fields above for corruption detection |

## 10.16 Logging and Event Priority Schema

| Event Level | Priority | Buffer Policy | Disk Policy | Examples |
| --- | --- | --- | --- | --- |
| CRITICAL | P0 | Always in RAM ring (1000 events). Duplicated to critical_events.log (never rolled). | Never dropped, disk or RAM | SHM_ACTIVATED, SPOOF_DETECTED, DMRL_ABORT, ENVELOPE_VIOLATION, OFFBOARD_LOSS, CORRIDOR_BREACH, ESKF_CORE_FAILURE |
| WARNING | P1 | In RAM ring | Dropped only on disk full after CRITICAL filled | TRN_SUPPRESSED, VIO_DEGRADED, CPU_HIGH, BIM_STALE, EW_MAP_STALE, ROUTE_PLANNER_MEMORY_HIGH |
| INFO | P2 | Circular buffer (10000 events) | Dropped on buffer > 80% | FSM_TRANSITION, CHECKPOINT_SAVED, CHECKPOINT_PURGED, ROUTE_PLANNED, NAV_MODE_TRANSITION, RETASK_COMPLETE |
| DEBUG | P3 | In-memory only (no disk) | Never written to disk | DMRL_UPDATE, VIO frame stats, EKF step |
| TELEMETRY | P2 | Time-series buffer. Rolled every 30 min or 200 MB. | Sample rate reduced on CPU_HIGH | USV at 10 Hz, setpoint_rate_hz, bim_trust_score |
| AVP_FALLBACK_ALTITUDE_REDUCTION | HIGH | Fields: {avp_profile_id, planned_alt_m_agl, actual_alt_m_agl, reason, mission_km} | Trigger: Operational altitude drops below AVP profile floor | AVP_FALLBACK_ALTITUDE_REDUCTION |
| AVP_FALLBACK_SPEED_REDUCTION | HIGH | Fields: {avp_profile_id, planned_speed_kmh, actual_speed_kmh, reason, mission_km} | Trigger: Operational speed drops below AVP profile cruise speed | AVP_FALLBACK_SPEED_REDUCTION |
| OM_SUPPRESSED | HIGH | Fields: {match_confidence, mission_km, consecutive_count, sigma_terrain} | Trigger: Orthophoto match confidence below threshold | OM_SUPPRESSED |
| OM_EXTENDED_GAP | HIGH | Fields: {gap_km, sigma_terrain_min} | Trigger: No successful OM correction for > 10 km over textured terrain | OM_EXTENDED_GAP |
| OM_RESUMED | MEDIUM | Fields: {mission_km, match_confidence, gap_km_since_last} | Trigger: Successful OM correction after extended gap | OM_RESUMED |
| NAV_DEGRADED_INS_ONLY | CRITICAL | Fields: {mission_km, reason, drift_proxy_m_per_km} | Trigger: Navigation demoted to INS-only — no VIO or OM correction available | NAV_DEGRADED_INS_ONLY |

# 11. Unit Test Plan

FORMAT: All unit tests follow the 18-field format. Tests 11.1–11.17 are unchanged from SRS-MicroMind-v1.1. They are reproduced here verbatim for document completeness. No test cases have been added or removed. UT-PLN-02, UT-PLN-03, UT-PX4-05, UT-MM-04, UT-RS-03, UT-RS-02 remain as defined gaps requiring SB-5 implementation — their specifications are unchanged.

REFERENCE: Full test specifications for UT-NAV-01-A through UT-FIM-02 (§11.1–§11.17) are identical to SRS-MicroMind-v1.1 §11.1–§11.17 and are carried forward without modification. The SB-5 entry criteria in §17 govern when new unit tests for UT-MM-04, UT-PX4-05, UT-VIZ-02, and UT-VIZ-03 are to be authored and integrated into the regression suite. All tests from v1.1 §11 are preserved with their exact procedure, pass/fail conditions, retry policies, and post-test states.

## 11.1 UT-NAV-01-A — INS Drift C-2 Envelope Gate

| Test ID | UT-NAV-01-A |
| --- | --- |
| Req ID | NAV-01 |
| Test Type | Unit |
| Objective | Verify Vehicle A cross-track drift falls within Monte Carlo calibrated C-2 envelopes for all 3 canonical seeds at km 60, 100, 120. |
| Entry Criteria | AVP Scope: validates AVP-02/03 corridor geometry and C-2 envelopes only. AVP-04 GNSS-denied segments beyond km 120 are not characterised. baseline_nav_sim.py available. PHASE_ENVELOPES loaded from bcmp2_drift_envelopes.py. STIM300 frozen constants: _ACC_BIAS_RW=9.81e-7, _GYRO_BIAS_RW=4.04e-8. |
| Stimulus | Seeds: [42, 101, 303]. max_km=150. No TRN, VIO, or GNSS corrections after km 30. |
| Procedure | 1. For each seed: generate disturbance_schedule, generate IMU noise (normal bias draw). 2. Run baseline_nav_sim.run() to 150 km. 3. Read drift_at_km60_m, drift_at_km100_m, drift_at_km120_m. 4. Compare to PHASE_ENVELOPES floors and ceilings. 5. Check all state values for NaN and Inf. |
| Expected Behaviour | drift_at_km60_m ∈ [5, 80] m. drift_at_km100_m ∈ [12, 350] m. drift_at_km120_m ∈ [15, 650] m. No NaN or Inf in any state vector. |
| Exit Criteria | All 9 gate checks complete (3 seeds × 3 boundaries) OR any gate fails. |
| Pass Condition | 9/9 C-2 gates PASS. Zero NaN or Inf values. Zero exceptions raised. |
| Fail Condition | Any gate outside envelope. Any NaN or Inf. Any Python exception. |
| Retry Policy | No retry. Fix code root cause first. |
| Recovery Behaviour | On fail: log failing seed, gate km, observed value, expected range. Do not continue to next test step. |
| Required Logs | c2_gates dict for each seed (drift_at_km60_m, _100_m, _120_m, passed). |
| Data Retention | On fail: save full run JSON for all 3 seeds to /tmp/ut_nav_01a_fail_{seed}.json. |
| Repeatability Count | 3 full passes on micromind-node01 with system Python 3.12.3. |
| Post-Test State | No persistent state. All RNG instances destroyed. |

## 11.2 UT-NAV-01-B — 150 km Numerical Stability

| Test ID | UT-NAV-01-B |
| --- | --- |
| Req ID | NAV-01 |
| Test Type | Unit |
| Objective | Verify ESKF-derived cross-track propagation produces no NaN, Inf, or physically unbounded values across 150 km INS-only propagation. |
| Entry Criteria | AVP Scope: validates AVP-02/03 corridor geometry and C-2 envelopes only. AVP-04 GNSS-denied segments beyond km 120 are not characterised. baseline_nav_sim.py available. 5 random seeds (not canonical). |
| Stimulus | Seeds: [7, 19, 55, 89, 200]. max_km=150. Full INS-only after km 30. |
| Procedure | 1. Run baseline_nav_sim for each seed. 2. After each run: scan all VehicleAState fields for NaN and Inf. 3. Check ¦cross_track_m¦ < 5000 at all steps. 4. Check heading_error_rad within ±π. |
| Exit Criteria | All 5 seeds complete or any seed produces NaN/Inf/OOB value. |
| Pass Condition | Zero NaN. Zero Inf. ¦cross_track_m¦ < 5000 m for all steps. ¦heading_error_rad¦ < π. |
| Fail Condition | Any NaN or Inf at any step. ¦cross_track_m¦ > 10000 m. ¦heading_error_rad¦ > 2π. |
| Retry Policy | No retry. Instrument failing step. |
| Required Logs | First failing step index, field name, value if fail occurs. |
| Data Retention | On fail: save full state history for failing seed. |
| Repeatability Count | 1 run per seed. |
| Post-Test State | Clean. |

## 11.3 UT-NAV-02-A — TRN Correction Applied Above Threshold

| Test ID | UT-NAV-02-A |
| --- | --- |
| Req ID | NAV-02 |
| Test Type | Unit |
| Objective | Verify TRN correction applied when NCC ≥ 0.45 and reduces INS position error to < 50 m CEP. |
| Entry Criteria | AVP Scope: REWRITE REQUIRED — current test validates superseded RADALT-NCC mechanism and is not valid evidence for NAV-02 v1.3 (orthophoto matching). See GAP-10. trn_stub.py available. DEM tile loaded with terrain roughness σ = 50 m. Inject known 200 m north error into INS position. |
| Stimulus | Call trn_correction() with ncc_scores = [0.45, 0.60, 0.80, 1.00]. Each call with 200 m north error. |
| Procedure | 1. Inject 200 m north position error into INS estimate. 2. Call trn_correction() with each NCC score. 3. Measure correction_magnitude_m. 4. Verify error post-correction. |
| Exit Criteria | All 4 NCC values processed. |
| Pass Condition | correction_applied=True for all NCC ≥ 0.45. Residual error < 50 m for all. No exception. |
| Fail Condition | correction_applied=False for any NCC ≥ 0.45. Residual error > 100 m for any. |
| Retry Policy | No retry. |
| Required Logs | ncc_score, correction_applied, correction_magnitude_m, residual_error_m for each call. |
| Data Retention | Log all 4 test vectors on failure. |
| Repeatability Count | 5 runs. |
| Post-Test State | TRN stub reset. |

## 11.4 UT-NAV-02-B — TRN Suppression Below Threshold

| Test ID | UT-NAV-02-B |
| --- | --- |
| Req ID | NAV-02 |
| Test Type | Unit |
| Objective | Verify TRN correction suppressed when NCC < 0.45 and consecutive suppression counter increments correctly. |
| Entry Criteria | AVP Scope: REWRITE REQUIRED — current test validates superseded RADALT-NCC mechanism and is not valid evidence for NAV-02 v1.3 (orthophoto matching). See GAP-10. trn_stub.py available. consecutive_suppressed_count = 0. |
| Stimulus | Call trn_correction() with ncc_scores = [0.44, 0.30, 0.10, 0.00]. |
| Procedure | 1. Call trn_correction() 3× with NCC = 0.44 (below threshold). 2. Verify correction_applied=False each time. 3. Verify consecutive_suppressed_count = 3 after 3rd call. 4. Verify nav_source_proxy switches to VIO-primary on count=3. |
| Exit Criteria | 3 suppression calls processed and counter checked. |
| Pass Condition | correction_applied=False for all 3. consecutive_suppressed_count = 3. TRN_SUPPRESSED logged 3×. Mode switch to VIO-primary triggered at count=3. |
| Fail Condition | Any correction applied with NCC < 0.45. Counter not = 3. Mode switch not triggered. |
| Retry Policy | No retry. |
| Required Logs | TRN_SUPPRESSED × 3, consecutive_count at each call, NAV_MODE_TRANSITION to VIO_PRIMARY. |
| Repeatability Count | 5 runs. |
| Post-Test State | TRN stub reset. consecutive_suppressed_count = 0. |

## 11.5 UT-NAV-03 — VIOMode OUTAGE After 3 s Starvation

| Test ID | UT-NAV-03 |
| --- | --- |
| Req ID | NAV-03 |
| Test Type | Unit |
| Objective | Verify VIONavigationMode transitions to OUTAGE after 3 s without accepted updates. |
| Entry Criteria | AVP Scope: validates AVP-02/03 altitude and feature-density conditions. AVP-04 sparse terrain at 500–2,000 m AGL not covered. VIONavigationMode in ACTIVE state. Tick rate: 25 Hz (dt=0.04 s). |
| Stimulus | Call tick() at 25 Hz for 3.1 s with accepted=False, innov_mag=0 on all updates. |
| Procedure | 1. Initialise VIONavigationMode. 2. Confirm initial mode = ACTIVE. 3. For 3.1 s: call process_update(accepted=False, innov_mag=0) at 25 Hz. 4. At t = 2.9 s: confirm mode ≠ OUTAGE yet. 5. At t = 3.0 s (±100 ms): confirm mode = OUTAGE. |
| Exit Criteria | mode = OUTAGE confirmed OR t > 4.0 s (timeout = fail). |
| Pass Condition | mode = VIOMode.OUTAGE at t = 3.0 s ± 100 ms. VIO_OUTAGE_EXTENDED logged. |
| Fail Condition | mode ≠ OUTAGE at t = 3.5 s. Any exception. |
| Retry Policy | 5 retries (stochastic timing). |
| Required Logs | VIO_OUTAGE_EXTENDED: {duration_ms}. vio_mode at each tick (optional debug). |
| Repeatability Count | 10 runs. |
| Post-Test State | VIONavigationMode reset. |

## 11.6 UT-BIM-01 — Spoof Detection Latency ≤ 250 ms

| Test ID | UT-BIM-01 |
| --- | --- |
| Req ID | NAV-05, EW-03 |
| Test Type | Unit |
| Objective | Verify BIM detects GNSS position spoof and transitions to RED within 250 ms. |
| Entry Criteria | BIM initialised. GNSS stream at 10 Hz. bim_trust_state = GREEN. bim_trust_score > 0.90. |
| Stimulus | At t=0: inject GNSS position offset of 500 m north from last valid position. |
| Procedure | 1. Run BIM at 10 Hz for 5 s with nominal GNSS. 2. Record t0. 3. Inject 500 m offset. 4. Poll BIM at 10 Hz. 5. Record t_red = first tick where bim_trust_state = RED. 6. Compute detection_latency_ms = t_red - t0. |
| Exit Criteria | bim_trust_state = RED detected OR t > 1.0 s (timeout = fail). |
| Pass Condition | detection_latency_ms ≤ 250. spoof_detected = True. gnss_gated = True within 50 ms of RED. |
| Fail Condition | detection_latency_ms > 500. bim_trust_state never reaches RED. gnss_gated = False while bim_trust_state = RED. |
| Retry Policy | 10 retries (build statistical latency profile across 10 runs). |
| Required Logs | SPOOF_DETECTED: {detection_latency_ms, score_at_detection}. GNSS_GATED: {latency_from_red_ms}. |
| Data Retention | Retain latency measurements for all 10 runs for statistical analysis. |
| Repeatability Count | 10 runs. |
| Post-Test State | BIM reset to GREEN. |

## 11.7 UT-BIM-02 — BIM Update Rate ≥ 10 Hz

| Test ID | UT-BIM-02 |
| --- | --- |
| Req ID | NAV-05 |
| Test Type | Unit |
| Objective | Verify BIM trust score updates at ≥ 10 Hz with no gaps > 200 ms. |
| Entry Criteria | BIM running. Nominal GNSS stream at 1 Hz. |
| Stimulus | Run BIM for 30 s. Count trust score update events. |
| Procedure | 1. Attach observer to BIM trust score output. 2. Run for 30 s. 3. Count updates. 4. Compute measured_rate = updates / 30. 5. Find largest gap between consecutive updates. |
| Exit Criteria | 30 s elapsed. |
| Pass Condition | measured_rate ≥ 10.0 Hz. max_gap_ms ≤ 200. Zero missing updates. |
| Fail Condition | measured_rate < 5 Hz for any 5 s window. max_gap_ms > 500. |
| Retry Policy | 3 runs. |
| Required Logs | update_rate_hz, max_gap_ms measured over 30 s run. |
| Repeatability Count | 3 runs. |
| Post-Test State | BIM running normally. |

## 11.8 UT-EW-01 — EW Cost Map Update ≤ 500 ms

| Test ID | UT-EW-01 |
| --- | --- |
| Req ID | EW-01 |
| Test Type | Unit |
| Objective | Verify EW cost map updates within 500 ms of new threat observation. |
| Entry Criteria | AVP Scope: validates AVP-02/03 speed and replan timing. AVP-04 replan at 200 km/h may require separate timing validation. EW Manager initialised. Vehicle position = (0, 0). Empty cost map. |
| Stimulus | Inject: {bearing=045°, range=3000 m, rssi=-80 dBm, type=JAMMER}. Record t0 = injection time. |
| Procedure | 1. Record t0. 2. Inject EW observation. 3. Poll cost map at 100 Hz. 4. Record t1 = first tick where map_age_ms < 100 and threat_count > 0. 5. Compute latency = t1 - t0. 6. Verify threat visible at estimated position in cost_matrix. |
| Exit Criteria | map updated (threat_count > 0) OR 1.0 s timeout. |
| Pass Condition | update_latency_ms ≤ 500. threat_count = 1. Threat at bearing 045° ±10°, range 3000 m ±200 m in cost_matrix. |
| Fail Condition | update_latency_ms > 1000. threat_count = 0 after 1 s. Threat position error > 500 m. |
| Retry Policy | 5 retries. |
| Required Logs | update_latency_ms, threat_count, threat_position_error_m. |
| Repeatability Count | 5 runs. |
| Post-Test State | EW Manager reset. Empty cost map. |

## 11.9 UT-EW-02 — Route Replan ≤ 1 s With Threat Clearance

| Test ID | UT-EW-02 |
| --- | --- |
| Req ID | EW-02 |
| Test Type | Unit |
| Objective | Verify EW-triggered replan meets 1 s latency and 500 m clearance. |
| Entry Criteria | AVP Scope: validates AVP-02/03 speed and replan timing. AVP-04 replan at 200 km/h may require separate timing validation. Route planner initialised. Planned route: (0,0) → (10000,0). Threat injected at (5000, 300) radius 200 m. |
| Stimulus | Inject threat into EW cost map at (5000, 300). Route planner notified. |
| Procedure | 1. Record t0. 2. Inject threat. 3. Record t1 when new_route returned. 4. Compute replan_latency_ms. 5. Compute min clearance from threat at all waypoints on new route. |
| Exit Criteria | new_route returned OR 2.0 s timeout. |
| Pass Condition | replan_latency_ms ≤ 1000. threat_clearance_m ≥ 500. route_validated = True. |
| Fail Condition | replan_latency_ms > 2000. threat_clearance_m < 200. route_validated = False. |
| Retry Policy | 5 retries. |
| Required Logs | replan_latency_ms, threat_clearance_m, waypoint_count, constraint_level_used. |
| Repeatability Count | 5 runs. |
| Post-Test State | Route planner reset to pre-threat route. |

## 11.10 UT-PLN-01 — Hybrid A* Plan Within 2 s

| Test ID | UT-PLN-01 |
| --- | --- |
| Req ID | PLN-01 |
| Test Type | Unit |
| Objective | Verify Hybrid A* returns valid route within 2 s nominal for 10 km horizon. |
| Entry Criteria | hybrid_astar.py available. Empty EW cost map. No NFZs. 3 test scenarios with different start/end pairs. |
| Stimulus | Scenario A: (0,0)→(10000,0). Scenario B: (0,0)→(7071,7071). Scenario C: (0,0)→(0,10000). |
| Procedure | 1. Time each plan request. 2. Verify route validity: no terrain collision, route_length ≤ 1.2 × straight-line. 3. Verify plan_latency_ms ≤ 2000. |
| Exit Criteria | All 3 scenarios complete. |
| Pass Condition | plan_latency_ms ≤ 2000 for all. route_validated = True for all. route_length_km ≤ 1.2 × straight_line for all. |
| Fail Condition | Any plan_latency_ms > 5000. Any route_validated = False. Any route > 2.0 × straight_line. |
| Retry Policy | 5 retries per scenario. |
| Required Logs | plan_latency_ms, route_length_km, waypoint_count for each scenario. |
| Repeatability Count | 5 runs per scenario. |
| Post-Test State | Planner reset. |

## 11.11 UT-PLN-02 — Retask Accepted in CRUISE Phase

| Test ID | UT-PLN-02 |
| --- | --- |
| Req ID | PLN-02 |
| Test Type | Unit |
| Objective | Verify valid Retask Command accepted in CRUISE phase and rejected in TERMINAL_APPROACH phase. |
| Entry Criteria | Mission Manager FSM in state CRUISE (phase=1). Valid signed RetaskCommand with new target. |
| Stimulus | Send RetaskCommand (valid signature, new target at (8000, 2000)). Then repeat with FSM in TERMINAL_APPROACH (phase=3). |
| Procedure | 1. Set FSM state = CRUISE. 2. Send valid RetaskCommand. 3. Verify retask_accepted=True and new_route computed. 4. Set FSM state = TERMINAL_APPROACH. 5. Send same RetaskCommand. 6. Verify retask_accepted=False and rejection_reason=RETASK_REJECTED_INVALID_PHASE. |
| Exit Criteria | Both scenarios processed. |
| Pass Condition | CRUISE: retask_accepted=True, new_route valid, retask_latency_ms ≤ 10000. TERMINAL: retask_accepted=False, reason=RETASK_REJECTED_INVALID_PHASE. |
| Fail Condition | CRUISE: retask_accepted=False. TERMINAL: retask_accepted=True (critical failure). Any latency > 30 s. |
| Retry Policy | 3 retries. |
| Required Logs | RETASK_ACCEPTED: {retask_latency_ms}. RETASK_REJECTED: {reason=RETASK_REJECTED_INVALID_PHASE}. |
| Repeatability Count | 3 runs. |
| Post-Test State | FSM reset to CRUISE. Retask state = NONE. |

## 11.12 UT-PLN-03 — Dead-End Recovery with SHM

| Test ID | UT-PLN-03 |
| --- | --- |
| Req ID | PLN-03 |
| Test Type | Unit |
| Objective | Verify ROUTE_DEADEND declared and SHM activated when all 4 constraint levels fail. |
| Entry Criteria | Hybrid A* configured. EW cost map: 90% of planning space at cost=1.0 (impassable). No valid route possible. |
| Stimulus | Request route from (0,0) to (10000,0) with near-complete EW coverage. |
| Procedure | 1. Request route at constraint level 0. Expect failure. 2. Auto-escalate to level 1, 2, 3. Expect failure at all levels. 3. Verify ROUTE_DEADEND declared. 4. Verify SHM activated within 2 s. 5. Verify full constraint trace logged. |
| Exit Criteria | ROUTE_DEADEND declared + SHM activated, OR > 30 s elapsed. |
| Pass Condition | ROUTE_DEADEND declared after level 3 failure. SHM_ACTIVATED within 2 s. ROUTE_DEADEND log contains constraint_trace for all 4 levels. |
| Fail Condition | System attempts flight without valid route. SHM not activated within 5 s. |
| Retry Policy | 3 retries. |
| Required Logs | ROUTE_DEADEND: {constraint_trace[0..3], vehicle_position}. SHM_ACTIVATED: {reason=ROUTE_DEADEND}. |
| Repeatability Count | 3 runs. |
| Post-Test State | SHM active. Route planner reset. |

## 11.13 UT-DMRL-01 — Lock Confidence After 5 Dwell Frames

| Test ID | UT-DMRL-01 |
| --- | --- |
| Req ID | TERM-01 |
| Test Type | Unit |
| Objective | Verify DMRL approach_authorised=True only after ≥ 5 consecutive frames with lock_confidence ≥ 0.85. |
| Entry Criteria | dmrl_stub.py available. Clean synthetic thermal target frames. No decoy. |
| Stimulus | Feed 10 clean target frames at 25 FPS (lock_confidence ≥ 0.85 each). |
| Procedure | 1. Feed frames 1–4. Verify approach_authorised=False after each. 2. Feed frame 5. Verify approach_authorised=True. 3. Feed frames 6–10. Verify approach_authorised remains True. 4. Verify aimpoint_correction_deg ≤ ±15 at all frames. |
| Exit Criteria | 10 frames processed. |
| Pass Condition | approach_authorised=False after frames 1–4. approach_authorised=True from frame 5 onward. lock_confidence ≥ 0.85 for all 10 frames. aimpoint_correction_deg ≤ ±15 for all. |
| Fail Condition | approach_authorised=True before 5 dwell frames. lock_confidence < 0.85 in clean scenario. |
| Retry Policy | 10 retries. |
| Required Logs | DMRL_LOCK_ACQUIRED: {confidence, dwell_count=5, mission_km}. lock_confidence at each frame. |
| Repeatability Count | 10 runs. |
| Post-Test State | DMRL reset. |

## 11.14 UT-DMRL-02 — Decoy Rejection at Frame 3

| Test ID | UT-DMRL-02 |
| --- | --- |
| Req ID | TERM-02 |
| Test Type | Unit |
| Objective | Verify decoy rejected (approach_authorised=False) at frame 3 when decoy_confidence ≥ 0.80 × 3 consecutive frames. |
| Entry Criteria | dmrl_stub.py available. Primary target at bearing 000°. Decoy at bearing 010°. |
| Stimulus | Feed 4 frames: decoy_confidence = [0.75, 0.82, 0.85, 0.88]. primary_confidence = 0.90 throughout. |
| Procedure | 1. Feed frame 1 (decoy 0.75). Verify decoy_detected=False. 2. Feed frame 2 (decoy 0.82). Verify decoy_detected=True but rejection not yet triggered. 3. Feed frame 3 (decoy 0.85). Verify rejection triggered (approach_authorised=False, abort_reason=DECOY_REJECTED). 4. Verify primary_lock_maintained=True throughout. |
| Exit Criteria | 4 frames processed. |
| Pass Condition | Frame 3: decoy_detected=True, approach_authorised=False, abort_reason=DECOY_REJECTED, primary_lock_maintained=True. Frame 1: no rejection. |
| Fail Condition | Frame < 3: abort triggered. Frame ≥ 3: no abort triggered. primary_lock_maintained=False at any point. |
| Retry Policy | 10 retries. |
| Required Logs | DECOY_DETECTED: {decoy_confidence, frame=2}. DECOY_REJECTED: {frame=3, primary_lock_maintained=True}. |
| Repeatability Count | 10 runs. |
| Post-Test State | DMRL reset. |

## 11.15 UT-PX4-01 — Setpoint Rate ≥ 20 Hz

| Test ID | UT-PX4-01 |
| --- | --- |
| Req ID | PX4-01 |
| Test Type | Unit |
| Objective | Verify PX4 Bridge publishes setpoints at ≥ 20 Hz with no gap > 200 ms over 60 s. |
| Entry Criteria | PX4 SITL running in OFFBOARD mode. PX4 Bridge connected. Dummy setpoint command loop active. |
| Stimulus | Continuous dummy setpoint (position hold) for 60 s. |
| Procedure | 1. Count SET_POSITION_TARGET_LOCAL_NED messages over 60 s. 2. Measure inter-message intervals. 3. Find max gap. 4. Compute mean rate. |
| Exit Criteria | 60 s elapsed. |
| Pass Condition | Total messages ≥ 1200 (≥ 20 Hz mean). max_gap_ms ≤ 200. OFFBOARD mode active throughout. offboard_continuity = 100% (no exits). |
| Fail Condition | Total messages < 600. max_gap_ms > 500. Any OFFBOARD Loss. |
| Retry Policy | 3 retries. |
| Required Logs | setpoint_count, mean_rate_hz, max_gap_ms, offboard_continuity_percent. |
| Repeatability Count | 3 runs. |
| Post-Test State | PX4 in OFFBOARD mode. |

## 11.16 UT-FIM-01 — FI-01 GNSS Loss Proxy Intercept

| Test ID | UT-FIM-01 |
| --- | --- |
| Req ID | NAV-05, EW-03 |
| Test Type | Unit |
| Objective | Verify sensor_fault_proxy intercepts GNSS correctly on FI-01 and BIM transitions toward RED. |
| Entry Criteria | fault_manager singleton clear. sensor_fault_proxy active. BIM initialised, bim_trust_state=GREEN. |
| Stimulus | Inject FI-01 (GNSS_LOSS) via get_fault_manager().inject(FI_GNSS_LOSS). Duration = indefinite. |
| Procedure | 1. Inject FI-01. 2. Call sensor_fault_proxy.get_gnss() 10 times. 3. Verify each returns gnss_availability=False. 4. Feed denied measurements to BIM for 1 s. 5. Verify BIM trust score declining. |
| Exit Criteria | 10 proxy calls processed and BIM polled for 1 s. |
| Pass Condition | All 10 calls: gnss_availability=False. bim_trust_score declining after 1 s of denial. FI_01 listed in get_fault_manager().active_faults. |
| Fail Condition | Any call returns valid GNSS while FI-01 active. bim_trust_score not declining. |
| Retry Policy | 5 retries. |
| Required Logs | FI_01 activation event in fault_manager log. GNSS_GATED event from BIM. |
| Repeatability Count | 5 runs. |
| Post-Test State | get_fault_manager().clear_all(). BIM reset. |

## 11.17 UT-FIM-02 — Frozen Core Constants Unchanged

| Test ID | UT-FIM-02 |
| --- | --- |
| Req ID | FIM integrity |
| Test Type | Unit |
| Objective | Verify ESKF frozen constants are unchanged after all fault injection presets. |
| Entry Criteria | ErrorStateEKF instantiated. fault_manager clear. |
| Stimulus | Inject PRESET_VIO_GNSS, then PRESET_VIO_RADALT_TERM, then clear_all(). |
| Procedure | 1. Record _ACC_BIAS_RW and _GYRO_BIAS_RW before any injection. 2. Inject PRESET_VIO_GNSS. 3. Read constants. 4. Clear. 5. Inject PRESET_VIO_RADALT_TERM. 6. Read constants. 7. Clear. 8. Read constants. |
| Exit Criteria | All 3 check points read. |
| Pass Condition | _ACC_BIAS_RW = 9.81e-7 at all 3 check points. _GYRO_BIAS_RW = 4.04e-8 at all 3 check points. Zero exceptions. |
| Fail Condition | Any constant deviates from expected value at any checkpoint. |
| Retry Policy | 3 retries. |
| Required Logs | Constant values at each checkpoint. |
| Repeatability Count | 3 runs. |
| Post-Test State | Fault manager clear. ESKF constants verified unchanged. |

# 12. Integration Test Plan

REFERENCE: Integration tests §12.1–§12.12 are identical to SRS-MicroMind-v1.1 §12.1–§12.12 and are carried forward without modification. Entry criteria, stimuli, procedures, pass/fail thresholds, retry policies, and required logs are unchanged. IT-PLN-01 and IT-PLN-02 remain as defined but not yet implemented — they are SB-5 Phase B deliverables per §17.

## 12.1 IT-NAV-01 — GNSS Loss During Cruise

| Test ID | IT-NAV-01 |
| --- | --- |
| Req IDs | NAV-01, NAV-04, NAV-05, EW-03 |
| Test Type | Integration |
| Objective | Verify navigation mode transition, drift progression, and corridor adherence during 90 km INS-only segment. |
| Entry Criteria | Vehicle in phase PRE_GNSS_DENIAL at km 30. bim_trust_state=GREEN. TRN enabled. VIO in ACTIVE mode. |
| Stimulus | Trigger GNSS_LOST event at km 30 (via disturbance schedule). Duration: 90 km of INS-only. |
| Procedure | 1. Run full mission to km 30. 2. Inject GNSS_LOST. 3. Verify NAV_MODE_TRANSITION from GNSS to TRN_PRIMARY within 500 ms. 4. Continue to km 120. 5. Check drift at km 60, 100, 120 against C-2 envelopes. 6. Verify TRN corrections logged every 2 km. 7. Verify no CORRIDOR_BREACH before km 76.9 (1σ seed). |
| Exit Criteria | Mission reaches km 120 OR any ABORT triggered. |
| Pass Condition | NAV_MODE_TRANSITION latency ≤ 500 ms. Drift at km 60 ∈ [5,80], km 100 ∈ [12,350], km 120 ∈ [15,650] m. TRN_SUPPRESSED events ≤ 3 consecutive. No CORRIDOR_BREACH for canonical seed 42. |
| Fail Condition | Transition > 2 s. Any drift outside ceiling. > 3 consecutive TRN suppressions without mode switch. |
| Retry Policy | 3 retries with different seeds. |
| Recovery Behaviour | On CORRIDOR_BREACH: log event, increase TRN polling 2→1 km. SHM if breach persists > 15 s. |
| Required Logs | NAV_MODE_TRANSITION × 1+. TRN_SUPPRESSED (any). CORRIDOR_BREACH (if any). drift_m trace at 10 Hz. |
| Data Retention | Full JSON run output on failure. |
| Repeatability Count | 3 seeds. |
| Post-Test State | Mission completed or SHM. All logs preserved. |

## 12.2 IT-NAV-02 — TRN Radalt Dropout Recovery

| Test ID | IT-NAV-02 |
| --- | --- |
| Req IDs | NAV-02, NAV-04 |
| Test Type | Integration |
| Objective | Verify TRN suspends on RADALT dropout, VIO takes over, and TRN resumes on RADALT recovery. |
| Entry Criteria | Vehicle in GNSS_DENIED phase. TRN active (NCC ≥ 0.45). VIO ACTIVE. RADALT delivering 10 Hz. |
| Stimulus | Inject FI-RADALT_LOSS at t=0 for 60 s. Then clear fault. |
| Procedure | 1. Verify TRN active at t=0. 2. Inject FI-RADALT_LOSS. 3. At t=1 s: verify TRN_RADALT_SUSPENDED logged. 4. Verify nav continues on VIO+INS. 5. At t=60 s: clear fault. 6. Within 5 s: verify TRN_RESUMED logged. 7. Verify no position jump > 20 m on resume. |
| Exit Criteria | TRN_RESUMED logged and 10 s elapsed, OR ABORT triggered. |
| Pass Condition | TRN_RADALT_SUSPENDED within 1 s of fault. Nav on VIO+INS with drift < 50 m over 60 s. TRN_RESUMED within 5 s of fault clear. Position jump < 20 m. |
| Fail Condition | TRN attempts correction without RADALT. Position jump > 50 m. TRN not resumed after 15 s. |
| Required Logs | TRN_RADALT_SUSPENDED, VIO_OUTAGE_EXTENDED (if any), TRN_RESUMED, position_discontinuity_m. |
| Repeatability Count | 3 runs. |
| Post-Test State | TRN active. RADALT fault cleared. |

## 12.3 IT-NAV-03 — VIO Dropout and Recovery

| Test ID | IT-NAV-03 |
| --- | --- |
| Req IDs | NAV-03, NAV-04 |
| Test Type | Integration |
| Objective | Verify VIO OUTAGE declared after 3 s, TRN takes over, and VIO re-integrates without position jump on recovery. |
| Entry Criteria | VIO ACTIVE. TRN available. GNSS denied. |
| Stimulus | Suppress VIO frame delivery for 10 s via sensor_fault_proxy FI-02. |
| Procedure | 1. Confirm vio_mode=ACTIVE. 2. Inject FI-02 (VIO_LOSS). 3. At t=3 s: verify vio_mode=OUTAGE. VIO_OUTAGE_EXTENDED logged. 4. Verify nav continues on TRN+INS. 5. At t=10 s: clear FI-02. 6. Within 5 s: verify vio_mode=ACTIVE. 7. Verify position_discontinuity_m < 20 m on re-integration. |
| Exit Criteria | vio_mode=ACTIVE after recovery OR 20 s timeout. |
| Pass Condition | OUTAGE at 3 s ±100 ms. Recovery to ACTIVE within 5 s of fault clear. position_discontinuity_m < 20 m. |
| Fail Condition | OUTAGE not declared after 5 s. position_discontinuity_m > 50 m. vio_mode stuck in OUTAGE. |
| Required Logs | VIO_OUTAGE_EXTENDED: {duration_ms}. NAV_MODE_TRANSITION on recovery. position_discontinuity_m. |
| Repeatability Count | 3 runs. |
| Post-Test State | VIO ACTIVE. Fault cleared. |

## 12.4 IT-EW-01 — GNSS Spoof Detection and TRN Transition

| Test ID | IT-EW-01 |
| --- | --- |
| Req IDs | EW-03, NAV-05 |
| Test Type | Integration |
| Objective | Full-chain test: spoof injected, BIM detects within 250 ms, GNSS gated, nav switches to TRN-primary. |
| Entry Criteria | bim_trust_state=GREEN. GNSS stream active. TRN available. |
| Stimulus | Inject GNSS_SPOOF_INJECTOR with offset = 500 m north at t=0. |
| Procedure | 1. Record t0. 2. Inject spoof. 3. Poll BIM at 100 Hz. 4. Record t_red. 5. Verify GNSS_GATED within 50 ms of RED. 6. Verify active_nav_mode=TRN_PRIMARY within 500 ms of RED. 7. Verify spoofed position not in ESKF state. |
| Exit Criteria | bim_trust_state=RED and nav_mode=TRN_PRIMARY confirmed OR 1 s timeout. |
| Pass Condition | detection_latency_ms ≤ 250. GNSS_GATED within 50 ms. nav_mode=TRN_PRIMARY within 500 ms. ESKF position delta from spoof < 5 m. |
| Fail Condition | Spoofed position incorporated into ESKF. detection_latency_ms > 500. nav_mode not TRN_PRIMARY within 2 s. |
| Required Logs | SPOOF_DETECTED, GNSS_GATED, NAV_MODE_TRANSITION to TRN_PRIMARY. |
| Repeatability Count | 10 runs (statistical latency distribution). |
| Post-Test State | BIM reset to GREEN after 10 clean readings. Nav in TRN_PRIMARY. |

## 12.5 IT-EW-02 — EW Map Contradiction and Route Replan

| Test ID | IT-EW-02 |
| --- | --- |
| Req IDs | EW-01, EW-02 |
| Test Type | Integration |
| Objective | Two simultaneous opposing EW threats: map update, replan triggered, new route clears both threats. |
| Entry Criteria | Planned route: (0,0) → (10000,0). No active threats. |
| Stimulus | Simultaneously inject: Threat A at (5000, 300) radius 200 m type JAMMER. Threat B at (5000, -300) radius 200 m type JAMMER. |
| Procedure | 1. Inject both threats. 2. Verify EW map update ≤ 500 ms. 3. Verify ROUTE_REPLAN triggered. 4. Verify replan_latency_ms ≤ 1000. 5. Verify new route clears both threats by ≥ 500 m. 6. If no valid route: verify ROUTE_DEADEND and SHM. |
| Exit Criteria | New route validated OR ROUTE_DEADEND declared. |
| Pass Condition | Map update ≤ 500 ms. Replan ≤ 1000 ms. threat_clearance_m ≥ 500 from both threats. route_validated=True. |
| Fail Condition | Map not updated within 1 s. Replan > 2 s. New route enters threat envelope. |
| Required Logs | EW map update timestamps, ROUTE_REPLAN, threat_clearance_m × 2, ROUTE_DEADEND (if triggered). |
| Repeatability Count | 3 runs. |
| Post-Test State | Threat map cleared. Route replanned or SHM. |

## 12.6 IT-PLN-01 — Dynamic Retask Under GNSS Denial

| Test ID | IT-PLN-01 |
| --- | --- |
| Req IDs | PLN-02, NAV-04 |
| Test Type | Integration |
| Objective | Verify valid Retask Command accepted and executed while vehicle is in GNSS_DENIED phase with TRN-primary navigation. |
| Entry Criteria | Vehicle in GNSS_DENIED phase at km 50. nav_mode=TRN_PRIMARY. VIO available. Valid RetaskCommand prepared. |
| Stimulus | Send RetaskCommand: {new_target=(12000,3000), valid_signature, phase_check=GNSS_DENIED}. |
| Procedure | 1. Send RetaskCommand. 2. Record t0. 3. Verify envelope validated within 200 ms. 4. Verify new route computed using TRN-primary position estimate. 5. Verify PX4 waypoint queue refreshed. 6. Record t1 (first new waypoint commanded). 7. Compute retask_latency = t1 - t0. |
| Exit Criteria | New waypoint commanded to PX4 OR timeout 30 s. |
| Pass Condition | retask_latency_ms ≤ 10000. new_envelope_valid=True. New route avoids NFZs. PX4 waypoint refreshed. Position uncertainty from GNSS_DENIED mode accounted for in route margin. |
| Fail Condition | retask_latency_ms > 30000. Invalid envelope not rejected. Route ignores TRN position uncertainty. RETASK_ROLLBACK triggered without cause. |
| Required Logs | RETASK_RECEIVED, RETASK_ACCEPTED, retask_latency_ms, ROUTE_PLANNED with new target. |
| Repeatability Count | 3 runs. |
| Post-Test State | Vehicle on new route. Retask state=ACTIVE→NONE on completion. |

## 12.7 IT-PLN-02 — Retask Rollback on Route Failure

| Test ID | IT-PLN-02 |
| --- | --- |
| Req IDs | PLN-02, PLN-03 |
| Test Type | Integration |
| Objective | Verify RETASK_ROLLBACK triggered and previous mission continues when new route computation fails at all constraint levels. |
| Entry Criteria | Vehicle in CRUISE at km 40. Current route valid. RetaskCommand prepared with new target in nearly impassable EW region. |
| Stimulus | Send RetaskCommand with new target at (5000, 5000). EW cost map: 90% cost=1.0 around new target. |
| Procedure | 1. Send RetaskCommand. 2. Route planner attempts levels 0→3. All fail. 3. Verify RETASK_ROUTE_FAILED logged. 4. Verify RETASK_ROLLBACK logged with previous_target. 5. Verify vehicle continues on original route. 6. Verify original envelope still active. |
| Exit Criteria | RETASK_ROLLBACK confirmed and original mission continues. |
| Pass Condition | RETASK_ROUTE_FAILED logged. RETASK_ROLLBACK logged. Vehicle on original route within 5 s. Original envelope active. |
| Fail Condition | Vehicle diverts to failed new route. Original envelope replaced with invalid envelope. |
| Required Logs | RETASK_ROUTE_FAILED, RETASK_ROLLBACK: {previous_target, reason}. |
| Repeatability Count | 3 runs. |
| Post-Test State | Vehicle on original route. Retask state=NONE. |

## 12.8 IT-PX4-01 — 30-Minute OFFBOARD Continuity

| Test ID | IT-PX4-01 |
| --- | --- |
| Req IDs | PX4-01 |
| Test Type | Integration |
| Objective | Verify OFFBOARD continuity ≥ 99.5% over 30-minute mission. |
| Entry Criteria | PX4 SITL in OFFBOARD mode. Continuous setpoint loop active. |
| Stimulus | 30-minute waypoint navigation mission with realistic setpoints. |
| Procedure | 1. Log setpoint_rate_hz and offboard_active every 1 s. 2. Count OFFBOARD_LOSS events. 3. Compute continuity = (1800 - total_offboard_loss_s) / 1800 × 100%. |
| Exit Criteria | 30 minutes elapsed. |
| Pass Condition | offboard_continuity ≥ 99.5%. setpoint_rate_hz ≥ 20 throughout. OFFBOARD_LOSS_count ≤ 1. |
| Fail Condition | offboard_continuity < 99.0%. OFFBOARD_LOSS_count > 1. Any unrecovered OFFBOARD Loss. |
| Required Logs | setpoint_rate_hz at 1 Hz, OFFBOARD_LOSS events, offboard_continuity_percent. |
| Repeatability Count | 3 runs. |
| Post-Test State | OFFBOARD active. |

## 12.9 IT-PX4-02 — PX4 Reboot Recovery

| Test ID | IT-PX4-02 |
| --- | --- |
| Req IDs | PX4-04, PX4-05 |
| Test Type | Integration |
| Objective | Verify PX4 reboot detected, OFFBOARD restored, Checkpoint loaded, mission resumed. |
| Entry Criteria | Vehicle in GNSS_DENIED at km 50. Checkpoint written at km 50. PX4 SITL running. |
| Stimulus | Send SIGKILL to PX4 SITL process at km 50. PX4 restarts (simulated reboot, sequence number reset). |
| Procedure | 1. Kill PX4 SITL. 2. Record t0. 3. Measure t_detected = HEARTBEAT seq reset detected. 4. SHM entered. 5. PX4 restart (auto). 6. Measure t_offboard = OFFBOARD restored. 7. Load Checkpoint. 8. Measure t_nav = NAV_RESTORED. 9. Measure position error vs Checkpoint position. |
| Exit Criteria | NAV_RESTORED logged and mission resumes, OR ABORT_MISS after 30 s no restore. |
| Pass Condition | t_detected - t0 ≤ 3 s. t_offboard - t0 ≤ 15 s. t_nav - t0 ≤ 25 s. position_error_m < 20 m. Mission resumes within 30 s. |
| Fail Condition | Reboot undetected > 10 s. OFFBOARD not restored in 30 s. position_error_m > 50 m. |
| Required Logs | PX4_REBOOT_DETECTED, OFFBOARD_RESTORED, CHECKPOINT_RESTORED, NAV_RESTORED, MISSION_RESUMED with all latency fields. |
| Repeatability Count | 3 runs. |
| Post-Test State | Mission running on restored Checkpoint state. |

## 12.10 IT-TERM-01 — Terminal Approach with Decoy, Full Chain

| Test ID | IT-TERM-01 |
| --- | --- |
| Req IDs | TERM-01, TERM-02, TERM-03 |
| Test Type | Integration |
| Objective | Full terminal chain: DMRL acquires primary, decoy detected and rejected, L10s-SE PROCEED, approach authorised. |
| Entry Criteria | Vehicle in TERMINAL_APPROACH. DMRL active. L10s-SE ready. Civilian confidence = 0.10. Valid envelope. |
| Stimulus | EO scene: primary target bearing 000°, decoy bearing 010°. Decoy confidence ramps to 0.85 over 4 frames, then drops to 0.40. |
| Procedure | 1. Feed frames 1–3 (decoy confidence 0.80–0.85 × 3). Verify DECOY_DETECTED at frame 2. DECOY_REJECTED at frame 3. 2. After rejection: feed 5 clean primary-only frames. 3. Verify dwell_frame_count ≥ 5. lock_confidence ≥ 0.85. 4. L10s-SE invoked: civilian=0.10. Verify PROCEED. 5. approach_authorised=True. |
| Exit Criteria | approach_authorised=True OR ABORT triggered. |
| Pass Condition | DECOY_REJECTED at frame 3. Primary lock maintained. L10s PROCEED within 2 s. approach_authorised=True after 5 clean dwell frames. L10S_DECISION logged. |
| Fail Condition | Decoy accepted. L10s bypassed. approach_authorised before 5 frames. |
| Required Logs | DECOY_DETECTED, DECOY_REJECTED, DMRL_LOCK_ACQUIRED, L10S_INVOKED, L10S_DECISION: {PROCEED}. |
| Repeatability Count | 5 runs. |
| Post-Test State | COMMITTED state. Logs preserved. |

## 12.11 IT-TERM-02 — L10s Abort on Civilian Detection

| Test ID | IT-TERM-02 |
| --- | --- |
| Req IDs | TERM-03 |
| Test Type | Integration |
| Objective | Verify L10s ABORT on civilian_confidence ≥ 0.70. |
| Entry Criteria | DMRL lock acquired (confidence 0.90). Vehicle in TERMINAL_APPROACH. L10s-SE ready. |
| Stimulus | L10sInputs: civilian_detection_confidence = 0.75. |
| Procedure | 1. Invoke L10s-SE with civilian=0.75. 2. Measure decision_latency_ms. 3. Verify decision=ABORT. 4. Verify terminal sequence halted. 5. Verify L10S_ABORT logged. |
| Exit Criteria | L10s decision received. |
| Pass Condition | decision=ABORT. abort_reason=CIVILIAN_DETECTED. decision_latency_ms ≤ 2000. approach_authorised=False. L10S_ABORT logged. |
| Fail Condition | decision=PROCEED. abort_reason≠CIVILIAN. decision_latency_ms > 2000. |
| Required Logs | L10S_INVOKED, L10S_DECISION: {ABORT, CIVILIAN_DETECTED, decision_latency_ms}, L10S_ABORT. |
| Repeatability Count | 10 runs (verify 100% of 10 produce ABORT). |
| Post-Test State | Vehicle in SHM or re-evaluation hold. |

## 12.12 IT-MM-01 — SHM Activation and Deactivation

| Test ID | IT-MM-01 |
| --- | --- |
| Req IDs | MM-03 |
| Test Type | Integration |
| Objective | Verify SHM entered within 2 s for all 8 defined triggers and deactivated correctly. |
| Entry Criteria | Vehicle in GNSS_DENIED at km 50. All subsystems nominal. |
| Stimulus | 8 separate trigger injections in separate runs: (a) bim_trust_score < 0.05 for 5 s, (b) ENVELOPE_VIOLATION, (c) OPERATOR_ABORT, (d) OFFBOARD Loss > 5 s, (e) L10s ABORT, (f) ROUTE_DEADEND, (g) FSM_UNDEFINED_STATE, (h) corridor_breach_flag=True for 15 s. |
| Procedure | For each trigger: 1. Inject trigger condition. 2. Measure shm_entry_latency_ms. 3. Verify PX4 in loiter. 4. Clear trigger. 5. Send clearance command. 6. Verify SHM deactivated. 7. Verify SHM_DEACTIVATED logged. |
| Exit Criteria | All 8 triggers tested (or any trigger fails to produce SHM). |
| Pass Condition | shm_entry_latency_ms ≤ 2000 for all 8. PX4 in loiter for all. SHM_ACTIVATED logged for all 8 with correct trigger_reason. SHM_DEACTIVATED logged on clearance. |
| Fail Condition | Any trigger with latency > 5 s. PX4 not in loiter after 5 s. Wrong trigger_reason logged. |
| Retry Policy | 3 retries per trigger. |
| Required Logs | SHM_ACTIVATED × 8 with trigger_reasons. SHM_DEACTIVATED × 8 with recovery_reason. |
| Repeatability Count | 1 run per trigger (8 total). |
| Post-Test State | SHM deactivated. Vehicle in CRUISE. |

# 13. Stress and Failure Campaign

REFERENCE: Stress tests §13.1–§13.7 are identical to SRS-MicroMind-v1.1 §13.1–§13.7 and are carried forward without modification. ST-DISK-01 (§13.8) is updated below to include checkpoint accumulation as a failure trigger (E-01 correction).

## 13.1 ST-END-01 — 2-Hour Mission Endurance

| Test ID | ST-END-01 |
| --- | --- |
| Req IDs | RS-01, RS-02, RS-04, NAV-01, PX4-01 |
| Test Type | Stress |
| Objective | Verify stack survives 2-hour continuous mission: no crashes, no OOM, log completeness ≥ 99%, drift bounded. |
| Entry Criteria | Full stack running. PX4 SITL. GNSS denied after km 30. Nominal EW environment. 120-minute clock started. |
| Stimulus | BCMP-1 scenario repeated × 2. GNSS denied at km 30. All subsystems at nominal load. |
| Expected Degraded Behaviour | Nav drift bounded by TRN+VIO. CPU ≤ 70% mean. log_completeness ≥ 99%. rss_growth < 50 MB. |
| Exit Criteria | 120 min elapsed OR abort condition reached. |
| Pass Condition | Zero crashes. Zero OOM kills. log_completeness ≥ 99%. rss_growth < 50 MB over 2 h. CPU mean ≤ 70%. |
| Fail Condition | Any crash or OOM kill. log_completeness < 95%. rss_growth > 200 MB. Any critical event dropped. |
| Abort Conditions | CPU > 95% sustained > 60 s. OOM kill. log_completeness < 90%. Any nav NaN. |
| Max Recovery Time | Process restart: nav restored ≤ 10 s. Mission resumes within 30 s of any recovery. |
| Required Logs | CPU at 1 Hz, RSS at 60 s, log_completeness at 60 s, drift_m at 10 Hz throughout. |
| Data Retention | Full log archive on failure. |
| Repeatability Count | 1 run minimum. 3 runs for certification baseline. |
| Post-Test State | Full log archive. Drift and CPU time-series preserved. |

## 13.2 ST-CPU-01 — CPU Overload via Concurrent Operations

| Test ID | ST-CPU-01 |
| --- | --- |
| Req IDs | RS-01, MM-04 |
| Test Type | Stress |
| Objective | Verify critical operations (nav, OFFBOARD setpoints) maintain function when CPU is overloaded by concurrent EW+replan+terminal. |
| Failure Trigger | 5 new EW threats simultaneously + forced route replan + DMRL active + VIO update active. |
| Expected Degraded Behaviour | EW map rate reduced to 0.5 Hz. VIO weight halved. Setpoint rate maintained ≥ 20 Hz. Nav continues. |
| Exit Criteria | 30 minutes elapsed OR abort condition. |
| Pass Condition | setpoint_rate_hz ≥ 20 throughout. nav_confidence never < 0.50. CPU_HIGH logged. No OFFBOARD Loss. |
| Fail Condition | setpoint_rate_hz < 10 for > 500 ms. Nav NaN. OFFBOARD Loss. |
| Abort Conditions | CPU > 98% sustained > 60 s. Nav NaN. OFFBOARD unrecovered for > 30 s. |
| Max Recovery Time | CPU below 90% within 30 s of trigger removal. |
| Required Logs | CPU at 1 Hz, setpoint_rate_hz, CPU_HIGH events, SETPOINT_RATE_LOW (if any). |
| Repeatability Count | 3 runs. |
| Post-Test State | CPU normalised. All subsystems operational. |

## 13.3 ST-MEM-01 — Memory Leak Detection Over 2 Hours

| Test ID | ST-MEM-01 |
| --- | --- |
| Req IDs | RS-04 |
| Test Type | Stress |
| Objective | Verify no unbounded RSS growth over 2-hour run. Linear regression slope < 25 MB/hour. |
| Failure Trigger | Continuous full-stack mission for 120 minutes. |
| Expected Degraded Behaviour | RSS growth bounded. No OOM kill. |
| Exit Criteria | 120 min elapsed OR OOM kill OR RSS growth > 200 MB. |
| Pass Condition | Linear regression slope ≤ 25 MB/h. No OOM kill. Zero process restarts due to memory. |
| Fail Condition | Slope > 50 MB/h. OOM kill in any process. |
| Abort Conditions | OOM kill. RSS growth > 200 MB at any point. |
| Required Logs | rss_total_mb, rss_per_process at 60 s intervals. OOM events. |
| Data Retention | Full RSS time-series on failure. |
| Repeatability Count | 1 run minimum. |
| Post-Test State | Processes still running. Full RSS trace preserved. |

## 13.4 ST-GNSS-01 — 30-Minute GNSS Spoof Endurance

| Test ID | ST-GNSS-01 |
| --- | --- |
| Req IDs | EW-03, NAV-05 |
| Test Type | Stress |
| Objective | Verify GNSS never incorporated and BIM remains RED throughout 30-minute continuous spoof attack. |
| Failure Trigger | Continuous GNSS position drift at 100 m/minute for 30 minutes. |
| Expected Degraded Behaviour | BIM in RED throughout. Nav on TRN+VIO+INS. Spoof never in ESKF. |
| Exit Criteria | 30 min elapsed OR spoofed position in ESKF (fail). |
| Pass Condition | bim_trust_state=RED throughout. gnss_gated=True throughout. ESKF position error from spoof < 5 m at all times. |
| Fail Condition | Spoofed position incorporated into ESKF at any moment. |
| Abort Conditions | Spoofed position in ESKF. |
| Required Logs | bim_trust_score at 10 Hz, gnss_gated flag, ESKF state vs spoof offset throughout. |
| Repeatability Count | 3 runs. |
| Post-Test State | BIM recovers to GREEN after 10 clean readings. Full log preserved. |

## 13.5 ST-PX4-01 — OFFBOARD Stress Under Max Load

| Test ID | ST-PX4-01 |
| --- | --- |
| Req IDs | PX4-01 |
| Test Type | Stress |
| Objective | Verify setpoint rate ≥ 20 Hz maintained under maximum simultaneous subsystem load. |
| Failure Trigger | Simultaneous: replan active, EW update at 2 Hz, DMRL active at 25 Hz, VIO at 25 Hz. |
| Expected Degraded Behaviour | Setpoint scheduler priority highest. Rate dips to 18 Hz briefly but recovers within 200 ms. |
| Exit Criteria | 30 min elapsed OR OFFBOARD unrecovered. |
| Pass Condition | setpoint_rate_hz ≥ 18 Hz minimum (any instantaneous). Mean ≥ 20 Hz. No OFFBOARD Loss. |
| Fail Condition | setpoint_rate_hz < 10 Hz for > 500 ms. Any unrecovered OFFBOARD Loss. |
| Abort Conditions | OFFBOARD unrecovered for > 30 s. |
| Required Logs | setpoint_rate_hz at 1 Hz, OFFBOARD_LOSS (if any), SETPOINT_RATE_LOW events. |
| Repeatability Count | 3 runs. |
| Post-Test State | OFFBOARD active. Setpoint rate ≥ 20 Hz. |

## 13.6 ST-MULTI-01 — Simultaneous GNSS + VIO + RADALT Failure

| Test ID | ST-MULTI-01 |
| --- | --- |
| Req IDs | NAV-02, NAV-03, PX4-03 |
| Test Type | Stress |
| Objective | Verify system enters SHM correctly when all three nav correction sources fail simultaneously. |
| Failure Trigger | PRESET_VIO_GNSS + FI-RADALT_LOSS injected simultaneously for 60 s. |
| Expected Degraded Behaviour | GNSS gated. VIO→OUTAGE. TRN suspended. INS-only propagation. Drift accumulates. SHM if corridor breach. |
| Exit Criteria | 60 s elapsed OR SHM entered OR ABORT triggered. |
| Pass Condition | All 3 faults logged within 1 s. Nav continues on INS-only. SHM triggered if drift > 400 m (corridor 80%). All fault events logged. |
| Fail Condition | Nav NaN. System crashes. SHM not entered despite drift > corridor. |
| Abort Conditions | Nav NaN. Process crash. |
| Required Logs | All fault injection events, nav_mode trace, drift_m trace, SHM_ACTIVATED (if triggered). |
| Data Retention | Full run artefacts on failure. |
| Repeatability Count | 3 runs. |
| Post-Test State | All faults cleared. Nav re-initialised. |

## 13.7 ST-RESTART-01 — Process Restart During GNSS-Denied Flight

| Test ID | ST-RESTART-01 |
| --- | --- |
| Req IDs | RS-03, PX4-05 |
| Test Type | Stress |
| Objective | Verify watchdog detects Navigation Manager crash, SHM entered, process restarted, nav restored from Checkpoint within 10 s. |
| Failure Trigger | SIGKILL to Navigation Manager process at km 50 (mid GNSS-denied segment). |
| Expected Degraded Behaviour | Watchdog fires within 500 ms. SHM entered. Process restarted within 2 s. Checkpoint loaded. Nav running within 10 s. |
| Exit Criteria | NAV_RESTORED logged and mission continues, OR ABORT_MISS after 30 s. |
| Pass Condition | Failure detected ≤ 500 ms. Process restarted ≤ 2 s. nav_restored_latency_ms ≤ 10000. position_error_m < 20 m after restore. |
| Fail Condition | Detection > 2 s. Nav not restored in 30 s. position_error_m > 50 m. |
| Abort Conditions | Nav not restored in 30 s → ABORT_MISS. |
| Required Logs | PROCESS_FAILURE, PROCESS_RESTARTED, CHECKPOINT_RESTORED, NAV_RESTORED with all latency fields. |
| Repeatability Count | 3 runs. |
| Post-Test State | Mission resumed or ABORT_MISS. All logs preserved. |

## 13.8 ST-DISK-01 — Disk Exhaustion During Logging (Updated for E-01)

| Test ID | ST-DISK-01 |
| --- | --- |
| Req IDs | RS-02, PX4-05 |
| Test Type | Stress |
| Objective | Verify critical events not dropped when disk exhausted. RAM buffer active. Checkpoint accumulation does not independently exhaust disk. |
| Failure Trigger | Fill disk to 99% capacity while full-stack mission runs. Separately: allow checkpoint accumulation without purge to verify purge policy activates correctly. |
| Expected Degraded Behaviour | LOG_DISK_WARNING at < 200 MB free. Non-critical telemetry suspended. Critical events in RAM ring buffer. Checkpoint retention reduced to 2 at disk < 500 MB free. Checkpoint writes suspended at disk < 100 MB free. |
| Exit Criteria | 30 minutes elapsed OR critical event dropped (fail) OR checkpoint accumulation exceeds 5 without purge (fail). |
| Pass Condition | dropped_critical_events_count = 0 throughout. LOG_DISK_WARNING logged. RAM buffer active. Mission continues. CHECKPOINT_RETENTION_REDUCED logged at disk < 500 MB. checkpoints_retained_count ≤ 5 at all times. |
| Fail Condition | Any CRITICAL event dropped. Any checkpoint accumulates without purge beyond rolling window limit. Mission aborted due to logging failure. |
| Abort Conditions | Critical event confirmed dropped. |
| Required Logs | LOG_DISK_WARNING, LOG_DISK_FULL, dropped_record_count at 1 Hz, CHECKPOINT_PURGED (at each rolling window purge), CHECKPOINT_RETENTION_REDUCED (at disk threshold), RAM buffer flush on disk recovery. |
| Repeatability | 1 run. |
| Post-Test State | Disk freed. RAM buffer flushed. Checkpoint count ≤ 5. |

# 14. Requirement Traceability Matrix

Requirements marked ⚠ have known gaps requiring SB-5 or first HIL sprint work. VIZ-02 and VIZ-03 are re-scoped in v1.2: VIZ-02 implementation is dependency-gated on §17 entry criteria; VIZ-03 (now the Tactical Live Demonstration) is a long-term roadmap item. New fields in §10.15 (Checkpoint schema) are traced to IT-PX4-02 and ST-RESTART-01.

| Req ID | Title | Priority | Unit Tests | Integration Tests | Stress Tests | Regression Suite | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NAV-01 | INS Drift Limit | Critical | UT-NAV-01-A, UT-NAV-01-B | IT-NAV-01 | ST-END-01, ST-RESTART-01 | run_bcmp2_tests.py AT-1/AT-2 | ✅ Complete |
| NAV-02 | TRN Accuracy | Critical | UT-NAV-02-A ⚠ rewrite, UT-NAV-02-B ⚠ rewrite | IT-NAV-01, IT-NAV-02 | ST-MULTI-01 | run_bcmp2_tests.py AT-2 | ⚠ Mechanism changed to OM (v1.3). Existing RADALT-NCC tests invalid. Rewrite required — see GAP-10. |
| NAV-03 | VIO Drift Rate | High | UT-NAV-03 | IT-NAV-03 | ST-MULTI-01 | run_s5_tests.py S5 | ✅ Complete |
| NAV-04 | Nav Transition | High | UT-NAV-03 | IT-NAV-01..03 | ST-MULTI-01 | run_bcmp2_tests.py AT-3 | ✅ Complete |
| NAV-05 | BIM / Spoof | Critical | UT-BIM-01, UT-BIM-02 | IT-EW-01 | ST-GNSS-01 | run_s5_tests.py S2 | ✅ Complete |
| NAV-06 | Two-Theatre | High | UT-NAV-02-B (TRN suppress) | IT-NAV-02 | ST-MULTI-01 | run_bcmp2_tests.py AT-2 | ✅ Complete |
| EW-01 | EW Map Update | High | UT-EW-01 | IT-EW-02 | ST-CPU-01 | run_s5_tests.py S4 | ✅ Complete |
| EW-02 | Route Replan | High | UT-EW-02 | IT-EW-02, IT-PLN-01 | ST-CPU-01 | run_s5_tests.py S4 | ✅ Complete |
| EW-03 | Spoof Rejection | Critical | UT-BIM-01 | IT-EW-01 | ST-GNSS-01 | run_s5_tests.py S2 | ✅ Complete |
| PLN-01 | Route Planner | High | UT-PLN-01 | IT-PLN-01 | ST-CPU-01 | run_s5_tests.py S4 | ✅ Complete |
| PLN-02 | Dynamic Retasking (v1.2) | Medium | UT-PLN-02 | IT-PLN-01, IT-PLN-02 | ST-CPU-01 | ⚠ No regression yet — SB-5 Phase B | ⚠ Regression gap |
| PLN-03 | Dead-End Recovery | High | UT-PLN-03 | IT-EW-02 | ST-CPU-01 | ⚠ No regression yet — SB-5 | ⚠ Regression gap |
| MM-01 | FSM Latency | Critical | run_s5_tests.py S1 | IT-MM-01 | ST-RESTART-01 | run_s5_tests.py S1 | ✅ Complete |
| MM-02 | Envelope Validation | Critical | run_s5_tests.py S5 | IT-TERM-01 | ST-END-01 | run_s5_tests.py S5 | ✅ Complete |
| MM-03 | Safe Hold Mode | Critical | UT-FIM-01 (SHM side) | IT-MM-01 | ST-MULTI-01 | run_s5_tests.py S1 | ✅ Complete |
| MM-04 | Queue Latency | High | ⚠ UT-MM-04 needed — SB-5 | IT-MM-01 (indirect) | ST-CPU-01 | ⚠ No regression | ⚠ Unit test gap |
| PX4-01 | OFFBOARD Rate | Critical | UT-PX4-01 | IT-PX4-01 | ST-PX4-01 | run_bcmp2_tests.py SIL | ✅ Complete |
| PX4-02 | MAVLink Heartbeat | High | ⚠ UT-PX4-02 (stub only) | IT-PX4-02 | ST-PX4-01 | ⚠ No unit test regression | ⚠ Unit test gap |
| PX4-03 | Sensor Driver | High | ⚠ Driver stubs (SIA v1.0) | IT-PX4-01 | ST-MULTI-01 | run_s5_tests.py SIA | ⚠ HIL required |
| PX4-04 | PX4 Reboot Recovery (v1.2) | High | ⚠ No unit test | IT-PX4-02 | ST-RESTART-01 | ⚠ No regression — SB-5 Phase A | ⚠ Unit test gap |
| PX4-05 | Checkpoint (v1.2) | High | ⚠ UT-PX4-05 needed — SB-5 | IT-PX4-02 | ST-RESTART-01 | ⚠ No regression | ⚠ Unit test gap |
| TERM-01 | DMRL Lock | Critical | UT-DMRL-01 | IT-TERM-01 | ST-END-01 | run_s5_tests.py S5 | ✅ Complete |
| TERM-02 | Decoy Rejection | Critical | UT-DMRL-02 | IT-TERM-01 | ST-END-01 | run_s5_tests.py S5 | ✅ Complete |
| TERM-03 | L10s-SE | Critical | run_s5_tests.py S5 | IT-TERM-02 | ST-END-01 | run_s5_tests.py S5 | ✅ Complete |
| RS-01 | CPU Ceiling | High | ⚠ Jetson only (GAP-06) | ST-CPU-01 | ST-CPU-01 | ⚠ No CI regression | ⚠ Hardware-only |
| RS-02 | Log Completeness (v1.2) | High | ⚠ UT-RS-02 needed — SB-5 | IT-PX4-01 (indirect) | ST-DISK-01 | ⚠ No regression | ⚠ Unit test gap |
| RS-03 | Process Restart (v1.2) | Medium | ⚠ UT-RS-03 needed — SB-5 | IT-PX4-02 (indirect) | ST-RESTART-01 | ⚠ No regression | ⚠ Unit test gap |
| RS-04 | Memory Bounds (v1.2) | Medium | ⚠ SIL only | ST-END-01 | ST-MEM-01 | ⚠ No CI regression | ⚠ Long-run only |
| VIZ-01 | Static Mission Report | High | run_bcmp2_tests.py AT-2 | run_bcmp2_tests.py AT-2 | — | run_bcmp2_tests.py AT-2 | ✅ Complete |
| VIZ-02 | Long-Range Demo (v1.2 re-scope) | High — dependency-gated | ⚠ UT-VIZ-02 — after §17 PASS | ⚠ IT-VIZ-02 — after §17 PASS | — | ⚠ No regression | ⚠ §17 prerequisites required |
| VIZ-03 | Tactical Live Demo (v1.2 roadmap) | Roadmap — not SB-5 | N/A — roadmap | N/A — roadmap | N/A | N/A | Roadmap: post-HIL |
| FR-107 (OM) | L2 Orthophoto Matching | Critical | ⚠ None — new mechanism | ⚠ None | ⚠ None | ⚠ None — Sprint C deliverable | ⚠ Not yet SIL-tested (GAP-10) |
| FR-110a | BIM Adaptive Spoof Resistance threshold | High | ⚠ UT-BIM-03 needed | IT-EW-01 (partial) | ST-GNSS-01 (partial) | ⚠ None | ⚠ Threshold defined (KPI-BIM-02). Test not yet written. |

# 15. Known Gaps and Open Items

## 15.1 Critical Gaps — Must Address Before TASL Engagement

| Gap ID | Req | Gap Description | Required Action | Sprint |
| --- | --- | --- | --- | --- |
| GAP-01 | VIZ-02 | Long-range mission demonstration not yet implemented. Implementation is dependency-gated on §17 entry criteria (EC-01 through EC-11 must be PASS first). | SB-5 Phase C: implement VIZ-02 after Phase A and Phase B prerequisites are confirmed PASS. bcmp2_animate.py with log-driven event overlays. | SB-5 Phase C |
| GAP-02 | PLN-02 | No regression test for dynamic retasking, terrain regeneration ordering, EW staleness check, or rollback completeness. Corrections R-01..R-05 now in spec; not yet in code. | SB-5 Phase B: implement PLN-02 with all R-01..R-05 corrections. Add UT-PLN-02, IT-PLN-01, IT-PLN-02 to run_bcmp2_tests.py. | SB-5 Phase B |
| GAP-03 | PLN-03 | No automated dead-end recovery regression. | SB-5 Phase B: add UT-PLN-03 to run_bcmp2_tests.py. | SB-5 Phase B |
| GAP-04 | PX4-04/05 | No PX4 reboot or Checkpoint unit tests. v1.2 Checkpoint schema has 6 new mandatory fields. Correction P-01 (SHM in Checkpoint) and P-02 (Mission Manager gate) in spec; not yet in code. | SB-5 Phase A: implement IT-PX4-02, UT-PX4-05 with updated schema. Verify D8a gate in Appendix D. | SB-5 Phase A |
| GAP-05 | MM-04 | Queue latency requirement defined but no unit test or instrumentation. | SB-5 Phase B: instrument event bus. Add UT-MM-04 with 100 ms gate. | SB-5 Phase B |
| GAP-10 | NAV-02 (v1.3) | Zero SIL tests exist for orthophoto image matching. UT-NAV-02-A and UT-NAV-02-B test the superseded RADALT-NCC mechanism. NAV-02 cannot be declared validated until OM SIL tests pass. | Sprint C: implement OrthophotoMatchingStub, rewrite UT-NAV-02-A and UT-NAV-02-B for OM mechanism. Add EC-13 to §17. | Sprint C (before SB-5) |
| GAP-11 | NAV-01 | C-2 drift envelopes not characterised beyond km 120. AVP-04 GNSS-denied segments of 100+ km require envelope extension. Current values not valid evidence for AVP-04 performance claims. | Monte Carlo N=300 extension to km 200 using STIM300 noise profile once OI-03 ALS-250 data is available. | Before AVP-04 capability claims |
| GAP-12 | FR-110a | BIM adaptive spoof resistance has quantified threshold (KPI-BIM-02: <= 1% false-negative under 3-sigma plausible-path adaptive spoof) but no test implements this scenario. UT-BIM-01 tests only naive position-jump spoofing. | Write UT-BIM-03 with adaptive spoof waveform that passes Doppler and multi-constellation checks. Add KPI-BIM-02 gate. | Before HIL |
| GAP-13 | VIZ-03 | VIZ-03 prerequisite incorrectly stated HIL on Jetson as a gate. Corrected by AD-22 (06 April 2026). SRS §9.4 updated. MICROMIND_DECISIONS.md updated. | No action required — correction recorded. | AD-22 closed. |

## 15.2 High-Priority Gaps — First HIL Sprint

| Gap ID | Req | Gap Description | Required Action |
| --- | --- | --- | --- |
| GAP-06 | RS-01 | CPU utilisation cannot be validated in SIL. Requires Jetson Orin NX hardware. | HIL Sprint 1: deploy stack on Jetson NX, run ST-CPU-01 and ST-END-01 on hardware. |
| GAP-07 | PX4-03 | Sensor drivers are MAVLink stubs only (SIA v1.0). Real MIPI/USB3 EO driver not tested. | HIL Sprint 1: validate real EO driver, verify frame rate and latency. |
| GAP-08 | RS-03 | Watchdog restartability classification now in spec (RS-03 v1.2) but not yet implemented. No automated regression. | SB-5 Phase D: add UT-RS-03 with SIGKILL stimulus, verify restartability_class logic. |
| GAP-09 | RS-02 | Log completeness has no formal unit test. Rolling policy now in spec (RS-02 v1.2) but not yet implemented. | SB-5 Phase D: add UT-RS-02 with synthetic log load, rolled file verification, dropped_count check. |

## 15.3 Architecture Ambiguities

| Item | Description | Recommended Resolution |
| --- | --- | --- |
| AMB-01 | NAV-06 σ_terrain auto-detection: now defined in v1.2 (std dev of RADALT over 1 km in PRE_GNSS_DENIAL). Implementation not yet verified. | Verify implementation matches spec before SB-5 Phase A begins. |
| AMB-02 | OTM named as third nav pillar but has no implementation, interface, or tests. | Formally defer to post-TASL. Note in NAV-06 as deferred capability. |
| AMB-03 | PLN-02 retask command arrival path during flight not specified. Pre-loaded is the stated default. | Default = pre-loaded offline envelope. RF link is a future enhancement. No code change required. |
| AMB-04 | FI-06 through FI-13 not documented in fault_manager.py header. | Document FI-06..FI-13 in fault_manager.py header before SB-5 Phase B begins. |
| AMB-05 | NavIC RS restoration condition undefined. | Define: NavIC available when ≥ 4 healthy satellites. Reintegrate after 30 consecutive readings with HDOP < 2.0. |
| AMB-06 | SAT-01 (BCMP-1 criterion) has no SRS test ID. FR-108 (satellite avoidance) appears in the BCMP-1 criteria and FR table but has no unit test, integration test, or KPI with a defined measurement method and pass criterion. | Before Sprint S3: write FR-108 test specification — TLE data source, injected satellite ground track, definition of correct masking manoeuvre, timing tolerance. Assign UT-SAT-01 and IT-SAT-01. |

# 16. Recovery Ownership Matrix

This section is new in v1.2 (correction O-01). It provides the single authoritative mapping of every critical mission event to the four ownership roles: Detects, Decides, Executes, Logs/Consumes. This matrix governs implementation. Any module handling an event not designated as its owner constitutes an ownership violation and must be corrected.

| Event | Detects | Decides (owns recovery) | Executes Recovery | Logs / Consumes Only |
| --- | --- | --- | --- | --- |
| GNSS denial | Navigation Manager (BIM) | Navigation Manager | Navigation Manager (ESKF mode switch to TRN_PRIMARY) | Mission Manager (phase update), Demo Tool (overlay) |
| VIO outage | Navigation Manager (VIOMode) | Navigation Manager | Navigation Manager (weight rebalance, mode demotion) | Mission Manager (USV consumer) |
| GNSS spoof detected | Navigation Manager (BIM) | Navigation Manager | Navigation Manager (GNSS gate, TRN-primary transition) | Mission Manager (trust state consumer), Demo Tool |
| EW spike — new threat | EW Manager | EW Manager (map update); Route Planner (replan trigger) | Route Planner (REPLAN or ROUTE_DEADEND) | Mission Manager (route update consumer), Demo Tool |
| Route dead-end | Route Planner | Mission Manager | Mission Manager (SHM entry, ESCALATE_OP) | Navigation Manager (hold position), PX4 Bridge |
| Retask validation fail | Mission Manager (VALIDATING state) | Mission Manager | Mission Manager (REJECTED → NONE, current mission continues) | Navigation Manager, Route Planner (no action) |
| PX4 reboot | PX4 Bridge (HEARTBEAT seq reset) | PX4 Bridge (D7–D9); Mission Manager (D8a gate) | PX4 Bridge (reconnect + OFFBOARD re-entry) | Mission Manager (SHM during reboot, D8a evaluation), Navigation Manager |
| OFFBOARD Loss | PX4 Bridge | PX4 Bridge (D1–D6 retry sequence) | PX4 Bridge (reconnect, OFFBOARD re-entry) | Mission Manager (SHM if > 5 s) |
| SHM entry | Mission Manager (trigger detection) | Mission Manager | Mission Manager (loiter command to PX4) | All modules (log only) |
| SHM exit | Mission Manager (clearance event) | Mission Manager | Mission Manager (resume phase) | PX4 Bridge (resume setpoints after D8a approval) |
| Checkpoint restore | Watchdog (process failure trigger) | Mission Manager (D8a evaluation gate) | PX4 Bridge (waypoint queue reload) | Navigation Manager (ESKF state restore), EW Manager (map restore) |
| Mission abort | Mission Manager | Mission Manager | Mission Manager (ABORT_MISS) | All modules (log and halt) |
| Terminal phase failure (DMRL/L10s) | DMRL / L10s-SE | Mission Manager (gate decision owns abort/proceed) | Mission Manager (ABORT_TERM or SHM) | Navigation Manager, PX4 Bridge (hold setpoint) |

ESKF core computation loop (NOT RESTARTABLE): If the ESKF core crashes, Mission Manager does not have a decision to make. The Watchdog triggers ABORT_MISS immediately via the NOT_RESTARTABLE classification. No module may attempt to recover from this failure.

# 17. SB-5 Entry Criteria

NORMATIVE: All items in this checklist must be in PASS state, verified on micromind-node01 with system Python 3.12.3 and PX4 SITL, before the SB-5 sprint begins. No item may be waived without TD approval and a documented rationale in BCMP2_JOURNAL.md with an IST timestamp.

| # | Capability | Pass Criterion | Current State | SB-5 Phase |
| --- | --- | --- | --- | --- |
| EC-01 | OFFBOARD continuity over 30 min | IT-PX4-01 passes: continuity ≥ 99.5%, ≤ 1 OFFBOARD Loss event, measured on micromind-node01 PX4 SITL. Stale setpoints discarded on recovery (PX4-01 v1.2). | IT-PX4-01 defined. Not yet tagged as regression gate. | Phase A |
| EC-02 | Checkpoint write and restore with v1.2 schema | UT-PX4-05 passes: Checkpoint written ≤ 200 ms, restored from SIGKILL within 10 s, position error < 20 m. Checkpoint includes shm_active, pending_operator_clearance_required, mission_abort_flag, eta_to_destination_ms, terrain_corridor_phase, route_corridor_half_width_m (all new v1.2 fields). Rolling purge verified: ≤ 5 checkpoints retained. | PX4-05 updated (v1.2). UT-PX4-05 not yet implemented. | Phase A |
| EC-03 | PX4 reboot recovery with Mission Manager gate | IT-PX4-02 passes: reboot detected ≤ 3 s, OFFBOARD restored ≤ 15 s, Mission Manager D8a evaluation gate executes before autonomous resume. pending_operator_clearance_required=False path: autonomous resume verified. pending_operator_clearance_required=True path: SHM maintained, AWAITING_OPERATOR_CLEARANCE logged. | IT-PX4-02 defined. D8a gate not yet implemented. | Phase A |
| EC-04 | Dynamic retask with all v1.2 corrections | UT-PLN-02 passes (CRUISE accept, TERMINAL reject, INS_ONLY constraint check). IT-PLN-01 passes (retask under GNSS_DENIED). IT-PLN-02 passes (rollback including EW map and terrain corridor restore). Terrain regeneration order verified (R-01). EW staleness check verified (R-02). 15 s timeout enforced (R-06). | PLN-02 updated (v1.2). Tests defined. Not yet implemented. | Phase B |
| EC-05 | 2-hour endurance: no crash, no OOM, log ≥ 99%, checkpoint purge active | ST-END-01 at least 1 run: zero crashes, zero OOM kills, log_completeness ≥ 99%, rss_growth < 50 MB, log rolling active (30 min or 200 MB), checkpoints_retained_count ≤ 5 at all times, CHECKPOINT_PURGED events present. | ST-END-01 defined. Log rolling and checkpoint purge not yet implemented. | Phase D |
| EC-06 | Long-range demo: log-driven event overlays, authoritative clock | VIZ-02 implementation: event markers driven from KPI JSON (not hardcoded). Mission clock rule verified: phase boundaries produce aligned Vehicle A and Vehicle B records at identical sim_timestamp_ms. Vehicle B interpolated at 100 ms display cadence. Reproducibility: same JSON produces byte-identical MP4. | VIZ-02 not yet implemented. | Phase C |
| EC-07 | Recovery ownership table implemented in code | §16 ownership table committed to BCMP2_JOURNAL.md and verified against live code. No module handles an event not designated as its owner. Watchdog restartability_class assigned per RS-03 v1.2. | Table defined in §16. Not yet verified against code. | Phase A |
| EC-08 | Queue latency instrumented and UT-MM-04 passes | Internal event bus queue latency measured and logged. UT-MM-04 passes: critical event delivery ≤ 100 ms under ST-CPU-01 load. | MM-04 defined. No instrumentation yet. | Phase B |
| EC-09 | TRN/VIO fallback stability over 90 km GNSS-denied | IT-NAV-01 passes with canonical seed 42: drift within C-2 envelopes at km 60/100/120. No CORRIDOR_BREACH before km 76.9. | IT-NAV-01 passes as part of BCMP-2 AT-2 (SB-3 ✅). Carry forward. | PASS — carry forward |
| EC-10 | Watchdog restarts non-critical process without ABORT | ST-RESTART-01 passes: SIGKILL to Navigation Manager, watchdog detects ≤ 500 ms, nav restored ≤ 10 s, mission continues. Restartability classification verified per RS-03 v1.2: ESKF core SIGKILL triggers ABORT_MISS immediately. | ST-RESTART-01 defined. Restartability classification not yet implemented. | Phase D |
| EC-11 | BCMP-2 seeds 42/101/303 produce correct comparative outcome | run_bcmp2_tests.py passes for all three seeds: Vehicle A exceeds corridor at P4, Vehicle B maintains corridor through P5 (AT-6 three-seed repeatability from BCMP-2 architecture). | AT-2 passes ✅ (SB-3). AT-6 three-seed repeatability is SB-5 closure deliverable. | PASS (AT-2). AT-6 is SB-5 closure. |
| EC-12 | Static Mission Report for all 3 seeds | VIZ-01 passes for seeds 42/101/303: business verdict block first, all charts render, file < 5 MB, no external dependencies. | VIZ-01 passes ✅ (SB-3). Carry forward. | PASS — carry forward |
| EC-13 | Orthophoto Matching SIL-tested for AVP-02 profile | UT-NAV-02-A and UT-NAV-02-B rewritten for OM mechanism and passing. OrthophotoMatchingStub implemented. At least 1 successful OM correction per 5 km over textured terrain (sigma_terrain >= 30 m) confirmed in 20-run Monte Carlo. KPI-N04 passes. | Zero OM SIL tests exist. Stub not yet implemented (OI-05). | Sprint C (before SB-5 Closure) |

## 17.1 SB-5 Sprint Sequencing (Normative)

The following phase sequencing prevents the TASL-facing demonstration being built on an unstable foundation. Phase C (VIZ-02) shall not begin until Phase A and Phase B are confirmed complete. Phase D may run concurrently with Phase C.

| Phase | Scope | Exit Gate | Risk if Skipped |
| --- | --- | --- | --- |
| Phase A (Days 1–5) | EC-01 (OFFBOARD 30-min), EC-02 (Checkpoint v1.2 schema), EC-03 (PX4 reboot + D8a gate), EC-07 (ownership table in code). Apply corrections: R-01 terrain ordering, P-01 SHM in Checkpoint, P-02 Mission Manager gate, P-03 stale setpoint discard, P-04 ETA in Checkpoint. | IT-PX4-01 ✅, IT-PX4-02 ✅, UT-PX4-05 ✅, §16 ownership verified in code. | TASL demo crashes on first PX4 reconnect. Retask leaves vehicle on stale route. Safety hold bypassed after reboot. |
| Phase B (Days 6–10) | EC-04 (retask with all R-01..R-06 corrections), EC-08 (queue latency). Apply: R-02 EW staleness, R-03 rollback completeness, R-04 waypoint upload order, R-05 INS_ONLY constraint, R-06 15 s timeout. | UT-PLN-02 ✅, IT-PLN-01 ✅, IT-PLN-02 ✅, UT-MM-04 ✅. | Retask demo shows vehicle taking wrong route. Vehicle accepts retask while corridor margin is insufficient. |
| Phase C (Days 11–16) | EC-06 (VIZ-02: authoritative clock, log-driven overlays, 100 ms interpolation cadence, reproducibility). Apply: V-01 clock rule, V-02 overlay rule, V-03 sync rule, V-04 reproducibility. Render seeds 42/101/303. | MP4 rendered for seed 42 ✅. Log-driven overlays verified (no hardcoding) ✅. UT-VIZ-02 ✅. | Animation shows GNSS denial at wrong km. Events hardcoded. Seeds 101/303 show wrong overlay positions. |
| Phase D (Days 17–21) | EC-05 (2-hour endurance with log rolling + checkpoint purge), EC-10 (watchdog + restartability). Apply: E-01 checkpoint purge, E-02 route fragment cleanup, E-03 restartability classification, E-04 log rolling. | ST-END-01 ✅ (1 run). ST-RESTART-01 ✅. CHECKPOINT_PURGED events present. | Live demo runs 20 min then OOM or disk full. Watchdog restarts ESKF core instead of triggering ABORT_MISS. |
| SB-5 Closure | AT-6 three-seed repeatability. BCMP-2 Closure Report. SRS v1.2 confirmed as final baseline. BCMP2_JOURNAL.md updated with all SB-5 IST timestamps. | run_bcmp2_tests.py AT-6 ✅ (seeds 42/101/303). SRS v1.2 confirmed baselined. | No formal programme evidence for TASL review. |

# 18. Glossary

- AVP – Air Vehicle Profile; reference UAV class used for testing

- ROS – Reference Operational Scenario; reusable scenario template for testing

- Boundary Condition – Maximum validated operational limit for a parameter

- Stress Condition – Elevated but realistic operating condition used during validation

- Nominal Condition – Expected operational condition for standard testing

# Appendix A — Recovery Behaviour Matrix

This matrix maps every recovery class (§1.3) to the specific modules that use it and the escalation path. Updated in v1.2 to include the restartability classification from RS-03.

| Recovery Class | Used By | Trigger | Max Duration | Escalation |
| --- | --- | --- | --- | --- |
| CONT_NOM | All | No fault detected | Indefinite | — |
| CONT_DEG | Nav Mgr, EW Mgr, PX4 Bridge | Minor sensor degradation | Indefinite until trigger clears | RETRY or SHM if threshold exceeded |
| RETRY | Nav Mgr (TRN), PX4 Bridge (OFFBOARD) | Operation failure | N × interval (per req) | See per-requirement escalation |
| RETRY_ALT | Nav Mgr (GNSS spoof, RADALT dropout) | Primary source failed | N × interval | CONT_DEG or SHM |
| HOLD_PATH | Route Planner, Mission Manager (INS_ONLY retask rejected) | Route uncertain or retask nav margin insufficient | 10 s max | SHM after 10 s |
| REPLAN | Route Planner | Route invalidated | 1 attempt per level (0→3) × plan_timeout | ROUTE_DEADEND → SHM |
| SHM | Mission Manager | 8 defined triggers (MM-03). Also: PX4 reboot until D8a clears. | Until cleared. ESCALATE_OP at 120 s. ABORT_MISS at 240 s | ESCALATE_OP |
| ABORT_TERM | DMRL, L10s-SE | Terminal fault only | Immediate | DMRL re-acquire or SHM |
| ABORT_MISS | Mission Manager, Watchdog (NOT_RESTARTABLE) | Unrecoverable fault or ESKF core crash | Immediate, terminal | None |
| ESCALATE_OP | Mission Manager | SHM active > 120 s | 120 s await retask | ABORT_MISS |
| PROC_RESTART | Watchdog (RESTARTABLE processes only) | Process heartbeat missed, restartability_class ≠ NOT_RESTARTABLE | Restart ≤ 2 s + restore ≤ 10 s | ABORT_MISS after 30 s |
| CHECKPOINT_RESTORE | Watchdog, PX4 Bridge (D8) | Process restart or PX4 reboot | Load + restore ≤ 10 s. Mission Manager D8a gate before autonomous resume. | PROC_RESTART if corrupt |

# Appendix B — Dynamic Retask State Machine

Updated in v1.2 to correct R-03 (rollback restores EW map, terrain corridor, and ETA) and R-04 (waypoint upload sequencing — PX4 waypoint queue only uploaded after validation).

| State | Entry Condition | Actions | Exit Conditions |
| --- | --- | --- | --- |
| NONE | System start or retask complete/rejected | No retask in progress | RETASK_RECEIVED event |
| VALIDATING | RETASK_RECEIVED | 1. Verify signature hash. 2. Verify timestamp_utc not stale (< 60 s). 3. Check permitted_phases includes current phase. 4. If GNSS_DENIED + INS_ONLY: check cross_track_error_m ≤ (route_corridor_half_width_m − 100); reject with RETASK_NAV_CONFIDENCE_TOO_LOW if not met. | VALIDATION_PASS → ROUTING. VALIDATION_FAIL → NONE (log RETASK_REJECTED with reason). |
| ROUTING | VALIDATION_PASS | 1. Regenerate terrain corridor from new target position using C-3 DEM parameterisation for new target phase. Log RETASK_TERRAIN_GEN_FAILED and go to ROLLBACK if fails. 2. Check EW map staleness — wait up to 2 s for fresh map; if stale, set retask_ew_stale=True, log RETASK_EW_MAP_STALE. 3. Invoke Route Planner with updated terrain and EW map. 4. Timeout = 10 s (nominal); extend to 15 s total if constraint level 1 retry triggered. | ROUTE_SUCCESS → COMMITTING. ROUTE_FAIL (all constraint levels) → ROLLBACK. TERRAIN_GEN_FAIL → ROLLBACK. |
| COMMITTING | ROUTE_SUCCESS | 1. Validate new route against new mission envelope (geographic, class, time window). 2. Only on validation success (new_envelope_valid=True AND route_validated=True): refresh PX4 waypoint queue. 3. Update active_nav_mode if needed. Note: PX4 waypoints are NOT uploaded before step 2 is complete. | COMMIT_SUCCESS → ACTIVE. COMMIT_FAIL → ROLLBACK. |
| ACTIVE | COMMIT_SUCCESS | Mission executing new route with new envelope. | Mission complete or new RETASK_RECEIVED → NONE. FAULT → ROLLBACK. Release partial route candidates from constraint levels 1–3 within 500 ms of ACTIVE entry (E-02 cleanup). |
| ROLLBACK | ROUTE_FAIL or TERRAIN_GEN_FAIL or COMMIT_FAIL or ACTIVE FAULT | 1. Restore previous route. 2. Restore previous mission envelope. 3. Restore previous EW map from Checkpoint ew_map_snapshot_path. 4. Restore previous terrain corridor (terrain_corridor_phase from Checkpoint). 5. Restore previous ETA. 6. Log RETASK_ROLLBACK: {previous_target, restored_ew_map_age_ms, restored_terrain_phase, reason}. Rollback shall complete within 2000 ms. | Rollback complete → NONE. |
| REJECTED | VALIDATION_FAIL | Log RETASK_REJECTED with reason. | Immediate → NONE. |

| Phase at Retask | Retask Permitted | Routing Mode | Notes |
| --- | --- | --- | --- |
| INIT | No | — | System not ready |
| PRE_GNSS_DENIAL | Yes | GNSS-aided | Full accuracy route planning |
| CRUISE | Yes | GNSS-aided | Full accuracy route planning |
| GNSS_DENIED (VIO/TRN active) | Yes | TRN/VIO-aided | Route uses nav uncertainty margin |
| GNSS_DENIED (INS_ONLY) | Yes, with constraint | INS-only — large position uncertainty | Permitted only if cross_track_error_m ≤ (route_corridor_half_width_m − 100). Otherwise: RETASK_NAV_CONFIDENCE_TOO_LOW, HOLD_PATH. |
| TERMINAL_APPROACH | No | — | Prohibited: phase lock active |
| COMMITTED | No | — | Prohibited: point of no return |
| SHM (trigger=ROUTE_DEADEND) | Yes | Any available nav source | Retask is primary SHM exit path |
| ABORT | No | — | Terminal state |

# Appendix C — Mission Demonstration Tool Capability Roadmap

Updated in v1.2 to reflect the revised §9 architecture: Layer 1 (Static Mission Report) is delivered. Layer 2 (Long-Range Mission Demonstration) is dependency-gated on §17 entry criteria. Layer 3 (Tactical Live Demonstration) is a long-term roadmap item. The SB-5 column in v1.1 has been replaced with a dependency-gated column.

| Capability | Layer 1: Static Report (SB-4 Complete) | Layer 2: Long-Range Demo (Post §17 PASS) | Layer 3: Tactical Live Demo (Post-HIL Roadmap) |
| --- | --- | --- | --- |
| Output format | Self-contained HTML + PNG. Air-gap safe. | Interactive log-driven playback. MP4 + GIF export. Reproducible from JSON. | Real-time Gazebo SITL/HITL feed with live telemetry overlay. |
| Vehicle markers | Static route lines on chart. | Moving markers (A and B simultaneously). Vehicle B interpolated at 100 ms cadence. | Real vehicle icons with live heading indicators. |
| GNSS denial event | Dot annotation on drift chart. | Explicit visual transition moment driven from gnss_available=False log record. | Pulsing event overlay driven from live telemetry. |
| Corridor breach | Drift chart post-mission. | Visible boundary crossing in animation. Vehicle A breach marker on route map. | Live breach alert overlay with countdown. |
| TRN corrections | None. | Correction markers at each trn_correction_applied=True log record on Vehicle B path. | Live TRN confidence heatmap. |
| EW threat overlay | None. | EW threat envelopes on route map driven from ew_threats[] log records. | Live EW environment updating in real time. |
| Retask event | None. | Visible route change animation driven from retask_event log records. | Live operator command injection with immediate Vehicle B response. |
| Terrain underlay | Partial (best-effort C-3 synthetic). | DEM-style terrain underlay. Srinagar Valley-inspired profile for TASL demos. 12 contour levels. | Real DEM tile for HIL corridor. |
| Demo modes | None. | Executive (2–3 min, P2 start), Engineering (full scrub), Side-by-side (time-locked dual window). | Operator command injection mode. Full status panel display. |
| Event overlays | None. | All overlays driven from run JSON. No hardcoded positions. | All overlays driven from live telemetry stream. |
| Pause/resume/scrub | None. | Required. Timeline bar is master cursor. All panels hold at cursor. Panel update ≤ 50 ms. | Full operator control with timeline scrub. |
| Reproducibility | N/A. | Byte-identical output from same JSON + render params. Re-rendering does not require re-simulation. | N/A — live mode. |
| Dependency | Delivered ✅ | §17 EC-01 through EC-11 all PASS first. | Stable HIL integration + VIZ-02 fully implemented. |

# Appendix D — PX4 Recovery Sequence

Updated in v1.2: step D8a added (Mission Manager evaluation gate before autonomous resume, correction P-02). Steps D1–D7 and D9 unchanged.

| Step | Trigger | Action | Timeout | Next on Success | Next on Fail |
| --- | --- | --- | --- | --- | --- |
| D1 | OFFBOARD Loss (any cause) | 1. Log OFFBOARD_LOSS with gap_ms. 2. Buffer last setpoint (to prevent OFFBOARD exit only — buffer is discarded, not replayed, on recovery). | 500 ms | D2 | D2 (continue) |
| D2 | D1 complete | Attempt SET_MODE OFFBOARD via MAVLink. Retry at 1 Hz. | 5 s total (5 retries) | D5 (success) | D3 |
| D3 | D2 timeout (5 s) | Activate SHM. Continue reconnect at 1 Hz. | Additional 5 s | D4 | D6 |
| D4 | D3 reconnect success | OFFBOARD restored. Log OFFBOARD_RESTORED. Discard stale setpoint buffer. Exit SHM. | — | D5 | — |
| D5 | OFFBOARD active | Issue fresh setpoint from current Navigation Manager state. Restore setpoint stream at ≥ 20 Hz. Log OFFBOARD_RESTORED. | — | Normal operation | — |
| D6 | D3 total timeout (10 s) | Log OFFBOARD_UNRECOVERED. ABORT_MISS. | — | — | Mission ends |
| D7 | PX4 reboot detected (seq reset) | Log PX4_REBOOT_DETECTED. Enter SHM. Begin reconnect (D1 path). | — | D8 | D6 |
| D8 | PX4 reconnect success | Load Checkpoint (full v1.2 schema including shm_active, pending_operator_clearance_required, mission_abort_flag, eta_to_destination_ms). Restore nav state. Command OFFBOARD re-entry. Proceed to D8a. | 15 s total | D8a | D6 |
| D8a | D8 complete (NEW in v1.2) | Mission Manager evaluates restored Checkpoint: (1) if mission_abort_flag=True → ABORT_MISS immediately; (2) if shm_active=True OR pending_operator_clearance_required=True → remain in SHM, command PX4 to loiter, log AWAITING_OPERATOR_CLEARANCE; (3) if all clear (both False, no abort) → autonomous_resume_approved=True, log AUTONOMOUS_RESUME_APPROVED, proceed to D9. | — | D9 (if autonomous_resume_approved) or SHM hold (if clearance needed) | D6 (if mission_abort_flag=True) |
| D9 | autonomous_resume_approved=True | Restore setpoint stream from current Navigation Manager state. Command new waypoint queue. Log MISSION_RESUMED. | — | Normal operation | — |
| D10 | PX4 returns in HOLD (post-reboot) | Command SET_MODE OFFBOARD within 1 s of reboot detection. Retry 3× at 2 s. | 6 s | D8a (after OFFBOARD confirmed) | D6 |

# Appendix E — Logging and Event Priority Table

| Event | Level | Dropped Allowed? | RAM Buffer | Disk Flush | Module |
| --- | --- | --- | --- | --- | --- |
| SHM_ACTIVATED | CRITICAL | Never | Always (ring 1000). Also critical_events.log. | Immediate | Mission Manager |
| SHM_DEACTIVATED | CRITICAL | Never | Always | Immediate | Mission Manager |
| SPOOF_DETECTED | CRITICAL | Never | Always | Immediate | Navigation Manager (BIM) |
| GNSS_GATED | CRITICAL | Never | Always | Immediate | Navigation Manager (BIM) |
| DMRL_ABORT | CRITICAL | Never | Always | Immediate | Terminal Guidance (DMRL) |
| ENVELOPE_VIOLATION | CRITICAL | Never | Always | Immediate | Mission Manager |
| OFFBOARD_LOSS | CRITICAL | Never | Always | Immediate | PX4 Bridge |
| CORRIDOR_BREACH | CRITICAL | Never | Always | Immediate | Navigation Manager |
| PROCESS_FAILURE | CRITICAL | Never | Always | Immediate | Watchdog |
| ESKF_CORE_FAILURE | CRITICAL | Never | Always | Immediate | Watchdog (NOT_RESTARTABLE path) |
| L10S_ABORT | CRITICAL | Never | Always | Immediate | L10s-SE |
| RETASK_ROLLBACK | WARNING | On disk full only | Yes | On flush | Mission Manager |
| ROUTE_DEADEND | WARNING | On disk full only | Yes | On flush | Route Planner |
| TRN_SUPPRESSED | WARNING | On disk full only | Yes | On flush | Navigation Manager |
| VIO_DEGRADED | WARNING | On disk full only | Yes | On flush | Navigation Manager |
| CPU_HIGH | WARNING | On disk full only | Yes | On flush | Runtime Monitor |
| EW_MAP_STALE | WARNING | On disk full only | Yes | On flush | EW Manager |
| ROUTE_PLANNER_MEMORY_HIGH | WARNING | On disk full only | Yes | On flush | Route Planner |
| CHECKPOINT_PURGED | INFO | On buffer > 80% | Circular 10k | Batch (rolled every 30 min/200 MB) | PX4 Bridge |
| FSM_TRANSITION | INFO | On buffer > 80% | Circular 10k | Batch | Mission Manager |
| CHECKPOINT_SAVED | INFO | On buffer > 80% | Circular 10k | Batch | PX4 Bridge |
| NAV_MODE_TRANSITION | INFO | On buffer > 80% | Circular 10k | Batch | Navigation Manager |
| ROUTE_PLANNED | INFO | On buffer > 80% | Circular 10k | Batch | Route Planner |
| RETASK_COMPLETE | INFO | On buffer > 80% | Circular 10k | Batch | Mission Manager |
| AWAITING_OPERATOR_CLEARANCE | INFO | On buffer > 80% | Circular 10k | Batch | Mission Manager |
| AUTONOMOUS_RESUME_APPROVED | INFO | On buffer > 80% | Circular 10k | Batch | Mission Manager |

# Appendix F — Test Exit Criteria Standard

All tests in this SRS must conform to the following standard. Tests that end ambiguously are non-compliant. Unchanged from v1.1.

| Field | Required? | Definition |
| --- | --- | --- |
| Exit Criteria | Mandatory | Exact measurable condition that terminates the test regardless of outcome. |
| Pass Condition | Mandatory | Quantitative threshold(s) that must all be true for a PASS verdict. |
| Fail Condition | Mandatory | Quantitative threshold(s) that produce immediate FAIL if any are true. |
| Retry Policy | Mandatory | Number of retries permitted. Timing. Escalation on Nth failure. |
| Recovery Behaviour | Mandatory | Typed recovery class from §1.3 expected if stimulus produces degradation. |
| Required Logs | Mandatory | Named log events that must be present. Test is invalid if required logs are absent. |
| Data Retention | Mandatory | What must be preserved when a test fails: run JSON, logs, core dumps, state snapshots. |
| Repeatability Count | Mandatory | Number of identical runs required. Critical tests: ≥ 5. High: ≥ 3. Medium/Low: ≥ 1. |
| Post-Test State | Mandatory | Required final system state after the test: clean, SHM active, fault cleared, etc. |

# Appendix G — Requirement Completeness Audit

Updated in v1.2. Requirements revised in this version are marked with (v1.2). New requirements (§16, §17) are shown with their status.

| Req ID | Description | Unit Test | Integration Test | Stress Test | Regression | Logging | Recovery | Complete? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAV-01 | INS Drift | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| NAV-02 | TRN Accuracy | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| NAV-03 | VIO Drift | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| NAV-04 | Nav Transition | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| NAV-05 | BIM / Spoof | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| NAV-06 | Two-Theatre (σ defined) | ✅ (partial) | ✅ | ✅ | ✅ | ✅ | ✅ | ⚠ σ auto-detect AMB-01 |
| EW-01 | EW Map Update | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| EW-02 | EW Replan | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| EW-03 | Spoof Rejection | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| PLN-01 | Route Planner | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| PLN-02 (v1.2) | Dynamic Retask — full corrections | ✅ | ✅ | ⚠ no dedicated | ⚠ SB-5 Phase B | ✅ | ✅ | ⚠ Regression + code gap |
| PLN-03 | Dead-End Recovery | ✅ | ✅ | ⚠ no dedicated | ⚠ SB-5 | ✅ | ✅ | ⚠ Regression gap |
| MM-01 | FSM Latency | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| MM-02 | Envelope Validation | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| MM-03 | SHM | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| MM-04 | Queue Latency | ⚠ SB-5 | ⚠ indirect | ✅ | ⚠ SB-5 | ✅ | ✅ | ⚠ Unit test + instrumentation gap |
| PX4-01 (v1.2) | OFFBOARD Rate + stale discard | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| PX4-02 | Heartbeat | ⚠ stub | ✅ | ✅ | ⚠ SB-5 | ✅ | ✅ | ⚠ Unit test gap |
| PX4-03 | Sensor Driver | ⚠ stubs | ✅ | ✅ | ⚠ HIL | ✅ | ✅ | ⚠ HIL required |
| PX4-04 (v1.2) | Reboot + D8a gate | ⚠ SB-5 Phase A | ✅ | ✅ | ⚠ SB-5 Phase A | ✅ | ✅ | ⚠ D8a not yet implemented |
| PX4-05 (v1.2) | Checkpoint + 6 new fields + purge | ⚠ SB-5 Phase A | ✅ | ✅ | ⚠ SB-5 Phase A | ✅ | ✅ | ⚠ New schema fields not yet implemented |
| TERM-01 | DMRL Lock | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| TERM-02 | Decoy Rejection | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| TERM-03 | L10s-SE | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ YES |
| RS-01 | CPU Ceiling | ⚠ Jetson | ✅ | ✅ | ⚠ HIL | ✅ | ✅ | ⚠ HIL required |
| RS-02 (v1.2) | Log Completeness + rolling | ⚠ SB-5 Phase D | ✅ indirect | ✅ | ⚠ SB-5 Phase D | ✅ | ✅ | ⚠ Rolling policy not yet implemented |
| RS-03 (v1.2) | Process Restart + restartability | ⚠ SB-5 Phase D | ⚠ indirect | ✅ | ⚠ SB-5 Phase D | ✅ | ✅ | ⚠ Classification not yet in code |
| RS-04 (v1.2) | Memory + route fragment cleanup | ⚠ SIL | ✅ | ✅ | ⚠ CI gap | ✅ | ✅ | ⚠ Cleanup not yet in code |
| VIZ-01 | Static Mission Report | ✅ | ✅ | — | ✅ | ✅ | ✅ | ✅ YES |
| VIZ-02 (v1.2 re-scope) | Long-Range Demo — dependency-gated | ⚠ After §17 | ⚠ After §17 | — | ⚠ After §17 | ✅ | ✅ | ⚠ Prerequisites required |
| VIZ-03 (v1.2 roadmap) | Tactical Live Demo — roadmap | N/A | N/A | N/A | N/A | ✅ | ✅ | Roadmap — post-HIL |
| §16 (new) | Recovery Ownership Matrix | N/A — governance | N/A | N/A | ⚠ Verify against code SB-5 Phase A | N/A | N/A | ⚠ Not yet verified in code |
| §17 (new) | SB-5 Entry Criteria | N/A — governance | N/A | N/A | N/A | N/A | N/A | Document complete. Execution pending. |

End of SRS-MicroMind-v1.2.