MicroMind / NanoCorteX

Sprint Handoff: S9 → S10

5 March 2026  |  Outgoing: S9 — NAV-01 Closure  |  Incoming: S10 — Simulation Infrastructure + Demo Artefact

Repo: amitkr-anand/micromind-autonomy  |  Commit: 7fba53c  |  Tag: s9-nav01-pass

S10 STATUS: SCOPE ASSESSED — NOT STARTED.
Confirm session goal with Amit before writing any code.

# 1  Outgoing Sprint: S9 Results

Sprint S9 closed NAV-01 with a 6.9× margin at 150 km, diagnosing and fixing five root causes in the TRN+ESKF integration that had been present but hidden since S3.

| Corridor | max_5km_drift | final_drift | NAV-01 | Margin |
| --- | --- | --- | --- | --- |
| 20 km | ~4 m | ~4 m | ✓ PASS | 25× |
| 50 km | 9.7 m | 3.7 m | ✓ PASS | 10× |
| 150 km | 14.5 m | 4.2 m | ✓ PASS | 6.9× |

## 1.1  Changes Delivered

| File | Change |
| --- | --- |
| core/ins/trn_stub.py | S9-1/S9-2: Internal Kalman removed. update() returns raw NCC offset only — no state mutation. |
| core/ekf/error_state_ekf.py | S9-4: Q-matrix corrected to STIM300 TS1524 rev.31. Position process noise added (_POS_DRIFT_PSD = 1.0 m/√s). |
| sim/als250_nav_sim.py | S9-3: Correct propagation order. corridor_km parameter. NAV-01 metric → 2D horizontal norm. |
| scenarios/bcmp1/bcmp1_runner.py | S9-0: S5 dispatcher routing fix. 111/111 regression restored. |

## 1.2  Regression at S9 Close

python run_s5_tests.py → 111/111 PASS

python tests/test_s6_zpi_cems.py → 36/36 PASS

python run_s8_tests.py → 68/68 PASS

Total: 215/215

# 2  Root Causes Fixed in S9

Retained permanently — these were non-trivial architectural errors. Full understanding is essential before modifying TRN or ESKF in any future sprint.

| RC | Root Cause | Fix |
| --- | --- | --- |
| RC-1 | TRN internal Kalman shadowed the ESKF — two filters fighting with inconsistent noise parameters | Removed internal Kalman from TRNStub entirely. update() now returns raw TRNCorrection only. Caller passes it to ESKF. |
| RC-2 | ESKF gyro bias RW 247× too large: _GYRO_BIAS_RW = 1e-5 vs correct 4.04e-8 rad/s/√s | Corrected to STIM300 TS1524 rev.31 values. Inflated Q caused over-weighting and prevented convergence. |
| RC-3 | Position block of Q was zero → K ≈ 4e-8 → TRN fired 33 corrections over 50 km but applied nothing | Added _POS_DRIFT_PSD = 1.0 m/√s → P_pos ≈ 27 m² → K ≈ 0.5 (healthy). |
| RC-4 | Propagation order violated — ins_propagate() called before eskf.propagate(), ESKF received stale state | Fixed to: ESKF propagate → INS propagate → TRN update → ESKF update/inject. |
| RC-5 | NAV-01 metric was 3D including altitude (~3.4 m/km uncorrected) | Changed to 2D horizontal norm. TRN is a horizontal navigation aid. Altitude is a separate baro channel. |

# 3  Live Interfaces Changed in S9

BREAKING CHANGE: TRNStub.update() signature changed. Any caller using the old INSState-based API will silently fail — no exception, wrong results.

## 3.1  TRNStub.update() — New Signature

Old (S3–S8, DEPRECATED): def update(self, ins: INSState, true_north_m, true_east_m, dt, ground_track_m, timestamp_s) -> TRNCorrection | None

New (S9+, CANONICAL — no side effects): def update(self, ins_north_m: float, ins_east_m: float, true_north_m: float, true_east_m: float, ground_track_m: float, timestamp_s: float = 0.0) -> TRNCorrection | None

## 3.2  ESKF Q Constants (S9 Canonical — Do Not Change Without Rationale)

_ACC_BIAS_RW  = 9.81e-7   # m/s²/√s — STIM300 TS1524 rev.31

_GYRO_BIAS_RW = 4.04e-8   # rad/s/√s — STIM300 TS1524 rev.31

_POS_DRIFT_PSD = 1.0       # m/√s — position process noise (S9 addendum)

# 4  TRN Manoeuvring Corridor Validation (Pre-S10 Sandbox)

TRN performance was validated on both linear corridors and manoeuvring mountain flight paths with heading changes up to 60°. The Hybrid gate configuration maintained NAV-01 compliance in both scenarios.

Prior to S10, a standalone TRN sandbox was executed to answer the forcing question: does the nav solution hold NAV-01 on a manoeuvring UAV flight over real Himalayan terrain? This question was unanswered by any prior sprint — all regression corridors were straight. The sandbox produced a defensible, evidence-backed answer before the TASL partnership meeting.

## 4.1  TRN Operational Envelope (Sandbox Findings)

The following gate configuration was validated across both a straight 161 km ALS-250 corridor and a 150 km five-segment Kashmir–Zanskar manoeuvring corridor over real SRTM terrain. These values are frozen and match the Hybrid-3 configuration from S9.

| Parameter | Value | Notes |
| --- | --- | --- |
| σ_terrain threshold | 10 m | Below this, terrain is ambiguous — update suppressed |
| Innovation gate | 150 m (2D Euclidean) | Both N and E axes physically resolved via 2D patch NCC |
| Max consecutive suppressed | 3 | Operative for ALS-250 straight corridor |
| Search radius | 400 m | Unchanged from Hybrid-3. Valid at heading changes up to 60°. |
| NCC patch size | 600 m × 600 m | 2D terrain patch. Both axes resolved. Faster than 1D profile NCC. |
| TRN correction interval | Every 2 km | Per FR-107. Maintained throughout manoeuvring corridor. |

TRN maintains bounded navigation error when σ_terrain > 10 m across the majority of the ground track. On the Himalayan manoeuvring corridor, 75% of the 150 km route exceeded this threshold, resulting in 68 accepted corrections out of 75 update attempts (90% acceptance rate).

## 4.2  Manoeuvring Corridor Results

| Metric | 1D Profile NCC (v1/v2) | 2D Patch NCC (v3, final) | Change |
| --- | --- | --- | --- |
| NAV-01 Worst 5 km | 421.9 m | 33.3 m | −92% |
| RMS Error | ~280 m | 17 m | −94% |
| P95 Error | ~366 m | 29 m | −92% |
| Accepted updates | 5/75 (7%) | 68/75 (90%) | +13.6× |
| Innovation rejected | 64–69 | 0 | Eliminated |
| Max correction gap | 127–150 km | 13.9 km | −89% |
| NAV-01 gate (100 m) | FAIL | PASS — 3.0× margin |  |

## 4.3  Known Boundary Conditions

Two boundary conditions were identified during the sandbox and are documented here for programme record. Both are addressed by future enhancements.

a) Flat terrain segments where σ_terrain < 10 m.

The first 15 km of the Himalayan corridor is Kashmir valley floor (flat river plain). Terrain roughness falls below the 10 m sigma gate threshold, causing TRN updates to be suppressed correctly. A forced-update mechanism (MAX_CONSEC_SUPP = 3) introduced erroneous corrections on ambiguous flat terrain, triggering an irreversible drift cascade. For the manoeuvring corridor, forced updates were disabled. This boundary condition is addressed in Phase 2 by VIO (Visual-Inertial Odometry) fusion as an auxiliary navigation source on low-roughness terrain.

b) Ridge-parallel ambiguity when using 1D terrain profiles.

The prior 1D profile NCC architecture samples terrain along the current heading. When the flight track is parallel or near-parallel to a ridge line, the 1D profile cannot resolve cross-track position error — only along-track matching is possible. This caused the cross-track component of the innovation vector to consist entirely of quantisation noise, driving innovation magnitude beyond the 150 m gate and rejecting geometrically correct matches. This boundary condition is fully addressed by the 2D terrain patch NCC architecture (v3), which extracts a rectangular patch and resolves both N and E offsets independently from the correlation peak. The 2D NCC implementation is the S10-1 deliverable.

# 5  Incoming Sprint: S10 — Confirmed Scope

Governing constraint (Demo Fork, 28 Feb 2026): Deliver 100% of a bounded, defensible autonomy core. S10 is simulation infrastructure + artefact generation only. No new functional modules. No architectural changes.

## 5.1  Confirmed Deliverables

| ID | Deliverable | Description | Gate | Risk |
| --- | --- | --- | --- | --- |
| S10-1 | NCC Vectorisation | trn_stub_ncc_patch.py replaces pure-Python nested NCC loop with numpy sliding_window_view. Drop-in: identical inputs/outputs. | 68/68 S8 tests pass. 50 km STIM300 NAV-01 pass. | LOW |
| S10-2 | S9 Regression Test | tests/test_s9_nav01_pass.py — 7 automated gates covering all S9 architectural decisions. | 7/7 pass. Suite → 222/222. | LOW |
| S10-3 | S8-D Drift Chart | dashboard/als250_drift_chart.py — three-curve PNG + PDF (STIM300 / ADIS16505-3 / BASELINE over 250 km). Primary TASL quantitative artefact. Closes M5. | Chart PNG + PDF generated. STIM300 NAV-01 PASS at 250 km. | LOW |
| S10-4 | Parallel IMU Runner | run_als250_parallel.py — subprocess.Popen per IMU model. No MKL fork deadlock. ~3× wall time reduction. Supersedes broken run_als250_parallel_v2.py. | 50 km smoke: all 3 IMUs PASS simultaneously. | LOW |
| S10-5 | Terrain Observability Instrumentation | Add structured logging of σ_terrain at each update attempt, NCC innovation magnitude, suppression events (sigma-gate and innov-gate), and accepted/rejected outcome. Output to mission log. Enables offline envelope analysis and post-flight debugging. | Logged fields present in .npy output. No regression impact on existing gates. | LOW |
| S10-6 | Manoeuvring Corridor Regression Test | Automated regression test: 150 km five-segment Kashmir–Zanskar corridor with heading changes up to 60°. Gates: NAV-01 < 100 m, accepted updates ≥ 60/75, max gap ≤ 20 km, zero innovation rejections. | All 4 gates pass. Suite → 226/222 (if added post-S10). | LOW |

Note: S10-5 and S10-6 are additions from the sandbox review. S10-5 should be implemented during S10-1 (no separate deployment step). S10-6 may be added to the suite after S10 close if the manoeuvring test script is packaged as a formal regression gate.

## 5.2  Execution Sequence (strict — each gate before next step)

- 1. git pull + 215/215 baseline (must be clean before any changes)

- 2. S10-1: python3 trn_stub_ncc_patch.py → 68/68 regression → 50 km smoke

- 3. S10-2: cp test_s9_nav01_pass.py tests/ → pytest 7/7

- 4. S10-4: cp run_als250_parallel.py repo root → 50 km parallel smoke (all 3 IMUs)

- 5. S10-3: cp als250_drift_chart.py dashboard/ → 250 km parallel run in tmux

- 6. Collect chart → git rm run_als250_parallel_v2.py → commit → push

## 5.3  Assessment Files Ready for Deployment

| File | Destination | Status |
| --- | --- | --- |
| trn_stub_ncc_patch.py | repo root | ✓ Built, end-to-end tested on copy of trn_stub.py. |
| run_als250_parallel.py | repo root | ✓ Built, syntax verified. subprocess.Popen per IMU. |
| test_s9_nav01_pass.py | tests/ | ✓ Built. 7 gates covering all S9 architectural decisions. |
| als250_drift_chart.py | dashboard/ | ✓ Built. Reads .npy files from sim/als250_results/, writes PNG + PDF. |

## 5.4  Explicitly Deferred from S10

| Item | Phase | Opens when |
| --- | --- | --- |
| Cybersecurity (FR-109–112) | Phase-2 | After TASL meeting confirms programme continuation |
| DMRL CNN upgrade | Phase-2 — hardware blocked | After GPU + training dataset + clearance |
| HIL / ROS2 / PX4 SITL | Phase-3 — hardware blocked | After TASL platform decision |
| VIO fusion (plains navigation) | Phase-2 | Addresses σ_terrain < 10 m boundary condition. Post-TASL. |
| 2D NCC GPU/CUDA optimisation | Phase-2 | Post-TASL, if Monte Carlo sweeps required |
| Any architectural change | Not in scope | Demo Fork: hardening and artefact only |

# 6  Performance Context

Critical lesson: Timing estimates from local benchmarks are lower bounds, not predictions. A 70-minute estimate for a 250 km run was wrong by 3× on the VM. Always run the 50 km smoke test first and observe actual wall time.

| Configuration | Steps/sec | 150 km wall time | 250 km × 3 models |
| --- | --- | --- | --- |
| verbose=False, Azure VM (measured) | ~1,810 | ~25 min | ~75 min |
| verbose=True + tee over SSH (measured) | ~45 | ~3.5 hours | Not viable |

MKL fork deadlock (permanent): run_als250_parallel_v2.py uses multiprocessing.Pool with fork() on Linux. MKL holds internal BLAS mutexes that the child inherits locked — permanent deadlock on first matrix call. Superseded by run_als250_parallel.py (subprocess.Popen).

# 7  Milestone State After S10

| Milestone | Gate Criterion | Evidence | Status after S10 |
| --- | --- | --- | --- |
| M1 — Autonomy Core | FSM deterministic; INS propagates without drift | S0+S1 passing | CLOSED |
| M2 — GNSS-Denied Navigation | Drift < 100 m/5 km over 250 km; BIM ≤ 250 ms; TRN every 2 km | S2/S3/S8/S9 + S8-D chart | CLOSES IN S10 |
| M3 — EW Survivability | Cost-map ≤ 500 ms; Replan ≤ 1 s; ZPI zero-RF enforced | S4+S6 passing | CLOSED |
| M4 — Terminal Autonomy | DMRL lock ≥ 0.85; Decoy reject ≥ 0.80/3f; L10s ≤ 2 s | S5 111/111 | CLOSED |
| M5 — Demo Presentation | HTML debrief; 9-panel dashboard; ALS-250 three-curve chart | S7 done; chart pending | CLOSES IN S10 |

After S10: all five Phase-1 milestones closed. Phase-1 demonstration package complete. Regression suite: 222/222 (226/222 if S10-6 manoeuvring regression added).

# 8  Checklists

## 8.1  Session Start Checklist for S10

- git checkout main && git pull origin main

- git log --oneline main | head -5  # Expected: 7fba53c at top

- python run_s5_tests.py  # 111/111

- python tests/test_s6_zpi_cems.py  # 36/36

- python run_s8_tests.py  # 68/68

- Must be 215/215 before any changes

- Confirm session goal with Amit before writing any code

## 8.2  End of Sprint Checklist for S10

- All new files committed and pushed to main

- run_als250_parallel_v2.py removed from repo root (git rm)

- Regression confirmed: 222/222 minimum

- S8-D chart files committed: dashboard/als250_drift_chart.png + .pdf

- SPRINT_STATUS.md updated — S10 COMPLETE, commit hash recorded

- Daily Logs/HANDOFF_S10_to_S11.md generated and committed

- Both files uploaded to Claude Project knowledge

- Tag applied: git tag s10-m5-closed && git push origin s10-m5-closed

- Pre-TASL spec action: ✅ COMPLETE (8 March 2026) — Part Two V7.1 Live spec updated. STIM300 ARW floor now reads ≤ 0.2°/√hr in both performance table and component spec table. File: MicroMind_PartTwo_V7_1_Live.docx. No further action required.

# 9  Known Issues Carried Forward

| Item | Status | Notes |
| --- | --- | --- |
| dashboard/als250_drift_chart.py (S8-D) | 🔲 Pending | Closes in S10-3. Requires S10-1 + S10-4 first. |
| V7 spec — STIM300 ARW floor | ✅ COMPLETE | Updated to ≤ 0.2°/√hr in V7.1 Live (8 March 2026). Both performance table and component spec table corrected. STIM300 typical ARW is 0.15°/√hr — now within spec. No further action. |
| run_als250_parallel_v2.py | 🔴 Broken | MKL fork deadlock. Remove from repo root when S10-4 deployed. |
| _POS_DRIFT_PSD = 1.0 | ⚠ Approx. | Engineering approximation, not derived from Allan variance. Acceptable for SIL. Revisit for TASL if precision required. |
| MAX_TRN_CORRECTION_M = 300 m | ⚠ Conservative | Change to 150 m before S10 (validated by sandbox). Can tighten to 50 m once system further validated. |

MicroMind / NanoCorteX  |  HANDOFF S9→S10  |  5 March 2026  |  PROGRAMME CONFIDENTIAL