MICROMIND SYSTEM FAMILY

PART TWO — IMPLEMENTATION

Autonomy · EW Resilience · GNSS-Denied Navigation Suite for Tactical UAV Operations

Concept Paper — Version 7 (Live Working Document)

Updated: February 2026  |  Incorporates Technical Review v1.1 and Team Amendments

| VERSION HISTORY | V6: Original draft — architecture, feature descriptions, system requirements. V7: Technical review feedback incorporated. Design Decision records added (DD-01, DD-02). Boundary conditions and acceptance criteria populated throughout. RADALT requirement phased across SIL/HIL/Production (team amendment). Compute Performance Specification replaces platform-specific SWaP table. Cross-mission learning deferred by design with Phase 1 minimum actions defined. Compute qualification sequencing revised: passive tracking from S1, active from S3. All placeholder thresholds (X, Y, Z) replaced with candidate engineering values. Formal NanoCorteX state machine added. Authority chain table formalised. |
| --- | --- |

# Table of Contents

Note: Right-click → Update Field in Microsoft Word to refresh page numbers.

1  Technical Appendix

1.1  Architectural Overview & Design Principles

1.2  NanoCorteX Deterministic State Machine

1.3  Authority Chain

1.4  Control Chain Overview & Data Flow

1.5  Sensor Fusion Layer

1.6  EW Engine

1.7  Navigation Engine

1.8  BIM — Beacon & Integrity Monitor (FR-101)

1.9  Terminal Guidance & DMRL (FR-103)

1.10  CEMS — Cooperative EW Sharing (FR-102)

1.11  ZPI Burst Scheduler (FR-104)

1.12  L10s-SE (FR-105)

1.13  Silent Homing Mode — SHM (FR-106)

1.14  Non-Functional Requirements

1.15  Algorithm Summary

1.16  Sensor Requirements

2  Failure Modes & Behaviour Logic

3  System Requirements Specification (SRS)

4  Design Decision Register

4.1  DD-01: Hardware Sovereignty & Compute Platform Selection

4.2  DD-02: Cross-Mission Learning Architecture

4.3  Compute Performance Specification

4.4  Compute Platform Qualification Candidates

5  Assurance, Security & Validation Frameworks

6  Growth Roadmap

7  Conclusion

8  Glossary of Terms

SECTION 1 — TECHNICAL APPENDIX

# 1  Technical Appendix

## 1.1  Architectural Overview & Design Principles

The system employs distributed autonomy across three segments: the commander interface (MicroMind-OS), the onboard mission computer (MicroMind-X), and the autonomy engine (NanoCorteX). The host autopilot (PX4 or equivalent) handles only vehicle stabilisation and is not modified.

Key architectural pillars:

- Mission Envelope as Sole Authority — all mission-phase parameters are pre-defined in a cryptographically signed envelope. The autonomy engine enforces these parameters; it cannot override them.

- Bounded Deterministic Autonomy — NanoCorteX executes a formally specified state machine with a finite set of states, legal transitions, and guards. No free-form AI decision-making occurs in-mission.

- Autonomy as a Payload — the MicroMind stack integrates with any compatible airframe via standard interfaces (MAVLink or equivalent). No flight controller modification is required.

- Full Redundancy in Sensing and State Estimation — degraded-mode navigation is available for every single sensor failure scenario.

- Silent Homing Mode (SHM) for final approach — enforces Zero RF Navigation in the terminal phase.

- Hardware-Agnostic Compute Architecture — no specific compute platform is named in the architecture. The system is specified against a Compute Performance Specification (Section 4.3). Platform selection occurs post-SIL. See DD-01.

## 1.2  NanoCorteX Deterministic State Machine

The NanoCorteX autonomy engine executes a formally defined state machine. All mode transitions must complete within ≤2 seconds (NFR-002). States, transitions, and guards are specified below.

### 1.2.1  Operational States

| State ID | State Name | Navigation Primary | RF Posture | Key Guard Conditions |
| --- | --- | --- | --- | --- |
| ST-01 | NOMINAL | GNSS (Trust: Green) | Full RF active | GNSS trust = Green; BIM OK; no active EW alert |
| ST-02 | EW_AWARE | GNSS (Trust: Amber) | Full RF active | EW cost map update detected; jammer hypothesis active; GNSS trust ≤ Amber |
| ST-03 | GNSS_DENIED | VIO + TRN primary | Full RF active | GNSS trust = Red; VIO feature count ≥ 20; TRN correlation valid |
| ST-04 | SILENT_INGRESS | VIO + TRN + DEM | ZPI bursts only | Terminal zone boundary crossed per envelope; RF suppressed except ZPI |
| ST-05 | SHM_ACTIVE | IMU + VO + DEM only | Zero RF | Final approach active; L10s-SE enforcement engaged; EO terminal lock acquiring |
| ST-06 | ABORT | Loiter / egress | Per envelope | L10s-SE abort decision; corridor violation; key mismatch detected |
| ST-07 | MISSION_FREEZE | N/A — all outputs suppressed | Zero RF | Anti-capture triggered; tamper detected; revoked key detected |

### 1.2.2  State Transition Rules

| From | To | Trigger | Guard | Max Transition Time |
| --- | --- | --- | --- | --- |
| ST-01 | ST-02 | EW Engine: jammer hypothesis confidence > 0.6 | GNSS trust ≤ Amber OR EW cost tile updated | ≤ 500 ms |
| ST-02 | ST-03 | BIM: GNSS trust state = Red | VIO feature count ≥ 20 or TRN correlation valid | ≤ 2 s |
| ST-02 | ST-01 | BIM: GNSS trust returns Green for 3 consecutive samples | No active jammer hypothesis above threshold | ≤ 500 ms |
| ST-03 | ST-04 | Navigation Engine: terminal zone boundary crossing (per mission envelope) | SHM pre-conditions met: VIO lock, DEM loaded, EO initialised | ≤ 2 s |
| ST-04 | ST-05 | L10s-SE: final approach initiated | EO target lock confidence ≥ 0.85; ≤ 10 s to impact | ≤ 1 s |
| ST-05 | ST-06 | L10s-SE: abort decision | Decoy flag; civilian detection; corridor violation; lock confidence < threshold | ≤ 500 ms |
| ANY | ST-06 | Mission envelope: abort command (pre-loaded) | Abort condition encoded in signed envelope | ≤ 2 s |
| ANY | ST-07 | Cybersecurity: key revocation OR tamper detection | Hardware tamper sensor OR software key revocation check | ≤ 1 s |

| IMPLEMENTATION NOTE | The state machine must be implemented as a deterministic finite automaton — not as conditional logic embedded in navigation code. A formal specification (UML statechart or equivalent) must be produced in SIL Sprint S1 and maintained as a controlled document thereafter. All state transitions must be logged with timestamp, trigger, and guard evaluation result. |
| --- | --- |

## 1.3  Authority Chain

The following hierarchy governs all decisions made by the MicroMind system. No lower layer can override a higher layer. The mission envelope (Layer 0) is the root of authority — it cannot be modified in flight.

| Layer | Authority Holder | Can Authorise | Cannot Override | Enforcement Mechanism |
| --- | --- | --- | --- | --- |
| 0 | Mission Envelope (signed, pre-loaded) | All mission-phase parameters, corridor, ROE, target | Nothing — is the root authority | PQC signature verification at load time; cryptographic hash checked each access |
| 1 | L10s-SE (FR-105) | Abort or continue in terminal phase (ST-05/ST-06) | Mission envelope corridor and ROE | Deterministic decision tree; no ML in this path |
| 2 | NanoCorteX Decision Core | Navigation mode transitions; route replan within envelope | L10s-SE terminal decision | EW cost map + BIM trust state; state machine guards |
| 3 | MicroMind-X (Sensor Fusion) | State vector outputs to NanoCorteX; GNSS trust score | Any routing or terminal decision | Fused pose + covariance; BIM output |
| 4 | Autopilot (e.g. PX4 or equiv.) | Vehicle attitude and stabilisation | Any mission logic | MAVLink or equivalent command interface from NanoCorteX |

## 1.4  Control Chain Overview & Data Flow

The architecture connects MicroMind-OS (commander), MicroMind-X (onboard mission computer), NanoCorteX (autonomy engine), and the aircraft autopilot, forming a fail-resilient mission-control chain.

| FIGURE PLACEHOLDER | Figure 1: MicroMind Family — System-Level Architecture Insert architecture diagram here. Original V6 figure retained from source document. Compute layer label to be updated to 'Heterogeneous Compute (Platform TBD per DD-01)' when diagram is regenerated. |
| --- | --- |

Data flow summary:

- MicroMind-OS generates the Mission Package: signed envelope containing route, EW heatmap, target signature, ROE parameters, ZPI hop plan seed, and DEM tiles.

- The Mission Package is encrypted (AES-256) and transferred to MicroMind-X at mission load time. No field-level modification is possible post-signing.

- MicroMind-X runs the Sensor Fusion pipeline (Section 1.5) and EW Observation, producing a Unified State Vector and GNSS Trust Score.

- NanoCorteX consumes the State Vector, BIM output, and EW Engine output to drive the state machine (Section 1.2) and issue navigation commands to the autopilot.

- Secure logs are written continuously to tamper-evident storage. ZPI bursts carry EW observations and telemetry during transit; a mandatory EW summary burst is transmitted in the pre-terminal window (per DD-02).

## 1.5  Sensor Fusion Layer

### 1.5.1  Inputs and Outputs

| Input | Rate | Role in Fusion |
| --- | --- | --- |
| IMU (Accel / Gyro / Baro) | 200–1000 Hz | High-rate inertial propagation; bias estimation and temperature compensation |
| EO/IR Camera (gimbal or nadir) | 25–50 FPS | Visual Odometry (feature tracking / optical flow); thermal ROI for DMRL |
| GNSS Raw (NMEA + Rx metadata) | 1–10 Hz | Position and velocity aiding; integrity checked by BIM before use |
| Visual Odometry delta (VO) | 25–50 Hz | Pose delta from consecutive camera frames; compensates IMU drift |
| SDR Signature snapshot | On event | EW environment input to BIM EW Impact Model |
| DEM Tiles (pre-loaded) | On access | Terrain correlation for TRN; terrain masking for satellite avoidance |
| RADALT (height AGL) | ≥ 10 Hz | TRN altitude reference — see phasing in Section 1.7 (FR-107) |

| Output | Rate | Consumer |
| --- | --- | --- |
| Unified State Vector (position, velocity, attitude, covariance) | 100 Hz internal; 30 Hz to autopilot | NanoCorteX; autopilot |
| GNSS Trust Score (continuous 0.0–1.0) | 10 Hz minimum | BIM; EKF measurement noise scaling |
| GNSS Integrity Flag / Events | On event | BIM; Decision Core |
| Fused Pose + Covariance (downsampled) | 100 Hz | NanoCorteX Navigation Engine |

### 1.5.2  Fusion Algorithm

Primary fusion uses Extended Kalman Filter (EKF) or Unscented Kalman Filter (UKF) depending on flight phase, with Visual-Inertial Odometry (VIO) and DEM correlation.

| Parameter | Value | Rationale |
| --- | --- | --- |
| Filter selection | EKF for cruise; switch to UKF when velocity > 20 m/s or attitude rate > 45°/s | UKF handles nonlinear dynamics better at high speed / terminal phase |
| IMU gyro ARW (minimum spec) | ≤ 0.2 °/√hr | Required to maintain acceptable drift over 100 km inertial hold |
| IMU gyro bias stability | ≤ 1 °/hr | Bounds position drift to ~1–2% of path over 50 km |
| VIO minimum feature count | ≥ 20 tracked feature points | Below this threshold: flag VIO degraded; increase TRN reliance |
| VIO fallback trigger | Feature count < 20 for 3 consecutive frames | Switch navigation primary from VIO to TRN; log event |
| GNSS trust score → EKF coupling | R_GNSS = R_nominal / trust_score (capped at 10× R_nominal) | Gracefully degrades GNSS measurement weight as trust falls |
| DEM tile pre-load window | ±20 km ahead of planned route; refresh at 5 Hz | SRTM 1-arc-sec (30 m resolution) is adequate for TRN and masking |
| State vector output rate (autopilot) | 30 Hz (ODOMETRY message or equivalent) | Standard rate for MAVLink-based autopilot odometry input |

| FIGURE PLACEHOLDER | Figure 2: Sensor Fusion Pipeline — IMU / VO / GNSS Integrity Insert pipeline diagram here. Original V6 figure retained from source document. |
| --- | --- |

## 1.6  EW Engine

### 1.6.1  Functions

- Wideband SDR sweep — spectral monitoring for jammer and spoof signatures

- Jam / spoof detection — classification of RF anomalies as jamming or spoofing events

- Signature clustering — grouping of SDR observations into jammer node hypotheses (DBSCAN)

- Bearing / RSSI triangulation — geolocation of jammer nodes

- EW cost-map generation — tile-based risk surface fed to route planner

- EW node prediction — Kalman-tracked jammer position and velocity; 30 s lookahead

- CEMS triangulation — bearing and RSSI intersection using peer observations

### 1.6.2  Boundary Conditions

| Parameter | Value | Notes |
| --- | --- | --- |
| SDR bandwidth (SIL) | 2–20 MHz | Adequate for SIL algorithm validation; see production note |
| SDR bandwidth (production target) | ≥ 40 MHz | L/S-band jammer coverage; subject to hardware selection (DD-01) |
| Signature clustering algorithm | DBSCAN | Preferred over KNN for noise robustness in dense EW environments |
| DBSCAN parameters | eps = 3 dB (RSSI domain); min_samples = 3 | Starting values; tune against BCMP-1 EW simulator in Sprint S4 |
| Cluster re-evaluation interval | Every 500 ms | Balances responsiveness vs. computational cost |
| Jammer position accuracy (solo UAV) | CEP-50 ≤ 500 m | Sufficient for route avoidance; CEMS multi-node improves significantly |
| EW cost-map refresh rate | ≤ 500 ms (NFR-005) | Aligns with route replan latency budget |
| EW cost-map tile resolution | ≤ 50 m | Required for meaningful route planner avoidance corridor |
| Spoof detection trigger | Multi-constellation position delta > 15 m AND C/N0 drop > 6 dB simultaneously | Dual-condition prevents single-fault false positives |
| EW node prediction horizon | 30 seconds ahead of current position | Allows route replan before entering predicted jammer zone |
| Route replan trigger | Route cost increase > 20% following EW update | Balances stability vs. responsiveness; tune in Sprint S4 |
| Route replan time budget | ≤ 1 second | Hybrid A* with pre-computed DEM should achieve this |

| FIGURE PLACEHOLDER | Figure 3: EW Engine Architecture — JAM & SPOOF Detection Insert EW Engine diagram here. Original V6 figure retained from source document. |
| --- | --- |

## 1.7  Navigation Engine

The Navigation Engine provides GNSS-denied route following, terrain correlation, and visual-inertial odometry under all operational states. In contested terrain (Himalayan snowfields, Thar Desert), Visual Odometry may degrade due to feature-sparse environments; the Terrain Relative Navigation (TRN) subsystem provides a complementary aiding source.

### 1.7.1  Navigation Modes and Algorithm Summary

| Mode | Primary Algorithm | Aiding Sources | Active In State |
| --- | --- | --- | --- |
| Full GNSS | EKF with GNSS measurement | IMU, GNSS, VO, DEM | ST-01 |
| EW-Aware GNSS | EKF with trust-weighted GNSS | IMU, GNSS (trust-scaled), VO, EW cost map | ST-02 |
| GNSS-Denied (VIO primary) | EKF/UKF; VIO primary | IMU, VO, TRN, DEM | ST-03 |
| Silent Ingress | EKF/UKF; TRN primary | IMU, VO, TRN (RADALT + DEM), no GNSS | ST-04 |
| Silent Homing (SHM) | Inertial + DEM only | IMU, VO, DEM; Zero RF | ST-05 |

### 1.7.2  Terrain Relative Navigation (TRN) — FR-107

TRN fuses radar altimeter data with pre-loaded Digital Elevation Maps to correct inertial drift without GNSS.

| RADALT PHASING — THREE-TIER REQUIREMENT | MVP-SIL Phase: RADALT simulated via Gazebo plugin. Validates TRN algorithm, DEM correlation logic, and fusion pipeline behaviour. No hardware procurement required to gate SIL progress. MVP-HIL Phase: Physical RADALT required. Validates real sensor noise characteristics, update rate jitter, ground bounce behaviour, and interface integration. Production / RADALT mandatory. The 100 km GNSS-denied ingress performance claim 100 km Claim: cannot be made without it. Barometric altitude is insufficient for Himalayan terrain at 30 m DEM resolution. RADALT Performance Specification (for SIL plugin modelling and HIL procurement): Altitude range: 0.5–50 m AGL Update rate: ≥ 10 Hz Mass budget: ≤ 40 g Power budget: ≤ 0.5 W Interface: UART or I2C; digital output preferred Specific unit selection deferred to hardware qualification phase per DD-01. |
| --- | --- |

| TRN Parameter | Value | Rationale |
| --- | --- | --- |
| TRN update rate | ≥ 2 Hz | TERCOM literature; adequate for platforms at ≤ 100 m/s |
| DEM correlation algorithm | Normalised Cross-Correlation (NCC) of altimeter profile vs DEM strip | TERCOM-heritage; computationally tractable; proven accuracy |
| Kalman correction interval | Every 2 km of ground track | Balances drift accumulation vs. correction frequency |
| TRN position correction error (target) | < 50 m CEP-95 | TERCOM-class on 30 m DEM; conservative estimate for mountainous terrain |
| DEM resolution | SRTM 1-arc-sec (≈ 30 m) | Freely available; adequate for TRN in Indian operational terrain |

### 1.7.3  Satellite Avoidance Logic — FR-108

The Navigation Engine ingests pre-loaded satellite ephemeris data to predict enemy satellite overpasses and trigger terrain masking manoeuvres.

| Parameter | Value | Notes |
| --- | --- | --- |
| Ephemeris source | Pre-loaded TLE files updated per mission | Live download not used in mission; pre-mission planning only |
| Propagator | SGP4 | Standard for LEO satellite ephemeris prediction |
| Prediction horizon | ± 2 hours from planned mission window | Covers typical loitering munition mission duration |
| Masking trigger — elevation | Predicted satellite elevation > 15° above horizon | Conservative; covers most electro-optical collection geometries |
| Masking trigger — lateral | Within 200 m of planned corridor | Triggers route adjustment to terrain defilade |
| Minimum terrain depression for masking | ≥ 15 m below surrounding terrain | Provides meaningful optical obscuration from overhead collection |

### 1.7.4  Additional Navigation Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Route planning algorithm | Hybrid A* with EW cost overlay | EW cost tiles fed directly into A* heuristic |
| Slope / gradient awareness | Terrain gradient included in A* cost function | Prevents route planning into terrain features beyond airframe climb limits |
| Drift correction mechanism | TRN Kalman update + VIO delta integration | Both active simultaneously where VIO feature density is sufficient |
| Silent Homing fallback | IMU + VO + DEM; Zero RF | Active in ST-05; see Section 1.13 |

## 1.8  BIM — Beacon & Integrity Monitor (FR-101)

BIM evaluates trust in GNSS signals and produces a continuous trust score and discrete trust state used by the EKF and the NanoCorteX state machine.

### 1.8.1  Trust Evaluation Inputs

| Input | Analysis Method | Contribution |
| --- | --- | --- |
| GNSS Raw (SNR, constellation, timing) | RAIM; multipath detection; timing anomaly scoring | Primary integrity signal |
| Fused Pose + Covariance | Pose consistency check; innovation residual analysis | Detects position jumps inconsistent with inertial trajectory |
| EW Events (spike, jam, spoof alerts) | EW Impact Model: frequency overlap and RSSI correlation | Contextualises GNSS anomalies vs. known EW environment |
| Operator Notes (optional) | Manual flag input from MicroMind-OS at mission load | Pre-informs BIM of known degraded GNSS areas |

### 1.8.2  Trust State Machine and Boundary Conditions

| Parameter | Value | Rationale |
| --- | --- | --- |
| Trust states | Green (G) / Amber (A) / Red (R) | Three-state model; G = full use; A = trust-weighted; R = exclude |
| RAIM PDOP threshold — Amber | PDOP > 3.0 | Standard aviation RAIM threshold; conservative for tactical use |
| RAIM PDOP threshold — Red | PDOP > 6.0 | GNSS solution considered unreliable above this value |
| Doppler deviation — Amber | ¦deviation¦ > 0.5 m/s | Indicates signal manipulation inconsistent with platform kinematics |
| Doppler deviation — Red | ¦deviation¦ > 1.5 m/s | High confidence of spoofing or severe jamming |
| Multi-constellation position delta — Red | GPS vs. GLONASS position delta > 15 m | Spoof signals rarely manipulate two independent constellations consistently |
| State transition hysteresis | 3 consecutive samples at new state before transition | Prevents rapid G→A→G oscillation in marginal signal conditions |
| Continuous trust score output | 0.0–1.0 at ≥ 10 Hz | Not just G/A/R; continuous score drives EKF noise scaling |
| EKF measurement noise scaling | R_GNSS = R_nominal / trust_score (capped at 10× R_nominal) | Gracefully degrades GNSS influence as trust falls toward Red |
| BIM processing latency budget | GNSS analyse ≤ 80 ms + score combine ≤ 30 ms + output ≤ 20 ms = ≤ 130 ms target | Total latency ≤ 250 ms (NFR-001) with margin |

### 1.8.3  Score Weighting Model (Starting Values — Tune in SIL Sprint S2)

| Score Component | Weight (initial) | Range | Notes |
| --- | --- | --- | --- |
| RAIM / PDOP score | 0.35 | 0–1 | Highest weight; most reliable integrity indicator |
| Doppler deviation score | 0.25 | 0–1 | Strong spoof discriminator |
| Multi-constellation consistency score | 0.20 | 0–1 | Requires ≥ 2 constellations tracked |
| Pose innovation residual score | 0.15 | 0–1 | Detects position jumps vs. inertial prediction |
| EW impact score | 0.05 | 0–1 | Contextual; increases sensitivity near jammer zones |

Weighted trust score = Σ(component score × weight). Thresholds: G if score ≥ 0.7; A if 0.4 ≤ score < 0.7; R if score < 0.4. All thresholds subject to SIL Sprint S2 tuning against recorded GNSS attack data.

| FIGURE PLACEHOLDER | Figure 4: BIM (Beacon & Integrity Monitor) Architecture Insert BIM diagram here. Original V6 figure retained from source document. |
| --- | --- |

## 1.9  Terminal Guidance & DMRL (FR-103)

### 1.9.1  Hardware Requirements

| MANDATORY HARDWARE — THERMAL SENSOR | A radiometric LWIR (Long-Wave Infrared) sensor is required for DMRL operation. Standard visible cameras are insufficient for decoy rejection in contested environments. Performance specification (for SIL modelling and HIL procurement): Spectral range: 8–14 μm (LWIR) Radiometric: Yes — temperature-calibrated output (16-bit) Resolution: ≥ 320×240 pixels Frame rate: ≥ 25 FPS Interface: GigE Vision or USB3 Specific unit selection deferred to hardware qualification per DD-01. |
| --- | --- |

Indigenous Threat Library: The autonomy engine is pre-loaded with target signature files specific to relevant threat assets, trained on synthetic data matching Indian border lighting and terrain conditions. Classification handling of this data is a programme security requirement and must be addressed before any data pipeline is designed.

### 1.9.2  DMRL Processing Pipeline

- Frame preparation — rectify, stabilise, apply radiometric calibration

- Feature extraction — edges, optical flow, thermal ROI identification

- Temporal association — multi-frame target tracking across ≥ 5 consecutive frames

- Decoy rejection — CNN classifier with thermal dissipation model; multi-frame confidence accumulation

- Lock confidence computation — probability + covariance; gate for L10s-SE

- Aimpoint and impact correction — ballistic offset computation within L10s-SE corridor

### 1.9.3  DMRL Boundary Conditions

| Parameter | Value | Rationale |
| --- | --- | --- |
| Lock confidence threshold to proceed | ≥ 0.85 probability | High confidence required before committing to terminal run |
| Decoy rejection confidence to abort | ≥ 0.80 CNN decoy probability over 3 consecutive frames | Multi-frame requirement prevents single-frame misclassification |
| Minimum temporal association window | ≥ 5 frames at 25 FPS = 200 ms dwell | Minimum dwell for thermal dissipation signature to manifest |
| Minimum thermal ROI size | 8 × 8 pixels at maximum engagement range | Below this, thermal discrimination is unreliable |
| Aimpoint correction limit | ± 15° bearing adjustment within L10s-SE window | Prevents grossly off-axis corrections that exceed airframe manoeuvre limits |
| Target re-acquisition timeout | If EO lock lost > 1.5 s during terminal run → abort | Prevents blind terminal continuation without confirmed target lock |
| L10s-SE abort/continue decision | Must be made within ≤ 2 s of L10s-SE activation | Leaves ≤ 8 s for terminal run; validate timing in SIL Sprint S5 |
| IMU fine filter coupling | IMU delta used for inter-frame gimbal stabilisation | Required when gimbal-stabilised; handled by frame prep step |

| FIGURE PLACEHOLDER | Figure 5: Terminal Guidance & DMRL — EO/IR Multi-frame Match Insert DMRL pipeline diagram here. Original V6 figure retained from source document. |
| --- | --- |

## 1.10  CEMS — Cooperative EW Sharing (FR-102)

CEMS enables peer-to-peer cooperative EW intelligence sharing between UAVs in a formation. Each platform collects SDR observations, packages them as signed canonical packets, and transmits them to peers via ZPI-aware burst scheduling. Received packets are merged into the local EW picture and fed back into the route generator.

### 1.10.1  CEMS Packet Flow

- Local SDR signatures → canonical packet builder (timestamp, freq, RSSI, bearing, signature metadata)

- Packet signed and encrypted using pre-shared keys or certificate (AES-256; PQC-ready)

- Transmission scheduled by ZPI Burst Scheduler (Section 1.11) — ZPI-window aware

- Peer packet received → Auth Validator (cert check + replay attack check)

- Validated packets → Spatial-Temporal Merge Engine → EW update to route generator and EW Engine

### 1.10.2  CEMS Boundary Conditions

| Parameter | Value | Rationale |
| --- | --- | --- |
| Spatial merge radius | Observations within 200 m = same jammer node | GPS-class position accuracy at peer range; conservative |
| Temporal merge window | Observations within 15 s merged; older decay at 0.1/s confidence | Jammer nodes are relatively stationary; 15 s is conservative |
| Minimum peer confidence to merge | Signature confidence ≥ 0.5 | Prevents low-quality observations polluting the shared EW picture |
| Replay attack validity window | Reject packets with timestamp delta > 30 s from local clock | Standard tactical burst replay protection window |
| Maximum CEMS packet size | ≤ 256 bytes | Must fit within ZPI burst constraint; drives packet schema design |
| Packet schema version field | Version byte in header (mandatory from day 1) | Required for fleet upgrade without protocol breaks |
| Merge engine update rate | ≥ 1 Hz when peers active | Ensures EW picture remains current during formation operations |
| Auth validator — key type | Pre-shared mission keys; cert-based for multi-formation future | Consistent with mission envelope key provisioning (FR-112) |

### 1.10.3  Spatial-Temporal Merge Algorithm (Specification Required Before Sprint S6)

The merge algorithm is novel IP. The following specification is the basis for implementation; the team should review and refine before Sprint S6 commencement.

- Receive validated peer packet: extract (timestamp, bearing, RSSI, position estimate, confidence)

- Compute spatial distance from all existing jammer hypotheses in local EW map

- If distance < 200 m from existing hypothesis: merge using confidence-weighted average of position estimates

- If distance ≥ 200 m: create new jammer hypothesis entry

- Apply temporal decay: multiply confidence of all hypotheses by (1 - 0.1 × Δt) per second for Δt > 15 s

- Remove hypotheses with confidence < 0.1

- Output merged EW map to route generator and EW Engine at ≥ 1 Hz

| FIGURE PLACEHOLDER | Figure 6: CEMS Architecture — Cooperative EW Sharing (Packet Flow) Insert CEMS diagram here. Original V6 figure retained from source document. |
| --- | --- |

## 1.11  ZPI Burst Scheduler (FR-104)

Low Probability of Intercept signalling for telemetry, EW flash updates, and CEMS peer exchange. The hop plan is seeded from the mission key, providing implicit time-synchronisation between UAVs sharing the same mission package without explicit clock exchange.

### 1.11.1  ZPI Boundary Conditions

| Parameter | Value | Rationale |
| --- | --- | --- |
| Burst duration | ≤ 10 ms | Sub-frame duration makes wideband direction-finding very difficult |
| Inter-burst interval | Randomised 2–30 s | Exceeds typical DF integration time of 1–3 s |
| Frequency hop range | ± 5 MHz around mission-defined centre | Forces DF to cover wide aperture per burst; drives up adversary resource cost |
| Maximum duty cycle | ≤ 0.5% | Limits detectability to near noise floor at 5 km stand-off |
| Power variation range | − 10 to 0 dB relative to maximum | Variable power prevents RSSI-based stand-off ranging |
| DF risk adaptation trigger | EW engine jammer bearing within 45° of own track | Conservative; triggers 80% reduction in burst rate when active |
| Anti-DF adaptation action | Reduce burst rate; increase hop range; switch to directional burst if available | Ordered response; directional burst is optional hardware capability |
| Hop plan seed | Derived from mission key using HKDF-SHA256 | Deterministic but unpredictable to adversary without mission key |
| Mandatory pre-terminal burst | EW observation summary packet transmitted before SHM activation | Required per DD-02 Phase 1 minimum; supports future cross-mission learning |

| FIGURE PLACEHOLDER | Figure 7: ZPI Burst Scheduler — Micro-burst Timing & Control Insert ZPI diagram here. Original V6 figure retained from source document. |
| --- | --- |

## 1.12  L10s-SE (FR-105)

The Last-10-Second Safety Envelope is the final-phase enforced authority layer. It operates in ST-05 (SHM_ACTIVE) and ST-06 (ABORT). The decision tree encoded here reflects the Rules of Engagement pre-loaded in the mission envelope. No machine learning is used in this decision path — it is deterministic and auditable.

| L10s-SE Function | Description | Boundary Condition |
| --- | --- | --- |
| Civilian detection | EO/IR classifier checks terminal scene for non-combatant signatures | Detection confidence ≥ 0.7 in any frame within L10s window → abort |
| Abort / continue decision | Deterministic tree: lock confidence, decoy flag, civilian flag, corridor | See authority chain Layer 1; decision within ≤ 2 s of activation |
| Corridor hard enforcement | Terminal trajectory must remain within envelope-defined corridor | Any predicted corridor violation → immediate abort to ST-06 |
| Temporal constraint | L10s-SE window = ≤ 10 seconds from activation to impact or abort | Hard temporal limit; enforced by timer in state machine |
| EO lock re-acquisition | Single attempt if lock lost; timeout = 1.5 s | If re-acquisition fails → abort (see DMRL boundary conditions) |
| Secure log entry | All L10s-SE events logged with timestamp, decision, and sensor state | Required for post-mission audit and ROE compliance review |

## 1.13  Silent Homing Mode — SHM (FR-106)

Silent Homing Mode is the zero-emission navigation mode active during the final approach (ST-05). All RF emissions — including ZPI bursts — are suppressed. The mandatory pre-terminal ZPI burst (DD-02) must be completed before SHM activation.

- Zero RF — all transmitters suppressed; pre-terminal ZPI burst must be complete

- IMU + VO + DEM — inertial navigation with visual odometry and DEM correlation as primary

- EO terminal lock — EO/IR sensor acquires and tracks target; DMRL decoy rejection active

- Decoy detection (DMRL) — multi-frame CNN classifier with thermal dissipation model

- Terminal logic under L10s-SE — all decisions governed by L10s-SE decision tree

| FIGURE PLACEHOLDER | Figure 8: Terminal Guidance & DMRL — reproduced for SHM context Insert SHM/DMRL diagram here. Original V6 figure retained from source document. |
| --- | --- |

## 1.14  Non-Functional Requirements

| NFR ID | Requirement | Value | V7 Status |
| --- | --- | --- | --- |
| NFR-001 | BIM processing latency | ≤ 250 ms end-to-end (target ≤ 130 ms internal) | Filled — derived from BIM budget in Section 1.8 |
| NFR-002 | Autonomy fallback transition time | ≤ 2 seconds for any state machine transition | Filled — enforced by state machine timer |
| NFR-003 | SDR EW processing latency | < 500 ms from snapshot to cost map update | Filled — aligns with EW Engine boundary conditions |
| NFR-004 | TRN update loop rate | ≥ 2 Hz | Filled — per TERCOM literature; see Section 1.7 |
| NFR-005 | EW cost-map refresh rate | ≤ 500 ms | Filled — consistent with NFR-003 |
| NFR-006 | EW detection — Probability of Detection | Pd ≥ 0.90 | Filled — Neyman-Pearson criterion; SIL starting value |
| NFR-007 | EW detection — Probability of False Alarm | Pfa ≤ 0.05 | Filled — SIL starting value; tune Sprint S4 |
| NFR-008 | EO/IR terminal lock latency | ≤ 400 ms (25 FPS × 10 frames) | Filled — derived from L10s-SE time budget |
| NFR-009 | Encryption standard | AES-256 at rest and in transit; PQC-ready signatures | Existing — maintained |
| NFR-010 | Post-quantum cryptography readiness | SPHINCS+ / Dilithium signature schemes | Existing — maintained |
| NFR-011 | Compute platform SWaP | Defined by Compute Performance Spec (Section 4.3) | Revised — platform-agnostic per DD-01 |
| NFR-012 | Secure boot | Hardware root of trust mandatory; specific mechanism per hardware qualification | Revised — abstracted per DD-01 |
| NFR-013 | Log completeness | ≥ 99% of expected log fields captured per mission | Filled — required for post-mission audit and DD-02 |
| NFR-014 | Anti-capture erase time | ≤ 5 seconds from trigger to mission data erasure | Filled — operational security requirement |

## 1.15  Algorithm Summary

| Domain | Algorithm(s) | FR / Section Reference |
| --- | --- | --- |
| Sensor fusion | EKF (cruise) / UKF (high-speed / terminal); Visual-Inertial Odometry | Section 1.5 |
| GNSS integrity | RAIM; multi-constellation consistency; Doppler deviation analysis; weighted score combiner | Section 1.8 (BIM) |
| Navigation — routing | Hybrid A* with EW cost overlay; slope/gradient awareness | Section 1.7 |
| Navigation — terrain | Normalised Cross-Correlation TRN; Kalman-corrected DEM matching | Section 1.7 (FR-107) |
| Navigation — masking | SGP4 ephemeris propagation; DEM depression analysis | Section 1.7 (FR-108) |
| EW detection | Spectrogram analysis; DBSCAN signature clustering; bearing/RSSI triangulation | Section 1.6 |
| EW prediction | Kalman-tracked jammer node prediction; 30 s lookahead | Section 1.6 |
| Terminal guidance | Multi-frame EO/IR association; CNN decoy rejection; thermal dissipation model | Section 1.9 (FR-103) |
| CEMS merge | Confidence-weighted spatial-temporal merge; temporal decay | Section 1.10 (FR-102) |
| Communications | Spread-spectrum ZPI; HKDF-SHA256 hop plan derivation from mission key | Section 1.11 (FR-104) |
| Cybersecurity | AES-256; SPHINCS+ / Dilithium PQC signatures; HKDF key derivation | Section 5.1 |

## 1.16  Sensor Requirements

| Sensor | Minimum Specification | Phase Availability | Notes |
| --- | --- | --- | --- |
| IMU | 200–1000 Hz; Gyro ARW ≤ 0.2 °/√hr; bias ≤ 1 °/hr | SIL + HIL + Production | Automotive-grade tactical IMU class |
| EO/IR Camera (thermal) | ≥ 320×240 px; radiometric LWIR; ≥ 25 FPS; GigE or USB3 | HIL + Production | Standard visible camera insufficient for DMRL |
| EO Camera (visible) | ≥ 720p; ≥ 25 FPS; global shutter preferred | SIL + HIL + Production | Used for VIO feature tracking |
| SDR Front-end | 2–20 MHz BW (SIL); ≥ 40 MHz BW (production) | SIL (2–20 MHz); Production (≥ 40 MHz) | Subject to hardware selection per DD-01 |
| RADALT | 0.5–50 m AGL; ≥ 10 Hz; ≤ 40 g; ≤ 0.5 W | SIL: simulated plugin; HIL + Production: physical | Three-tier phasing — see Section 1.7.2 |
| GNSS Receiver | Multi-constellation (GPS + GLONASS minimum); raw NMEA + metadata output | SIL + HIL + Production | Multi-constellation required for BIM spoof detection |

SECTION 2 — FAILURE MODES & BEHAVIOUR LOGIC

# 2  Failure Modes & Behaviour Logic

NanoCorteX transitions between operational states (Section 1.2) in response to failure conditions. All transitions must complete within ≤ 2 seconds (NFR-002) and are enforced by the deterministic state machine. All failure events are logged to the tamper-evident secure log.

| Failure Mode | Detection Method | State Transition | Recovery / Behaviour | Max Transition |
| --- | --- | --- | --- | --- |
| GNSS Spoofing | BIM: trust score < 0.4; multi-constellation delta > 15 m | ST-01/02 → ST-03 | Navigation switches to VIO + TRN primary; GNSS excluded from EKF | ≤ 2 s |
| GNSS Jamming | BIM: C/N0 drop; RAIM failure; Doppler deviation | ST-01 → ST-02 → ST-03 | Amber: trust-weighted use; Red: exclude GNSS entirely | ≤ 2 s (per step) |
| IMU Drift | EKF innovation residual spike; VIO-inertial consistency check | Any → increased covariance | Increase EKF process noise; rely more heavily on VIO and TRN | No state change; covariance update ≤ 500 ms |
| EO / VIO Blackout | Feature count < 20 for 3 consecutive frames | Any → TRN primary | Switch navigation to TRN primary; log VIO degraded event | ≤ 500 ms |
| SDR Saturation / Overload | SDR IQ clip detection; processing latency spike | Any → EW Engine alert | Flag EW Engine output as unreliable; hold last valid cost map | ≤ 500 ms |
| Corridor Violation (predicted) | Navigation Engine: predicted trajectory exits envelope corridor | Any → ST-06 ABORT | Immediate abort command to autopilot; egress or loiter per envelope | ≤ 500 ms |
| Target Lock Loss (terminal) | DMRL: EO lock confidence < threshold for > 1.5 s | ST-05 → ST-06 ABORT | Abort; secure log entry with final sensor state | ≤ 500 ms |
| SHM Trigger | State machine: terminal zone boundary; L10s-SE activation | ST-04 → ST-05 | Pre-terminal ZPI burst transmitted; all RF suppressed; inertial + EO only | ≤ 1 s |
| Key Mismatch / Revocation | Cybersecurity: signature verification failure; revocation list hit | Any → ST-07 MISSION_FREEZE | All outputs suppressed; anti-capture erase sequence initiated | ≤ 1 s |
| Tamper Detection | Hardware tamper sensor; mechanical intrusion flag | Any → ST-07 MISSION_FREEZE | Mission data erase within ≤ 5 s (NFR-014) | ≤ 1 s |

SECTION 3 — SYSTEM REQUIREMENTS SPECIFICATION

# 3  System Requirements Specification (SRS)

## 3.1  Functional Requirements (FR)

| FR ID | Requirement | Key Boundary Conditions | SIL Sprint | Status |
| --- | --- | --- | --- | --- |
| FR-101 | BIM: evaluate GNSS trust; output continuous score and G/A/R state | PDOP thresholds; Doppler deviation limits; hysteresis rule; latency ≤ 250 ms — see Section 1.8 | S2 | Boundary conditions filled V7 |
| FR-102 | CEMS: peer-to-peer cooperative EW intelligence sharing | Packet ≤ 256 bytes; merge radius 200 m; temporal window 15 s; replay window 30 s — see Section 1.10 | S6 | Boundary conditions filled V7 |
| FR-103 | DMRL: multi-frame terminal guidance with decoy rejection | Lock threshold 0.85; abort threshold 0.80; min dwell 5 frames; re-acq timeout 1.5 s — see Section 1.9 | S5 | Boundary conditions filled V7 |
| FR-104 | ZPI: low probability of intercept burst communications | Burst ≤ 10 ms; interval 2–30 s; duty cycle ≤ 0.5%; pre-terminal EW burst mandatory — see Section 1.11 | S4 / S6 | Boundary conditions filled V7 |
| FR-105 | L10s-SE: final-phase safety envelope enforcement | Decision within ≤ 2 s; corridor hard enforcement; temporal limit ≤ 10 s — see Section 1.12 | S5 | Boundary conditions filled V7 |
| FR-106 | SHM: zero-emission final approach navigation | Zero RF; IMU + VO + DEM; pre-terminal ZPI burst complete before activation — see Section 1.13 | S5 | Boundary conditions filled V7 |
| FR-107 | TRN: terrain relative navigation using altimeter + DEM | TRN rate ≥ 2 Hz; NCC algorithm; correction ≤ 50 m CEP-95; RADALT phasing — see Section 1.7 | S3 (sim) | Phasing documented V7 |
| FR-108 | Satellite avoidance: ephemeris-driven terrain masking | SGP4 propagator; elevation > 15° trigger; depression ≥ 15 m required — see Section 1.7 | S3 | Boundary conditions filled V7 |
| FR-109 | Mission envelope cryptographic validation | AES-256 storage; PQC signatures (SPHINCS+ / Dilithium); hash checked each access | S1 | Existing requirement |
| FR-110 | Adversarial robustness logic | CNN white-box attack resistance; BIM black-box spoof resistance; integrated in SIL red-team | S2/S5 | Red-team schedule added V7 |
| FR-111 | Secure boot and firmware integrity enforcement | Hardware root of trust; signed firmware only; mechanism TBD at hardware qualification per DD-01 | HIL | Abstracted per DD-01 |
| FR-112 | Key management: provision, rotation, revocation | Depot-level provisioning; ephemeral session keys; pre-loaded revocation list; HKDF-SHA256 derivation | S1 | Algorithm specified V7 |
| FR-113 | Anti-capture logic (optional for MVP) | Tamper sensor trigger; secure erase ≤ 5 s; mission freeze state (ST-07) | HIL+ | Erase time budget added V7 |

## 3.2  Non-Functional Requirements (NFR)

See Section 1.14 for the full NFR table with filled values. Summary of key latency chain:

| NFR | Value | Chain Position |
| --- | --- | --- |
| NFR-001 | BIM latency ≤ 250 ms | GNSS anomaly → BIM score → EKF noise update |
| NFR-002 | State machine transition ≤ 2 s | Any failure event → new operational state → behaviour change |
| NFR-003 | SDR EW processing < 500 ms | RF event → EW Engine → cost map tile → route planner |
| NFR-004 | TRN update ≥ 2 Hz | Altimeter + DEM → Kalman correction → state vector update |
| NFR-008 | EO/IR lock latency ≤ 400 ms | Terminal acquisition → DMRL confidence ≥ 0.85 → proceed gate |

## 3.3  Algorithm Notes

| Algorithm | Application | Key Parameters |
| --- | --- | --- |
| Hybrid A* | EW-aware route generation | EW cost tiles as heuristic; gradient-aware; replan ≤ 1 s on > 20% cost increase |
| Weighted DEM (NCC) | Terrain matching for TRN | SRTM 30 m tiles; Kalman correction every 2 km; update ≥ 2 Hz |
| CNN + thermal dissipation | DMRL decoy rejection | Min 5-frame dwell; lock confidence ≥ 0.85; decoy abort ≥ 0.80 over 3 frames |
| DBSCAN | EW signature clustering | eps = 3 dB RSSI; min_samples = 3; interval 500 ms |
| Spread-spectrum ZPI | LPI burst scheduling | Burst ≤ 10 ms; interval 2–30 s; duty cycle ≤ 0.5%; HKDF seed |
| SGP4 | Satellite ephemeris propagation | ± 2 hr prediction window; masking trigger at elevation > 15° |
| EKF / UKF | Sensor fusion state estimation | EKF for cruise; UKF above 20 m/s or 45°/s attitude rate |
| Confidence-weighted spatial-temporal merge | CEMS EW picture fusion | Merge radius 200 m; decay 0.1/s after 15 s; min confidence 0.5 |
| HKDF-SHA256 | ZPI hop plan and key derivation | Seeded from mission key; deterministic but adversarially unpredictable |
| Adversarial signature rejection | DMRL and BIM robustness | FGSM / PGD white-box testing in SIL Sprint S5; adaptive spoof in Sprint S2 |

SECTION 4 — DESIGN DECISION REGISTER

# 4  Design Decision Register

This section records programme-level design decisions that have architectural implications. These decisions are formally documented so they are visible to all stakeholders, including future hardware partners.

## 4.1  DD-01: Hardware Sovereignty & Compute Platform Selection

| DESIGN DECISION DD-01 | Decision: No compute platform is named or implied in the Phase 1 architecture. Date: February 2026 Owner: Programme Director Review: End of SIL Sprint S5 Rationale: US Export Administration Regulations (EAR) controls have been applied to high-performance compute with minimal notice (NVIDIA A100/H100 ban, 2022-23). Jetson Orin-class devices are currently classified EAR 3A991 but reclassification to MTCR-controlled 3A001 is a credible risk over this programme's operational lifetime. Hard-coding the architecture to any US-origin compute creates a single point of strategic failure for an Indian defence programme. Approach: The architecture is specified against a Compute Performance Specification (Section 4.3). Any candidate compute platform is qualified against that specification. SWaP limits are derived outputs of whichever platform passes qualification — not inputs to the architecture. SIL note: Jetson Orin NX may be used as SIL development proxy (highest TOPS, best toolchain, lowest friction for algorithm validation). This use must be explicitly documented as SIL-only and must not create production dependency. Production: Platform selection at end of SIL. Primary qualification candidates: Hailo-8 (Israeli; low power; strong inference); Ambarella CV3 (US; lower EAR risk). CDAC Vega (Indian domestic) engagement to begin at programme level regardless of technical maturity — strategic relationship value is independent of timeline. |
| --- | --- |

## 4.2  DD-02: Cross-Mission Learning Architecture

| DESIGN DECISION DD-02 | Decision: Full cross-mission learning pipeline deferred to Phase 2 pending architecture design. Phase 1 minimum actions defined below. Date: February 2026 Owner: Software Architect Review: SIL Sprint S1 (schema) / Sprint S4 (ZPI packet) Rationale: Standard cross-mission learning assumes the platform returns. Loitering munitions are one-way. Learning is therefore fleet-level, not platform-level. The ZPI burst protocol already provides the data exfiltration channel. Cross-mission learning is the persistence layer of CEMS — writing EW observations back to MicroMind-OS to update the next mission package. However, the ground truth signal (BDA, mission complete/abort status, ZPI burst receipt log) must be formally defined before the learning pipeline can be specified. This design work is deferred. Phase 1 minimum actions (zero additional engineering cost at SIL): 1. Mission log schema: include EW observations, BIM trust timeseries, DMRL confidence scores, route deviation events, and final state at mission end. 2. ZPI pre-terminal burst: mandatory EW observation summary packet transmitted before SHM activation (see FR-104 and Section 1.11). 3. MicroMind-OS: document 'mission debrief ingest' hook for Phase 2 implementation. Risk of not doing Phase 1 minimum: Phase 2 discovers existing logs lack the fields needed for learning. Missions already flown generate incomplete data that cannot be retroactively enriched. |
| --- | --- |

## 4.3  Compute Performance Specification

Any candidate compute platform must meet or exceed all parameters in this specification. This replaces a fixed SWaP table tied to a named platform. SWaP limits are determined at hardware qualification.

| Parameter | Minimum Requirement | Rationale |
| --- | --- | --- |
| AI inference throughput (INT8) | ≥ 10 TOPS | DMRL CNN at 25 FPS + EW signature clustering in parallel real-time |
| CPU cores (real-time capable) | ≥ 4 cores at ≥ 1.5 GHz | NanoCorteX state machine + EKF/UKF + A* routing concurrent execution |
| RAM | ≥ 4 GB LPDDR4/5 | DEM tiles + sensor fusion state + EW cost map + threat library in-memory |
| Storage (mission package) | ≥ 32 GB eMMC or NVMe | DEM tiles (100 km radius) + mission envelope + target signature library |
| Real-time OS support | PREEMPT-RT Linux or certified RTOS | NFR-002 (≤ 2 s transition) and NFR-001 (≤ 250 ms BIM) require deterministic scheduling |
| Operating temperature | −20°C to +65°C | Himalayan and Thar Desert operational extremes |
| Peak power (at sustained AI load) | ≤ 15 W | Platform power budget ceiling; verified under DMRL + EKF concurrent load at qualification |
| Export control classification | Not EAR 3A001 / not MTCR Category I | Hardware sovereignty requirement per DD-01 |
| Volume envelope | ≤ 100 × 80 × 30 mm (indicative) | Airframe integration constraint — to be confirmed per airframe at HIL |
| Interface compatibility | UART / I2C / USB3 / GigE / CAN / PCIe | Required for sensor integration; specific subset confirmed at HIL |

## 4.4  Compute Platform Qualification Candidates

| QUALIFICATION TIMING — TEAM DECISION | Passive tracking (datasheets, export status, contact names, CDAC relationship initiation): → Begin immediately, from Sprint S1. Long-lead relationships justify early start. Active qualification (eval board procurement, stack porting, benchmarking against perf spec): → Defer until Sprint S3 complete. Performance data from S3 defines what 'adequate' means. → Starting deep vendor engagement before stack performance is understood risks anchoring to a platform before the real load is characterised. |
| --- | --- |

| Platform | Origin | TOPS (INT8) | Export Risk | Assessment | Role |
| --- | --- | --- | --- | --- | --- |
| Hailo-8 / Hailo-8L | Israel | 26 / 13 | LOW | Strong DMRL inference candidate; 2.5 W; pairs with ARM host CPU | Primary production candidate |
| Ambarella CV3 | USA | ~20 | MED | Lower export sensitivity than NVIDIA; integrated ISP; good EO pipeline | Primary production candidate |
| Qualcomm QRB5165 | USA | ~15 | MED | Snapdragon Flight heritage; proven UAV stack; EAR99 historically | Secondary candidate |
| NXP i.MX 95 | Netherlands | ~4 | LOW | Lower TOPS; may need Hailo co-processor for DMRL; strong for sensor fusion | Secondary / hybrid option |
| Rockchip RK3588 | China | ~6 | MED | Supply chain sovereignty concern for Indian defence application | SIL proxy use acceptable only |
| CDAC Vega (future) | India | TBD | NONE | Low current maturity; strategically critical; initiate relationship immediately | Phase 2+ production target |
| NVIDIA Jetson Orin NX | USA | 70–100 | HIGH | EAR 3A991 now; reclassification to MTCR Cat I is credible risk; best toolchain | SIL DEVELOPMENT ONLY per DD-01 |

SECTION 5 — ASSURANCE, SECURITY & VALIDATION FRAMEWORKS

# 5  Assurance, Security & Validation Frameworks

## 5.1  Cybersecurity & Zero-Trust Architecture

MicroMind operates under Zero Trust: all components verify each other's identity and integrity; no implicit trust is granted based on network position or physical proximity.

### 5.1.1  Threat Model

| Threat | Attack Vector | Mitigation |
| --- | --- | --- |
| Remote compute exploitation | Network or RF link during mission window | Zero RF in SHM; rolling session keys; autonomy-only fallback on key mismatch |
| Firmware / mission envelope tampering | Physical or supply chain access | Secure boot; hardware root of trust; AES-256 envelope; tamper-evident logs |
| SDR spoof injection targeting control loops | Adversarial RF transmission | BIM spoof vector probability; SDR signature whitelist validation |
| Control link hijack | RF interception and replay | Link-layer cryptographic handshake; rolling session keys; fallback to autonomy-only |
| GNSS spoofing | False GNSS signal injection | BIM multi-constellation consistency; Doppler deviation analysis; EW corroboration |
| False ZPI micro-burst injection | Adversary mimics peer CEMS packet | Auth Validator: cert check + replay window; signature whitelist |

### 5.1.2  Security Architecture

- Secure Boot + Hardware Root of Trust: mandatory for onboard compute; mechanism defined at hardware qualification (DD-01); ensures only signed firmware loads

- Encrypted Mission Envelope: AES-256 storage; PQC-ready signatures (SPHINCS+ / Dilithium); cryptographic hash verified on each access

- Key Provisioning: keys provisioned at depot level into secure enclave; no field-level key exposure

- Key Rotation: mission-specific ephemeral session keys derived via HKDF-SHA256 from master mission key; valid only within defined mission time window

- Key Revocation: pre-loaded revocation list checked at boot and at each envelope access; automatic MISSION_FREEZE (ST-07) if revoked key detected

- Tamper-Evident Logging: all events logged with SHA-256 hash chain and timestamp; log completeness ≥ 99% (NFR-013)

- Anti-Capture (Optional / FR-113): secure erase trigger ≤ 5 s (NFR-014); tamper sensors; MISSION_FREEZE state

## 5.2  Adversarial Robustness & Red-Teaming

### 5.2.1  Adversarial Attack Categories

| Attack Category | Specific Attacks | Target Module | SIL Sprint |
| --- | --- | --- | --- |
| Adversarial thermal / EO | Fake thermal sources; IR camouflage; frame-timing attacks | DMRL (FR-103) | S5 |
| Digital signal attacks | SDR replay; patterned spoof signatures; adversarial RF masking | BIM (FR-101); EW Engine | S2 / S4 |
| CNN white-box attacks | FGSM and PGD adversarial patches on DMRL classifier | DMRL (FR-103) | S5 |
| BIM black-box spoofing | Adaptive spoofing mimicking plausible flight paths (harder than documented spoof vectors) | BIM (FR-101) | S2 |
| CEMS replay injection | Replay of previously captured peer CEMS packets | CEMS Auth Validator | S6 |

Deliverables from red-team sprints: attack taxonomy document; robustness metrics per module; mitigation response library. These are produced during the SIL phase, not as a separate post-SIL activity.

## 5.3  Validation & Testing

### 5.3.1  SIL Test Case Format

All SIL test cases must conform to the following format to ensure unambiguous pass/fail determination and auditability for TASL demonstration.

| Field | Description | Example |
| --- | --- | --- |
| Test ID | FR reference + sequence number | BIM-TC-001 |
| FR / NFR | Requirement under test | FR-101: GNSS spoof detection latency ≤ 250 ms |
| Preconditions | System state before test | GNSS Green; all constellations tracked; EKF converged; ST-01 active |
| Stimulus | Injected input or event | GNU Radio GNSS spoof injection: position offset +200 m at T=0 |
| Expected Result | Quantified pass criterion | BIM trust state → Amber within 250 ms; Red within 500 ms |
| Measurement Method | How result is recorded | ROS2 bag: bim_state topic timestamp delta vs inject event timestamp |
| Pass / Fail | Binary determination | PASS if latency ≤ 250 ms in ≥ 9 of 10 consecutive runs |
| Red-Team Variant | Adversarial extension (where applicable) | Adaptive spoof mimicking plausible flight path; same acceptance criteria |

### 5.3.2  SIL Build Sequence and KPI Measurement Plan

| Sprint | Duration | Primary Deliverable | KPI Measured | TASL Signal |
| --- | --- | --- | --- | --- |
| S1 | Wks 1–3 | ROS2 skeleton; state machine formalised; all interfaces stubbed; mission log schema defined (DD-02) | Log completeness ≥ 99%; state machine transition timing | Architecture is runnable — not a diagram |
| S2 | Wks 4–6 | BIM operational: GNU Radio GNSS spoof → G/A/R transition with logged evidence | BIM latency ≤ 250 ms; spoof rejection ≥ 99% (100-run Monte Carlo); black-box adaptive spoof test | First novel IP demonstrated end-to-end |
| S3 | Wks 7–10 | Navigation Engine: 50 km GNSS-denied Gazebo flight; TRN with GDAL DEM and RADALT sim plugin | Drift < 2% at 5 km; TRN error < 50 m CEP-95 | 100 km GNSS-denied claim de-risked; compute qualification active engagement begins |
| S4 | Wks 11–14 | EW Engine + Hybrid A*: BCMP-1 jammer scenario; cost map; replan; ZPI pre-terminal burst (DD-02) | EW cost-map ≤ 500 ms; route replan ≤ 1 s; avoidance success rate | Differentiator demonstrated — EW-aware autonomous routing |
| S5 | Wks 15–18 | DMRL-lite: synthetic thermal scene; CNN decoy rejection; L10s-SE gate; full BCMP-1 run end-to-end | Terminal lock rate ≥ 85%; decoy rejection ≥ 90%; L10s-SE compliance 100%; fallback ≤ 2 s | CEO demo scenario: complete mission in simulation against quantified KPIs |
| S6 | Wks 19–24 | CEMS + ZPI multi-UAV sim; peer EW sharing; burst protocol; cybersecurity envelope signing | CEMS merge latency; ZPI LPI metrics; key verification correctness | Swarm and security stack complete |

### 5.3.3  BCMP-1 — Baseline Certification Mission Profile (Formal Definition)

BCMP-1 is the primary SIL acceptance gate. A successful end-to-end BCMP-1 run against all KPIs below is the minimum required before approaching hardware partners.

| BCMP-1 SCENARIO DEFINITION | Mission profile: 100 km ingress; GNSS denied from T+5 min; RF link lost at T+15 min. Terrain: Mountainous (DEM-representative of LAC corridor terrain). EW threat: 2× jammer nodes at mid-ingress; 1× spoofer at terminal approach. Jammer positions cause mandatory route replan ≥ 2× during ingress. Satellite window: 1× hostile satellite overpass at T+20 min; terrain masking required. Terminal: Thermal target with 1× decoy present; DMRL must reject decoy. Terminal EW: Highest EW density of mission; ZPI suppressed; SHM active. BCMP-1 Pass Criteria (ALL must be met): NAV-01: Drift < 2% of total path length at GNSS loss point (5 km check) NAV-02: TRN correction error < 50 m CEP-95 during GNSS-denied segment EW-01: EW cost-map response ≤ 500 ms from jammer activation EW-02: Route replan executed ≤ 1 s; avoidance successful both replans EW-03: GNSS spoof rejected by BIM; trust = Red within 250 ms SAT-01: Terrain masking manoeuvre executed at correct time window TERM-01: Thermal target acquired; EO lock confidence ≥ 0.85 TERM-02: Decoy correctly rejected; mission continues to simulated impact TERM-03: L10s-SE compliance = 100% SYS-01: All state machine transitions completed within ≤ 2 s SYS-02: Log completeness ≥ 99%; pre-terminal ZPI burst confirmed |
| --- | --- |

### 5.3.4  HIL Validation

| HIL Objective | Test Scenario | Acceptance Metric |
| --- | --- | --- |
| Real sensor integration | Live IMU + camera + SDR against synthetic environment | State vector consistency vs. Gazebo ground truth within tolerance |
| Mission envelope crypto loading | Load and verify AES-256 + PQC signed envelope on target hardware | Load time < 2 s; hash verification pass rate 100% |
| Fallback transitions on real compute | RF jamming → SDR overload → EW alert → replan | Fallback initiation ≤ 2 s (NFR-002) |
| Real RADALT integration | RADALT unit in loop; TRN correction active | NFR-004: TRN update ≥ 2 Hz; RADALT output variance within spec |
| SDR bandwidth processing | Full EW sweep on production-candidate SDR front-end | NFR-003: processing < 500 ms at target BW |

### 5.3.5  Field Trials (Progressive)

| Phase | Focus | Key Activities |
| --- | --- | --- |
| Phase 1 — Safe Flight | Baseline navigation | Open-area flights with GNSS toggling; drift profiling 5–15 km; passive EW sweeps |
| Phase 2 — Controlled EW | EW resilience | Authorised EW range; low/mid-power jamming; GNSS spoof field injection; terrain-masking ingress |
| Phase 3 — Terminal | Terminal behaviour | EO/IR target acquisition at range; thermal decoy; DMRL-lite rejection; L10s-SE gating |

## 5.4  Supply Chain & Component Risk Register

| Component | Risk Flags | Mitigation |
| --- | --- | --- |
| Compute platform (primary) | Export control; single-supplier; see DD-01 | Qualification matrix (Section 4.4); passive tracking from Sprint S1; active qualification from Sprint S3 |
| Thermal camera (LWIR) | Seasonal availability of thermal cores; export sensitivity for radiometric units | Identify ≥ 2 supplier options before HIL; non-US suppliers preferred for production |
| SDR front-end | Production BW (≥ 40 MHz) units have narrower supplier base | Prototype on ADALM-PLUTO or equivalent; identify production-grade unit before HIL |
| Tactical IMU | Export sensitivity varies by grade; high-grade units may require licensing | Confirm export classification before procurement; automotive-grade is generally EAR99 |
| RADALT | Small market; lead times 8–16 weeks typical | Initiate HIL procurement at Sprint S3 completion; multiple non-US suppliers available in spec class |

## 5.5  Policy, Legal & Ethical Risk

| AUTONOMY IN LETHAL CONTEXTS | MicroMind operates within a commander-bounded envelope, but terminal decisions executed autonomously create legal ambiguity across jurisdictions, ROE interpretation risks, and human oversight threshold questions. Risk flag: This section requires consultation with MoD legal, operational commanders, and policy groups before any field trial phase. Planned actions: • Produce a legal-technical briefing for Indian MoD stakeholders • Identify mission types requiring mandatory Human-In-The-Loop (HITL) review • Define phased autonomy levels: Pilot Mode → Assisted → Bounded Autonomy • L10s-SE decision tree content must be reviewed and approved by MoD legal before it is encoded — AI can structure the tree; policy must come from authorised sources |
| --- | --- |

## 5.6  Key Performance Indicators

| KPI ID | KPI | Target Value | Measurement Method | Sprint |
| --- | --- | --- | --- | --- |
| KPI-N01 | GNSS-denied drift | < 1–2% of path length at 50 km | Gazebo ground truth vs. fused pose; ROS2 bag log | S3 |
| KPI-N02 | BIM spoof detection latency | ≤ 250 ms | ROS2 topic timestamp delta: inject event vs. BIM state change | S2 |
| KPI-N03 | TRN correction error | < 50 m CEP-95 | Ground truth comparison; 20-run Monte Carlo in Gazebo | S3 |
| KPI-E01 | EW cost-map response | ≤ 500 ms | GNU Radio jammer ON → cost tile update timestamp in ROS2 bag | S4 |
| KPI-E02 | GNSS spoof rejection rate | ≥ 99% | 100-run Monte Carlo with validated waveform set; false-negative count | S2 |
| KPI-E03 | EW avoidance success rate | ≥ 90% of simulated jammer zones avoided | Route success rate in BCMP-1 EW scenario; 20 runs | S4 |
| KPI-T01 | Terminal lock rate | ≥ 85% | 50 terminal approach runs in synthetic thermal scene; successful locks | S5 |
| KPI-T02 | Decoy rejection rate | ≥ 90% | 50 runs with thermal decoy present; correct rejections | S5 |
| KPI-T03 | L10s-SE compliance | 100% (hard requirement) | Every terminal run logged; any compliance failure = test failure | S5 |
| KPI-S01 | Autonomy fallback transition | ≤ 2 seconds | Failure inject event → new state in ROS2 log; timestamp delta | S1 / S3 |
| KPI-S02 | Log completeness | ≥ 99% | Post-run log field coverage audit vs. schema (DD-02) | S1 onwards |

## 5.7  Field Operations, Sustainment & Serviceability

A full field-support doctrine will be prepared after hardware selection. The following tasks are scoped for post-hardware-qualification development:

- Define field calibration requirements for IMU, RADALT, and thermal camera

- Define maintenance intervals and spares ratio per 100 sorties

- Define operator-level diagnostic workflow and go/no-go checklist

- Define mission log extraction and chain-of-custody procedure for post-mission analysis

SECTION 6 — GROWTH ROADMAP

# 6  Growth Roadmap

| Phase | Timeline | Key Capabilities | Milestone Gate |
| --- | --- | --- | --- |
| MVP-SIL | 0–6 months | Core NanoCorteX autonomy; GNSS-denied navigation (VIO + TRN sim); BIM; DMRL-lite; L10s-SE; Mission Envelopes; basic EW detection; BCMP-1 SIL run | BCMP-1 SIL pass against all KPIs → TASL approach |
| MVP-HIL | 6–12 months | Real hardware integration on Jetson (SIL proxy) → qualify production compute; physical RADALT; real SDR; real thermal camera; SHM; enhanced L10s-SE | HIL fallback transition ≤ 2 s; all NFRs met on hardware |
| MVP+1 | 12–18 months | Advanced terminal logic; CEMS multi-UAV; decoy rejection model; terrain A* refinement; ZPI burst full integration; compute production platform selected | Field trial Phase 1 (safe flight); compute platform locked |
| Phase 2 | 18–30 months | Full CEMS; ZPI burst integration; multi-UAV EW triangulation; cross-mission learning pipeline (DD-02 Phase 2); swarm integration; CDAC Vega engagement | Field trial Phase 2 (EW); Phase 3 (terminal); export classification confirmed for production platform |
| Phase 3 | 30+ months | UAE / NATO interoperability; OEM SDK release; full fleet-level learning; indigenous compute transition (CDAC or equivalent) | International demonstration; OEM SDK released |

SECTION 7 — CONCLUSION

# 7  Conclusion

Western technology is built for Air Superiority. Indian reality is Air Denial. MicroMind is not built for the parade ground; it is built for the GPS-dead, spectrum-screaming reality of a two-front war where the enemy knows our frequencies and watches us from space. It is the brain that survives when the network dies.

Version 7 of this document represents the first iteration of Part Two as a live engineering specification rather than a concept sketch. Architectural philosophy is preserved from V6. What has changed is precision: boundary conditions are populated, design decisions are documented, and the SIL programme has a formal acceptance gate in BCMP-1.

The architecture is sound. The SIL programme is achievable. The hardware sovereignty position is defensible. The next 30 days convert this document into running code.

| IMMEDIATE ACTION LIST — NEXT 30 DAYS | 1. Formalise NanoCorteX state machine: UML statechart from Section 1.2; controlled document 2. Create BCMP-1 scenario definition sheet: DEM selection; jammer positions; satellite window 3. Stand up ROS2 / PX4 SITL skeleton: Sprint S1; node graph; interface stubs; CI pipeline 4. Define mission log schema with learning fields: Sprint S1 deliverable per DD-02 5. Begin passive compute platform tracking: datasheets; export status; CDAC contact 6. Initiate MoD legal consultation on autonomy policy (Section 5.5) — long lead item 7. RADALT performance specification finalised: for SIL plugin modelling and HIL procurement planning |
| --- | --- |

SECTION 8 — GLOSSARY OF TERMS

# 8  Glossary of Terms

| Term | Definition |
| --- | --- |
| A* / Hybrid A* | Pathfinding algorithm used for corridor and route generation; Hybrid A* adds kinodynamic constraints and EW cost overlay |
| AES-256 | Advanced Encryption Standard, 256-bit; used for mission package and log encryption |
| ARW | Angle Random Walk — IMU gyroscope noise characteristic; expressed as °/√hr |
| BCMP-1 | Baseline Certification Mission Profile 1 — the primary SIL acceptance scenario: GNSS denied, RF link lost, adaptive EW, terrain masking, terminal zone highest EW density |
| BIM | Beacon & Integrity Monitor — evaluates GNSS trust; produces continuous score and G/A/R state |
| CEMS | Cooperative EW Sharing Module — exchanges signed EW observations between UAVs |
| CEP-50 / CEP-95 | Circular Error Probable at 50% / 95% confidence — standard position accuracy metric |
| CNN | Convolutional Neural Network — used for decoy rejection in DMRL terminal phase |
| DD-01 | Design Decision 01 — hardware sovereignty; no compute platform named in architecture |
| DD-02 | Design Decision 02 — cross-mission learning deferred; Phase 1 minimum actions defined |
| DBSCAN | Density-Based Spatial Clustering of Applications with Noise — EW signature clustering algorithm |
| DEM | Digital Elevation Model — terrain height data used for TRN, masking, and route planning |
| DF | Direction Finding — adversary technique for locating UAV signal source |
| DMRL | Decoy Multi-frame Rejection Logic — filters false targets using EO/IR and thermal signatures |
| EAR | Export Administration Regulations — US export control framework governing dual-use technology |
| EKF / UKF | Extended / Unscented Kalman Filter — sensor fusion state estimators |
| EO/IR | Electro-Optical / Infrared cameras |
| EW | Electronic Warfare — includes jamming, spoofing, and signal interference |
| FGSM / PGD | Fast Gradient Sign Method / Projected Gradient Descent — adversarial ML attack methods |
| FR | Functional Requirement |
| GNSS | Global Navigation Satellite System — GPS, GLONASS, Galileo, NavIC, etc. |
| HKDF-SHA256 | HMAC-based Key Derivation Function using SHA-256 — used for ZPI hop plan and session key derivation |
| HIL | Hardware-in-the-Loop — testing environment using real hardware |
| IMU | Inertial Measurement Unit — accelerometer, gyroscope, barometer |
| L10s-SE | Last-10-Second Safety Envelope — enforces ROE and abort logic in terminal phase |
| LWIR | Long-Wave Infrared — 8–14 μm thermal imaging band; required for DMRL |
| MicroMind-OS | Mission planning and commander interface system |
| MicroMind-X | Onboard mission computer; runs sensor fusion, EW observation, and secure logging |
| MTCR | Missile Technology Control Regime — export control framework for rocket and UAV technology |
| NanoCorteX | Autonomy engine — executes state machine, navigation, EW-aware routing, terminal guidance |
| NCC | Normalised Cross-Correlation — terrain matching algorithm for TRN |
| NFR | Non-Functional Requirement |
| NMEA | Standard GNSS metadata and log format — used for BIM integrity scoring |
| NPU | Neural Processing Unit — dedicated AI inference accelerator |
| PQC | Post-Quantum Cryptography — SPHINCS+ and Dilithium signature schemes |
| RAIM | Receiver Autonomous Integrity Monitoring — GNSS integrity analysis |
| RADALT | Radar Altimeter — provides precise height AGL; required for TRN in mountainous terrain |
| RF | Radio Frequency |
| ROE | Rules of Engagement — governs allowed actions encoded in mission envelope |
| RSSI | Received Signal Strength Indicator — used for jammer bearing estimation |
| SDR | Software Defined Radio — used for EW observation and ZPI burst communications |
| SGP4 | Simplified General Perturbations 4 — standard LEO satellite propagator for ephemeris prediction |
| SHM | Silent Homing Mode — final-phase navigation with zero RF emissions |
| SIL | Software-in-the-Loop — simulation-based algorithm validation before hardware |
| SRTM | Shuttle Radar Topography Mission — 30 m global DEM data source |
| SWaP | Size, Weight, and Power — hardware constraint envelope |
| TLE | Two-Line Element — satellite ephemeris data format for SGP4 propagation |
| TRN | Terrain Relative Navigation — altimeter + DEM fusion for GNSS-denied drift correction |
| VIO | Visual-Inertial Odometry — camera + IMU fusion for GNSS-denied navigation |
| VO | Visual Odometry — camera-based pose estimation from feature tracking / optical flow |
| ZPI | Zero Probability of Intercept — burst communication protocol for LPI/LPD signalling |

END OF DOCUMENT — MicroMind System Family Part Two — Version 7  |  February 2026  |  DRAFT CONFIDENTIAL

This is a live working document. All boundary condition values are candidate engineering starting points subject to team review and SIL calibration. No values in this document should be treated as validated until confirmed by SIL testing.